# Submission ID #: 70366

# Scriptwriter Name: Deepthi Das

# Project Page Link: https://review.jove.com/files_upload.php?src=21461678

# Title: Optical Trawl Surveys of Groundfish; A Video Trawl Survey of Western Gulf of Maine Atlantic Cod (Gadus Morhua)

# Authors and Affiliations:

# Nicholas M. Calabrese, Stephanie L. Merhoff, Helena L. Norton, Kevin D. E. Stokesbury

School for Marine Science and Technology, University of Massachusetts Dartmouth

# Corresponding Authors:

Nicholas M. Calabrese     ncalabrese@umassd.edu

# Email Addresses for All Authors:

smerhoff@umassd.edu

hnorton@umassd.edu

kstokesbury@umassd.edu

ncalabrese@umassd.edu

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

3. Filming location: Will the filming need to take place in multiple locations?   Yes

3 Miles

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials.  No

Current Protocol Length

Number of Steps: 11

Number of Shots: 28

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Nicholas Calabrese: This research provides a minimally invasive method for collecting trawl survey data to quantify abundance, distribution, and biomass.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research contexts or systems in which this protocol can be applied.

  - Kevin Stokesbury: This system can be applied to any otter or midwater trawl net and adapted for science or increased fishing information.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Kevin Stokesbury: Researchers can estimate the abundance of species in trawl nets and assess diversity, increasing size selectivity for small fish and invertebrates.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Nicholas Calabrese: Operating delicate optical equipment in harsh environments is challenging and requires some electrical repair skills and access to spare parts.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Kevin Stokesbury: Automating video analysis using AI and integrating stereoscopic measurements are the next steps in development.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

NOTE: Timestamps for SCREEN shots were added at the postshoot stage. Please contact post shoot integrator  (Pallavi Sharma) for any discrepancies

- Equipment Setup on the Vessel Deck

Demonstrators: Stephanie Merhoff and Helena Norton

  - To begin, arrange the survey net and pressure-sensitive winch on the vessel’s deck [1]. Attach the survey trawl doors to the vessel’s winch cables [2]. Then, secure the winch to the deck using a temporary weld [3].

    - WIDE: Talent arranging the survey net and winch on the deck.
Videographer: Please obtain CU shot as well for this shot.

    - Talent attaching the trawl doors to the winch cables.

    - Talent welding the winch to the deck.

  - Next, wire the winch power cable into the vessel’s breaker box to establish a stable electrical supply [1].

    - Talent wiring the winch power cable into the vessel’s breaker box.

  - Attach a cable block to a mount on the vessel’s A-frame or similar support structure [1] and route the coaxial cable through the block toward the stern, positioning it adjacent to the net drum [2- TXT].

    - Talent attaching a cable block to a mount on the vessel’s A-frame.

    - Talent routing the coaxial cable through the block towards the stern, positioning it adjacent to the net drum.

  - Attach the junction box to the rigid polyethylene cylinder [1]. Then, mount the cameras and lights onto the cylinder [2]. Connect the cameras and lights to the junction box using cables fitted with waterproof connectors [3].

    - Talent attaching the junction box to the polyethylene cylinder.

    - Talent mounting the cameras and lights onto the polyethylene cylinder.

    - Talent connecting the cameras and lights to the junction box using cables fitted with waterproof connectors.

- Wheelhouse Equipment Set-up for Video and Data Capture

  - First, secure the computer and monitors to a stable workstation in the wheelhouse [1]. Then, connect the GPS (G-P-S) devices to the computer to enable real-time spatial data acquisition [2]. Connect the net mensuration receiver to the computer [3] and connect cameras and lights to the power in the vessel’s wheelhouse [4].

    - WIDE: Talent securing the computer and monitors to a stable workstation in the wheelhouse.

    - Talent connecting GPS devices to the computer ports.

    - Talent connecting the net mensuration receiver to the computer.

    - Talent connecting cameras and lights to power in the vessel’s wheelhouse.

  - After verifying proper equipment function, launch the mapping software to display the vessel location [1]. Check if the time and date stamps are correctly displayed in the video recording software [2]. Enter the tow number to be overlayed on video [3]. Launch the net mensuration sensor software and begin recording the wingspread and doorspread data [4].

    - SCREEN: 3_2_1.mp4: 00:06-00:08, 00:17-00:35

    - SCREEN: 3_2_2.mp4: 00:01-00:08

    - SCREEN: 3_2_3.mp4:  00:03-00:09

    - SCREEN: 3_2_4.mp4:  00:02-00:05, 00:17-00:19, 00:58-01:26

  - Using the vessel’s hydraulic winches, deploy the net and doors slowly [1] and start recording the video [2]. Turn on the lights after the cylinder enters the water [3].

    - Talent setting the net and doors out using the vessel’s hydraulic winches.

    - SCREEN: 3_3_2.mp4: 00:02-00:08

    - Talent turning on the lights after the cylinder enters the water.

- Video and Data Acquisition for Each Tow and The Subsequent Review

  - Once the vessel’s hydraulic winches are locked into position, proceed to record the location and speed data [1-TXT].

    - SCREEN: 4_1_1.mp4: 00:06-end.

  - Stop recording location and speed data once the vessel’s hydraulic winches are engaged for haul back, indicating the end of the tow [1]. Switch off the lights once the net and doors are hauled back and the lights reach the surface. Similarly, capture video and data at all stations [2-TXT].

    - SCREEN: 4_2_1.mp4

    - SCREEN: 4_2_2.mp4

  - After completing the tow, secure the codend of the net using the codend clip [1]. Document the trawl catch aboard the vessel following standardized procedures [2].

    - Shot of crew attaching the codend clip.

    - TEXT ON A PLAIN BACKGROUND
-     Empty the codend onto the vessel deck

- Sort the catch into baskets by species

- Measure, weigh, and sex a standardized subsample

- Record counts and weights for remaining catch

  - Offload all data from the wheelhouse computer to external hard drives [1]. Upload all videos to the annotation software [2]. Trained technicians shall review and annotate all target species and unidentified fish [3].

    - Shot of the external hard drives connected to the wheelhouse computer during data transfer. Videographer: Please show the data transfer progressing on the computer screen as well.

    - SCREEN: 4_4_2.mp4: 00:03-00:10, 00:19-00:21

    - SCREEN: 4_4_3.mp4: 00:03-00:15

# Results

- Results

  - During the 10 days of sampling, 47 usable survey tows were completed [1], including 4 closed codend [2] and 43 open codend tows during daylight hours [3].

    - LAB MEDIA: Table 1 Video editor: Emphasize the two columns Catch (Ind) and Catch (kg) and highlight all rows except those corresponding to 40 and 43.

    - LAB MEDIA: Figure 2 Video editor: Emphasize the red dots.

    - LAB MEDIA: Figure 2 Video editor: Emphasize the blue dots (mostly look like lines).

  - The length-frequency distribution shows that Atlantic cod captured during the survey ranged from 41 to 81 centimeters [1].

    - LAB MEDIA: Figure 3 Video Editor: Please show the full graph and then emphasize the right end with length from 41 cm to 81 cm.

  - The length-weight relationship demonstrates the expected increase in weight with fish length, confirming consistent biological measurements from the cod sampled during the survey [1].

    - LAB MEDIA: Figure 4 Video Editor: Please show the entire graph and highlight the equation given in the graph (y = 1E-05x2.9778; R² = 0.962).

- Pressure-sensitive
Pronunciation link: https://www.merriam-webster.com/dictionary/pressure-sensitive
IPA: /ˈpreʃər ˈsensətɪv/
Phonetic Spelling: presh·ur sen·suh·tiv

- Winch
Pronunciation link: https://www.merriam-webster.com/dictionary/winch
IPA: /ˈwɪntʃ/
Phonetic Spelling: winch

- Trawl
Pronunciation link: https://www.merriam-webster.com/dictionary/trawl
IPA: /ˈtrɔːl/
Phonetic Spelling: trawl

- Coaxial
Pronunciation link: https://www.merriam-webster.com/dictionary/coaxial
IPA: /koʊˈæksiəl/
Phonetic Spelling: koh·ak·see·uhl

- Polyethylene
Pronunciation link: https://www.merriam-webster.com/dictionary/polyethylene
IPA: /ˌpɑːliˈeθəˌliːn/
Phonetic Spelling: pol·ee·eth·uh·leen

- Waterproof
Pronunciation link: https://www.merriam-webster.com/dictionary/waterproof
IPA: /ˈwɔːtərˌpruːf/
Phonetic Spelling: waw·tur·proof

- Junction
Pronunciation link: https://www.merriam-webster.com/dictionary/junction
IPA: /ˈdʒʌŋ(k)ʃən/
Phonetic Spelling: jungk·shuhn

- Mensuration
Pronunciation link: https://www.merriam-webster.com/dictionary/mensuration
IPA: /ˌmenʃəˈreɪʃən/
Phonetic Spelling: men·shuh·ray·shuhn

- Hydraulic
Pronunciation link: https://www.merriam-webster.com/dictionary/hydraulic
IPA: /haɪˈdrɔːlɪk/
Phonetic Spelling: hy·draw·lik

- Spatial
Pronunciation link: https://www.merriam-webster.com/dictionary/spatial
IPA: /ˈspeɪʃəl/
Phonetic Spelling: spay·shuhl

- Acquisition
Pronunciation link: https://www.merriam-webster.com/dictionary/acquisition
IPA: /ˌækwəˈzɪʃən/
Phonetic Spelling: ak·wuh·zi·shuhn

- Overlayed
Pronunciation link: https://www.howtopronounce.com/overlayed
IPA: /ˌoʊvərˈleɪd/
Phonetic Spelling: oh·vur·layd

- Wingspread
Pronunciation link: https://www.merriam-webster.com/dictionary/wingspread
IPA: /ˈwɪŋˌspred/
Phonetic Spelling: wing·spred

- Doorspread
Pronunciation link: No confirmed link found
IPA: /ˈdɔːrˌspred/
Phonetic Spelling: door·spred

- Annotation
Pronunciation link: https://www.merriam-webster.com/dictionary/annotation
IPA: /ˌænəˈteɪʃən/
Phonetic Spelling: an·uh·tay·shuhn

- Codend
Pronunciation link: https://www.howtopronounce.com/codend
IPA: /ˈkɑːdˌɛnd/
Phonetic Spelling: kod·end

- Subsample
Pronunciation link: https://www.merriam-webster.com/dictionary/subsample
IPA: /ˈsʌbˌsæmpəl/
Phonetic Spelling: sub·sam·puhl

- Technicians
Pronunciation link: https://www.merriam-webster.com/dictionary/technician
IPA: /tekˈnɪʃən/
Phonetic Spelling: tek·nish·uhn

- Atlantic
Pronunciation link: https://www.merriam-webster.com/dictionary/Atlantic
IPA: /ətˈlæntɪk/
Phonetic Spelling: ut·lan·tik

- Centimeters
Pronunciation link: https://www.merriam-webster.com/dictionary/centimeter
IPA: /ˈsentəˌmiːtər/
Phonetic Spelling: sen·tuh·mee·tur

- Biological
Pronunciation link: https://www.merriam-webster.com/dictionary/biological
IPA: /ˌbaɪəˈlɑːdʒɪkəl/
Phonetic Spelling: by·uh·loj·ih·kuhl

- Standardized
Pronunciation link: https://www.merriam-webster.com/dictionary/standardize
IPA: /ˈstændərˌdaɪz/
Phonetic Spelling: stan·derd·dyz

- Vessel
Pronunciation link: https://www.merriam-webster.com/dictionary/vessel
IPA: /ˈvesəl/
Phonetic Spelling: ves·uhl

- Tension
Pronunciation link: https://www.merriam-webster.com/dictionary/tension
IPA: /ˈtenʃən/
Phonetic Spelling: ten·shuhn

- Frequency
Pronunciation link: https://www.merriam-webster.com/dictionary/frequency
IPA: /ˈfriːkwənsi/
Phonetic Spelling: free·kwuhn·see