# Submission ID #: 70375

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=21464423

# Title: Hypercapnic Hyperventilation to Accelerate Emergence from Inhaled Volatile Anesthetics: A Practical Method Using Standard Hospital Equipment

# Authors and Affiliations:

# Blaz Pecek1, Lucas T. van Eijk2, Jörgen Bruhn3

# 1Department of Anesthesia, Izola General Hospital

# 2Department of Anesthesiology, Pain and Palliative Medicine, Radboud University Medical Center

# 3Department of Anesthesiology, Pain and Palliative Medicine, Radboud University Medical Center

# Corresponding Authors:

# Lucas T. van Eijk	lucas.vaneijk@radboudumc.nl

# Email Addresses for All Authors:

# Blaz Pecek		blazpecek@gmail.com

# Jorgen Bruhn		jorgen.bruhn@radboudumc.nl

# Lucas T. van Eijk	lucas.vaneijk@radboudumc.nl

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 07/06/2026

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  25

Number of Shots:  47

# Interviews

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context.

We have chosen the 5 best answers

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Blaz Pecek: Our research asks whether controlled hypercapnic hyperventilation can accelerate emergence from volatile anesthesia using only standard equipment available in the EU.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Blaz Pecek: Commercial devices for this technique are scarce, so we recreated hypercapnic hyperventilation using equipment that every operating room already has.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

1.3.. Blaz Pecek: This protocol lets researchers measure how controlled carbon-dioxide elevation and increased minute ventilation accelerate volatile anesthetic washout and shorten time to emergence.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the most important consideration or challenge when performing this protocol.

  - Blaz Pecek: The greatest challenge is keeping carbon dioxide within the target range and restoring the breathing circuit to normal.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Blaz Pecek: Future randomized trials should compare this technique against propofol-based anesthesia and define safe hypercapnia thresholds in larger, diverse populations.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Title Card

No separate study-specific ethics approval was required, as the work did not involve any additional interventions beyond standard clinical care. The patient provided broad consent for the use of their anonymized medical data.

# Protocol

- Preparation of the Anesthesia Circuit

Demonstrator: Blaz Pecek

  - To begin, turn off the volatile anesthetic vaporizer connected to the patient who underwent an elective surgery with stable intraoperative hemodynamics [1]. Ensure that no anesthetic agent is being delivered to the circuit [2].

    - WIDE: Talent turning off the volatile anesthetic vaporizer connected to the patient.

    - Talent pointing to and confirming that no anesthetic agent is being delivered to the circuit.

  - Set the inspired oxygen fraction to 1 by adjusting the anesthesia machine oxygen blender to deliver 100% oxygen [1].

    - Talent adjusting the anesthesia machine oxygen blender to deliver 100% oxygen.

  - Reduce the fresh gas flow to approximately 0.2 liters per minute using the flow control knobs on the anesthesia machine and promote rebreathing within the circuit [1].

    - Talent reducing the fresh gas flow to approximately 0.2 liters per minute using the flow control knobs.

  - Then, attach one active charcoal filter to the inspiratory limb, immediately distal to the Y-piece [1]. Ensure the correct directional orientation according to the manufacturer's instructions [2].

    - Talent attaching a single active charcoal filter to the inspiratory limb of the anesthesia circuit immediately distal to the Y-piece.

    - Close-up of the correctly oriented active charcoal filter on the anesthesia circuit.

  - Verify that the filter is securely connected without leaks and that it is correctly oriented for unidirectional gas flow [1].

    - Talent checking the filter connection for an airtight seal.

  - Refill the breathing circuit with 100% oxygen by flushing the oxygen button for 1 to 2 seconds in the event of an unwanted disconnection [1].

    - Talent refilling the breathing circuit by flushing the oxygen button for 1 to 2 seconds.

  - Disconnect the carbon dioxide-absorbing canister containing soda lime from the anesthesia machine to allow carbon dioxide accumulation within the breathing loop [1].

    - Talent disconnecting the carbon dioxide-absorbing canister containing soda lime from the anesthesia machine.

  - During the initiation of hypercapnic hyperventilation, continuously monitor the inspired volatile anesthetic concentration [1].

    - Talent monitoring and pointing to the inspired volatile anesthetic concentration on the anesthesia machine display.

  - Confirm that all circuit connections are airtight [1] and verify that the anesthesia machine displays the normal pressure and flow readings before proceeding [2].

    - Talent inspecting all circuit connections for airtight seals.

    - Show the anesthesia machine display with the normal pressure and flow readings.

- Inducing Hypercapnic Hyperventilation

  - Apply the protocol in either mechanically ventilated patients or spontaneously breathing patients, depending on the intraoperative ventilation status at the end of surgery [1]. Set the mechanical ventilation to achieve controlled hypercapnia [2].

    - Talent assessing the patient’s intraoperative summary chart and checking the ventilator.

    - Talent setting the mechanical ventilation parameters to achieve controlled hypercapnia.

  - Increase the tidal volume to 8 to 10 milliliters per kilogram of ideal body weight [1]. Adjust minute ventilation to 10 to 12 liters per minute [2].

    - Talent increasing the tidal volume setting on the ventilator.

    - Talent adjusting the minute ventilation setting on the ventilator.

  - Reduce the fresh gas flow as demonstrated earlier [1]. Remove the carbon dioxide absorber to allow controlled accumulation [2]. Allow the rising carbon dioxide levels to stimulate the respiratory drive, resulting in a physiological increase in the minute ventilation without external ventilator adjustments [3].

    - Talent reducing the fresh gas flow.

    - Talent removing the carbon dioxide absorber.

    - Shot of the increasing respiratory activity associated with the rising carbon dioxide levels.

  - Continuously monitor the end-tidal carbon dioxide via capnography [1].

    - Show the end-tidal carbon dioxide values on the capnography display.

  - Increase the fresh gas flow in increments of approximately 0.1 to 0.3 liters per minute when the end-tidal carbon dioxide exceeds 7 to 7.3 kilopascals or 52.5 to 55 millimeters of mercury [1]. Reassess the end-tidal carbon dioxide within 1 to 2 minutes after each adjustment to evaluate the response [2].

    - Talent increasing the fresh gas flow setting.

    - Talent pointing to the end-tidal carbon dioxide values after the adjustment.

  - Decrease the fresh gas flow to a minimum of approximately 0.2 liters per minute when the end-tidal carbon dioxide is below the target range. Avoid abrupt or large adjustments  [1]. Maintain the fresh gas flow within a low-flow range of 0.2 to 1 liters per minute throughout the procedure, titrated according to continuous capnographic feedback [2].

    - Talent decreasing the fresh gas flow setting.

    - Show the display indicating maintained fresh gas flow settings while monitoring the capnographic feedback.

  - Increase the fresh gas flow slightly from approximately 0.2 to 0.5 liters per minute once the target end-tidal carbon dioxide range is reached to stabilize the carbon dioxide levels [1].

    - Talent increasing the fresh gas flow setting slightly to stabilize the carbon dioxide levels.

- Monitoring and Post-Procedure Restoration

Demonstrator: Click here to enter name of demonstrator(s)

  - Maintain the fraction of inspired oxygen at 0.7 to 1 during the entire procedure [1]. Monitor inspiratory and expiratory volatile anesthetic concentrations with a calibrated gas analyzer [2].

    - Talent adjusting the fraction of inspired oxygen on the anesthesia system.

    - Talent checking and showing the inspiratory and expiratory volatile anesthetic concentrations on the calibrated gas analyzer.

  - Observe the patient’s hemodynamics, oxygenation, and ventilation parameters [1]. Continue monitoring until the inspired volatile anesthetic concentration decreases to zero [2].

    - Show the patient’s hemodynamics, oxygenation, and ventilation parameters.

    - Show the inspired volatile anesthetic concentration until it reaches zero.

  - Assess the return of spontaneous breathing [1]. Observe the clinical signs of emergence, including eye opening, purposeful movement, or response to verbal commands [2].

    - Talent assessing the return of spontaneous breathing.

    - Talent observing the clinical signs of emergence.

  - Immediately terminate the hypercapnic hyperventilation by increasing the fresh gas flow to 10 liters per minute if oxygen saturation falls, hemodynamic instability occurs, or end-tidal carbon dioxide exceeds the limit despite corrective measures [1-TXT].

    - Talent increasing the fresh gas flow to 10 liters per minute to terminate the hypercapnic hyperventilation. TXT: SpO₂ < 94%; EtCO₂ > 8.5 kPa; 64 mmHg

  - Reinstall the carbon dioxide absorber [1] and return the fresh gas flow to the standard settings [2]. Resume the conventional ventilation parameters appropriate for the patient and anesthetic phase [3].

    - Talent reinstalling the carbon dioxide absorber.

    - Talent returning the fresh gas flow to the standard settings.

    - Talent adjusting the conventional ventilation parameters appropriate for the patient and anesthetic phase.

  - Proceed with extubation according to the standard institutional criteria [1]. After completing the protocol, remove the active charcoal filter from the inspiratory limb before preparing the circuit for the next case [2].

    - Talent performing extubation after the patient demonstrates adequate conditions.

    - Talent removing the active charcoal filter from the inspiratory limb.

  - Visually inspect the circuit and manually confirm that the filter has been removed [1] and the standard configuration restored [2].

    - Talent inspecting the inspiratory limb to confirm the filter removal.

    - Talent physically checking the circuit to confirm restoration of the standard configuration.

  - Reconnect or reinstall the carbon dioxide absorber with soda lime before initiating anesthesia for the next case [1].

    - Talent reconnecting or reinstalling the carbon dioxide absorber with soda lime.

  - Confirm normal carbon dioxide absorption [1] and stable baseline capnography before induction for the next case [2].

    - Talent checking the carbon dioxide absorber.

    - Talent pointing to and verifying the stable baseline capnography before induction.

# Results

- Results

  - Oxygenation remained stable throughout the protocol, with the fraction of inspired oxygen maintained around 0.7 [1] and oxygen saturation remaining at 100% [2].

    - LAB MEDIA: Table 1. Video editor: Highlight the (FiO₂) column.

    - LAB MEDIA: Table 1. Video editor: Highlight the SpO2 column.

  - Hemodynamic safety was verified by a stable heart rate [1] and blood pressure trends throughout the maneuver [2].

    - LAB MEDIA: Table 1. Video editor: Highlight the heart rate column.

    - LAB MEDIA: Table 1. Video editor: Highlight the blood pressure  (RR MM/Hg) column.

  - The patient spontaneously opened their eyes after 3.5 minutes and was extubated without complications [1].

    - LAB MEDIA: Table 1. Video editor: Highlight the row corresponding to t+3 and t+4 minutes .

  - Representative recordings showed that end-tidal carbon dioxide increased progressively from 39 millimeters of mercury at the start of hypercapnic hyperventilation [1] to 54 millimeters of mercury after 5 minutes [2].

    - LAB MEDIA: Figure 3. Video editor: Highlight the blue data point labeled 39 at time 0 minutes.

    - LAB MEDIA: Figure 3. Video editor: Highlight the blue data point labeled 54 at time +5 minutes.

  - Expired sevoflurane concentration decreased from 0.9 volume percent before hypercapnic hyperventilation [1] to 0 volume percent within 3 minutes [2].

    - LAB MEDIA: Figure 3. Video editor: Highlight the red data point labeled 0.9 before the start of hypercapnic hyperventilation mark.

    - LAB MEDIA: Figure 3. Video editor: Highlight the red data point labeled 0.0 at t+3 min.

  - Following carbon dioxide elevation, spontaneous breathing with tidal volume and respiratory rate within a physiological range was observed [1].

    - LAB MEDIA: Table 1. Video editor: Highlight the rows t+3, t+4, t+5 in the RR column.