# Submission ID #: 70445

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/account/file-uploader?src=21485773

Title: Synthesis, Hemoglobin Encapsulation and Biorthogonal PEGylation in Hierarchically Porous UiO-66 Nanoparticles for Oxygen Delivery Applications

# Authors and Affiliations:

Weiguang Jin and Leticia Hosta-Rigau*

Department of Health Technology, Technical University of Denmark

# Corresponding Authors:

Leticia Hosta-Rigau			(leri@dtu.dk)

# Email Addresses for All Authors:

Weiguang Jin 			(weig@dtu.dk)

Leticia Hosta-Rigau		(leri@dtu.dk)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  NO

3. Filming location: Will the filming need to take place in multiple locations?   Yes

In same building but different lab needs 5 min walking

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps:  15

Number of Shots:  42

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Weiguang Jin: The focus of our research is to develop a stable MOF-based nanoparticle for hemoglobin encapsulation and oxygen delivery.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Weiguang Jin: Hierarchically porous MOFs overcome pore-size limitations for hemoglobin loading, while PEGylation enhances nanoparticle stability in physiological environments.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 3.1.4

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Weiguang Jin: This protocol enables the synthesis of hierarchically porous MOF nanoparticles, hemoglobin loading, surface PEGylation, and evaluation of oxygen-release functionality.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 4.5.1

Are there any additional methods or analyses that can be performed following this procedure?

  - Weiguang Jin: Using this protocol, additional physicochemical characterization and functional studies can be conducted, including in vitro analyses of oxygen release and nanoparticle biocompatibility.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.  Suggested B roll: Figure 5

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Weiguang Jin: Future studies can focus on preserving hemoglobin functionality for longer durations, such as incorporating antioxidant components to prevent oxidative degradation.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Azide Functionalization of HP-UiO-66-NH2 NPs (HP-UiO-66-N3 NPs)

Demonstrator: Weiguang Jin

  - To begin, accurately weigh 50 milligrams of prepared hierarchically porous-UiO-66 (U-I-O-Sixty-Six) amine nanoparticles into a 25-milliliter round-bottom flask [1]. Add 10 milliliters of tetrahydrofuran to the flask [2]. Then, ultrasonicate the mixture for 5 minutes to fully disperse the particles [3], producing a uniform opaque suspension [4].

    - WIDE: Talent weighing HP-UiO-66-NH2 nanoparticles and transferring them into a 25-milliliter round-bottom flask. Videographer: Please film a CU shot as well

    - Talent adding tetrahydrofuran to the round-bottom flask.

    - Talent placing the round-bottom flask in an ultrasonic bath for sonication.

    - Close shot of the round-bottom flask showing an opaque but uniform suspension.

  - Place the round-bottom flask in an ice-water bath on a magnetic stirrer set to 400 rpm [1]. Using a glass syringe, add 0.4 milliliters of tert-butyl nitrite dropwise over 5 minutes while maintaining the temperature below 5 degrees Celsius [2].

    - Talent placing the round-bottom flask into an ice-water bath positioned on a magnetic stirrer.

    - Talent adding tert-butyl nitrite dropwise into the flask with a glass syringe.

  - Immediately add 0.35 milliliters of trimethylsilyl azide dropwise under the same cold conditions [1]. A mild effervescence may appear, indicating nitrosation progress [2]. Then, wrap the ice-water bath in aluminum foil to protect the reaction from light [3].

    - Talent adding trimethylsilyl azide dropwise into the reaction flask with a syringe while the flask remains in the ice-water bath.

    - Close shot of mild effervescence appearing in the reaction mixture.

    - Talent wrapping aluminum foil around the ice-water bath.

  - Continue stirring the reaction mixture in the ice-water bath overnight [1]. After washing and drying the product, use FTIR (F-T-I-R) spectroscopy to verify the chemical transformation [2].

    - Talent placing the reaction flask in the ice-water bath on the magnetic stirrer for overnight stirring.

    - Talent places the sample in the spectroscope.

- Hemoglobin Loading and Quantification

  - Transfer 5 milligrams of hierarchically porous UiO-66-azide nanoparticles into a 2-milliliter centrifuge tube [1]. Add 0.5 milliliters of HEPES (Hepes) buffer [2] and sonicate for 2-5 minutes [3]. Add 0.5 milliliters of HEPES buffer containing 20 milligrams per milliliter hemoglobin and gently invert the tube several times to fully disperse the nanoparticles [4].

    - Talent transferring HP-UiO-66-N3 nanoparticles into a 2 milliliter centrifuge tube.

    - Talent adding HEPES buffer tothe centrifuge tube.

    - Talent placing the tube in the sonicator.

    - Talent adding HEPES buffer containing hemoglobin into tube and gently inverting the centrifuge tube several times to disperse the nanoparticles.

  - Now, place the tube on a thermoshaker and incubate at room temperature with shaking at 1,000 rpm for 2 hours [1].

    - Talent placing the centrifuge tube onto the thermoshaker.

  - After incubation, centrifuge the tube at 13,500 g for 5 minutes at room temperature to pellet the resulting hemoglobin at hierarchically porous UiO 66 azide nanoparticles [1]. Then, carefully collect the supernatant for quantification of unentrapped hemoglobin [2].

    - Talent placing the centrifuge tube into the centrifuge.

    - Talent transferring the supernatant from the centrifuge tube into a new tube.

  - Wash the pellet gently with 1 milliliter of ultrapure water by pipetting up and down to remove loosely adsorbed hemoglobin [1-TXT]. Then, carefully collect the supernatant of each wash for quantification of unentrapped hemoglobin [2]. Resuspend the final pellet in 0.5 milliliters of ultrapure water for characterization [3-TXT].

    - Talent adding ultrapure water to the centrifuge tube containing the nanoparticle pellet. TXT: Repeat wash 2x

    - Talent collects the supernatant and transfers it into a fresh tube.

    - Talent resuspending the final nanoparticle pellet in ultrapure water. TXT: Keep samples in refrigerator to prevent oxidation.

  - Use the sodium lauryl sulfate hemoglobin assay to determine unentrapped hemoglobin in the collected supernatants [1]. Pipette 10 microliters of each supernatant into a 96-well microplate [2]. Add 100 microliters of sodium lauryl sulfate reagent [3] and incubate for 5 minutes at room temperature in the dark [4]. Using an ultraviolet-visible spectrophotometer, measure absorbance at 412 nanometres [5].

    - Talent at the bench with the sodium lauryl sulfate hemoglobin assay reagent placed in front.

    - Talent pipetting supernatant into wells of a 96-well microplate.

    - Talent adding sodium lauryl sulfate reagent to the wells.

    - Shot of the microplate incubating in the dark at room temperature.

    - Talent placing the microplate into a ultraviolet-visible spectrophotometer to read absorbance.

  - Afterwards, prepare serial dilutions of hemoglobin stock in HEPES buffer to generate a standard curve [1]. Transfer the prepared dilutions into the prepared 96-well plate to generate a standard curve for converting absorbance readings into hemoglobin concentrations [2].

    - Serial dilution of stock of hemoglobin (20 mg mL-1) in 2 mL centrifuge tubes

    - Talent transferring hemoglobin at different concentration into the same 96-well microplate.

- Fabrication of Hb@HP-UiO-66-PEG NPs: PEGylation via SPAAC

  - Transfer 10 milligrams of hemoglobin at hierarchically porous-UiO-66-azide nanoparticles into a 2-milliliter centrifuge tube containing 0.5 milliliters of ultrapure water [1] and sonicate for 2-5 minutes [2]. Add 20 milligrams of Dibenzylcyclooctyne modified with polyethylene glycol to achieve an equimolar ratio of polyethylene glycol to nanoparticles [3]. Gently vortex the tube until the mixture is homogenized [4].

    - Talent transferring Hb@HP-UiO-66-N3 nanoparticles into a 2 milliliter centrifuge tube containing ultrapure water.

    - Talent sonicating the tube for 2-5 minutes.

    - Talent adding the DBCO-PEG solution to the centrifuge tube.

    - Talent vortexing the centrifuge tube to mix the suspension.

  - Place the tube on a thermoshaker and incubate at room temperature with shaking at 1,000 rpm for 1 hour [1]. Then transfer the tube into a refrigerator maintained at 4 degrees Celsius [2]

    - Talent placing the centrifuge tube onto the thermoshaker.

    - Talent placing the centrifuge tube into the refrigerator.

  - Centrifuge the tube to pellet the polyethylene glycol-modified hemoglobin at hierarchically porous-UiO-66-Polyethylene glycol nanoparticles [1-TXT]. Then, discard the supernatant containing unreacted polyethylene glycol [2].

    - Talent placing the centrifuge tube into the centrifuge. TXT: 12,000 g; 5 min; RT

    - Talent discarding the supernatant from the centrifuge tube.

  - Wash the pellet three times with 2 milliliter ultrapure water each by pipetting up and down [1] and centrifuge under the same conditions as demonstrated earlier [2-TXT].

    - Talent adding ultrapure water to the centrifuge tube containing the nanoparticle pellet and pipetting up and down.

    - Talent placing the centrifuge tube into the centrifuge. TXT: Repeat wash 3x

  - Resuspend the purified polyethylene glycol-modified hemoglobin at hierarchically porous-UiO-66-Polyethylene glycol nanoparticles in 2 milliliters of ultrapure water [1]. Store the suspension at 4 degrees Celsius [2]. The resulting colloid appears homogeneous and slightly opalescent, indicating good dispersion stability [3]. Then, confirm successful PEGylation (Pegylation) by dynamic light scattering or DLS (D-L-S) [4]

    - Talent resuspending the purified nanoparticle pellet in ultrapure water.

    - Talent placing the centrifuge tube in a refrigerator.

    - Close shot of the centrifuge tube showing a homogeneous and slightly opalescent nanoparticle suspension.

    - Talent loading the sample in the spectroscope DLS equipment.

# Results

- Results

  - Surface amino groups were successfully converted to azides [1], as demonstrated by the appearance of a characteristic asymmetric azide stretching band at 2122 (Two-One-Two-Two) centimeters inverse in the FTIR spectrum of hierarchically porous UiO-66-azide nanoparticles [2], which was absent in hierarchically porous UiO-66-amine  nanoparticles [3].

    - LAB MEDIA: Figure 4A.

    - LAB MEDIA: Figure 4A. Video editor: Highlight the peak labelled 2122 on the spectrum for “HP-UiO-66-N₃ NPs”.

    - LAB MEDIA: Figure 4A. Video editor: Show the spectrum labelled “HP-UiO-66-NH₂ NPs” and indicate that the peak at 2122 is absent.

  - Hemoglobin encapsulation was visually indicated by a distinct yellow-to-red color change [1].

    - LAB MEDIA: Figure 5A. Video editor: Show the particle labeled  “Hb@HP-UiO-66-N₃ NPs” with the red color.

  - Nitrogen uptake decreased after hemoglobin loading, indicating occupation of mesopores by hemoglobin molecules [1]. The zeta potential decreased from 42.9 millivolts for hierarchically porous UiO-66-azide nanoparticles [2] to 20.4 millivolts for hemoglobin at hierarchically porous UiO-66-azide nanoparticles after hemoglobin encapsulation [3].

    - LAB MEDIA: Figure 5B. Video editor: Highlight the lower curve labelled “Hb@HP-UiO-66-N₃ NPs” compared with the higher curve labelled “HP-UiO-66-N₃ NPs”.

    - LAB MEDIA: Figure 5C. Video editor: Highlight the bar labeled 42.9 millivolt above the green particle icon.

    - LAB MEDIA: Figure 5C. Video editor: Highlight the bar labeled 20.4 millivolt above the orange particle icon.

  - Successful polyethylene glycol conjugation was confirmed by the disappearance of the azide stretching vibration at 2122 centimetres inverse [1], and the appearance of polyethylene glycol-associated signals in the FTIR spectrum [2]. DLS analysis showed an increase in hydrodynamic diameter from approximately 114.9 to 140.8 nanometers after PEGylation, while maintaining a low PDI (P-D-I) [3].

    - LAB MEDIA: Figure 6A. Video editor: Highlight the peak labeled 2122 in the spectrum for “Hb@HP-UiO-66-N₃ NPs”.

    - LAB MEDIA: Figure 6A. Video editor: Highlight the additional peaks in the spectrum labeled “Hb@HP-UiO-66-PEG NPs”.

    - LAB MEDIA: Figure 6C. Video editor: Highlight the DLS size distribution labeled “Hb@HP-UiO-66-PEG NPs”.