TITLE:

Synthesis, Hemoglobin Encapsulation and Biorthogonal PEGylation in Hierarchically Porous UiO-66 Nanoparticles for Oxygen Delivery Applications

AUTHORS AND AFFILIATIONS:

Weiguang Jin and Leticia Hosta-Rigau*

Department of Health Technology, Technical University of Denmark, 2800 Kongens Lyngby, Denmark

EMAIL ADDRESS OF CO-AUTHOR:

Weiguang Jin 																						(weig@dtu.dk)

*CORRESPONDING AUTHOR:

Leticia Hosta-Rigau												 (leri@dtu.dk)

KEYWORDS:

hemoglobin-based oxygen carriers; metal–organic frameworks; hierarchically porous nanoparticles; bio-orthogonal click chemistry; PEGylation/surface functionalization

SUMMARY:

This protocol details the synthesis, hemoglobin encapsulation, and surface PEGylation of hierarchically porous UiO-66 nanoparticles designed for oxygen delivery. The procedure combines defect engineering with strain-promoted biorthogonal conjugation to generate highly permeable and physiologically stable carriers suitable for biomedical applications.

ABSTRACT:

Hemoglobin-based oxygen carriers (HBOCs) are an important platform for oxygen delivery to mitigate hypoxia in several biomedical contexts. However, free hemoglobin (Hb) is unstable and vasoactive outside red blood cells. Here, we present a reproducible, scalable step-by-step protocol for fabricating hierarchically porous UiO-66 nanoparticles (HP-UiO-66 NPs) that encapsulate Hb and are stabilized by bioorthogonal polyethylene glycol (PEG) shells. Hierarchical mesoporosity is introduced by dodecanoic acid modulation and subsequent acid washing, yielding HP-UiO-66-NH₂ NPs with preserved crystallinity and combined type I/IV N2 isotherms. Surface amines are converted to azides (HP-UiO-66-N₃ NPs), enabling catalyst-free strain-promoted alkyne–azide cycloaddition (SPAAC) to produce Hb@HP-UiO-66-PEG NPs. The workflow comprises: i) defect engineering of UiO-66-NH₂ NPs, ii) modulator removal, iii) azide installation, iv) Hb loading in buffered aqueous media, and v) SPAAC-mediated PEGylation. Critical experimental parameters, standard failure modes, and troubleshooting strategies are detailed at each stage to enhance inter-laboratory reproducibility. The protocol is amenable to gram-scale production of HP-UiO-66-NH₂ NPs per batch. Encapsulation efficiencies of ~35% (loading content ~24 wt%) for Hb are obtained under mild conditions, with redispersible NPs with a hydrodynamic diameter of ~140 nm and low polydispersity after PEGylation. Comprehensive physicochemical characterization verifies the integrity of the framework, mesoporosity, successful surface chemistry, and a diffuse PEG corona up to ~12 nm wide. Functional assays show reversible oxygenation/deoxygenation of encapsulated Hb, indicating retained oxygen-transporting capacity. PEGylation markedly enhances colloidal and structural stability in saline, HEPES, and serum, preventing phosphate-induced degradation observed for non-PEGylated counterparts. By integrating detailed troubleshooting guidance, video-assisted visualization of critical steps, and scale-up considerations, this modular protocol provides a robust and accessible route to metal-organic framework-based HBOCs. The strategy is readily adaptable to other biomacromolecules or enzymes requiring protective yet accessible microenvironments for therapeutic delivery.

INTRODUCTION:

Adequate oxygenation is critical for effective therapy and tissue healing across diverse biomedical settings1. In trauma and surgical hemorrhage, severe oxygen loss can be life-threatening, highlighting the need for safe blood substitutes when donor blood is unavailable2. In oncology, hypoxic tumor regions limit the efficacy of oxygen-dependent treatments, and supplemental oxygen delivery can markedly enhance therapeutic efficacy3–6. Oxygen also plays a central role in wound healing, where increased oxygen availability promotes angiogenesis and tissue repair7. Similarly, in ischemic conditions and organ transplantation, oxygen carriers can reduce hypoxic injury and improve tissue viability and recovery8,9.

Synthetic hemoglobin (Hb)-based oxygen carriers (HBOCs) represent a promising approach to mitigate hypoxia10. However, Hb outside of red blood cells (RBCs) is inherently unstable and rapidly dissociates into dimers that can cause nephrotoxicity, oxidative damage, and vasoconstriction due to nitric oxide scavenging11. To mitigate these challenges, encapsulation strategies have been developed to stabilize Hb within biocompatible nanocarriers such as liposomes12, polymersomes13, and polymeric nanoparticles (NPs)14–17. Although these systems enhance Hb stability, achieving high encapsulation efficiency while preserving native oxygen-binding function remains challenging18. In addition, oxidative conversion to methemoglobin and heme leakage can compromise oxygen-carrying performance19. Rapid clearance and premature Hb leakage further limit circulation time and clinical translation20.

Metal-organic frameworks (MOFs) have recently emerged as versatile nanoplatforms for drug and biomacromolecule delivery due to their crystalline architecture, large surface area, and tunable porosity21. Composed of metal nodes coordinated with organic linkers, MOFs enable molecular diffusion and structural customization, making them attractive for biomedical applications. However, most MOFs possess predominantly microporous cavities (<2 nm), which severely restrict post-synthetic encapsulation of macromolecules such as proteins, nucleic acids, or enzymes22–25. One approach to overcome this limitation is in situ biomimetic mineralization, where biomolecules act as nucleation centers during MOF growth, as demonstrated for ZIF-8 and ZIF-9026–28. Despite its effectiveness, this strategy requires mild synthesis conditions, which restrict the range of compatible MOF chemistries29,30.

UiO-66, a zirconium-based MOF constructed from Zr₆O₄(OH)₄ octahedral nodes and 1,4-benzenedicarboxylate (BDC) linkers, stands out for its exceptional chemical stability and biocompatibility31. Its robust framework resists decomposition in aqueous and physiological environments, making it ideal for biomedical applications32. Nevertheless, its intrinsic microporous structure limits its utility for biomacromolecule encapsulation. Hb, with a molecular dimension of ~5 nm, cannot be accommodated within the internal pores of pristine UiO-66. Correspondingly, only a limited number of studies have explored UiO-66 in Hb-related systems, in which the MOFs are primarily used for surface-based Hb sensing rather than for framework encapsulation33,34. As a result, UiO-66-based platforms for Hb encapsulation remain relatively underdeveloped.

Very recently, we introduced a hierarchically porous UiO-66 NP (HP-UiO-66 NP) that overcomes this pore size restriction by incorporating mesoporous domains capable of accommodating Hb, thereby extending the functional scope of UiO-66 for HBOC development35. Hierarchical porosity is introduced through defect engineering, specifically by employing long-chain monocarboxylic acids, such as dodecanoic acid (DA), as modulators during synthesis36,37. This modulator competes with the BDC ligand for coordination with Zr4+, leading to defect formation and enlarged mesopores. Subsequent removal of the modulator yields HP-UiO-66 NPs containing coexisting micro- and mesoporous domains, with tunable pore size through modulator concentration or chain length36.

Although the introduction of mesoporosity improves molecular accessibility, it may also compromise framework integrity in phosphate-containing buffers. Phosphate ions can coordinate with Zr clusters, leading to partial or complete structural disintegration38,39. Accordingly, surface stabilization strategies are essential to preserve colloidal and chemical stability under physiological conditions40. In general, MOF surface stabilization can be achieved through noncovalent interactions (e.g., van der Waals interaction and hydrogen bonding)41, coordination interactions with exposed metal sites42, or covalent attachment to organic linkers43. Among different linkers, polyethylene glycol (PEG) is particularly attractive due to its broad availability and minimal synthetic complexity44. Importantly, PEG surface modification, commonly referred to as PEGylation, confers several favorable properties for drug delivery systems, including excellent biocompatibility, reduced nonspecific protein adsorption, and reduced immune recognition and macrophage uptake45,46. Collectively, these effects reduce immunogenic responses and prolong systemic circulation47.

Traditional PEGylation techniques often rely on carbodiimide or maleimide chemistry, which typically require catalysts or reactive intermediates that can damage sensitive biomolecules, such as Hb. To circumvent this limitation, we employ strain-promoted alkyne-azide cycloaddition (SPAAC), a catalyst-free, bioorthogonal reaction between azide- and dibenzylcyclooctyne (DBCO)-functionalized species, enabling site-specific conjugation under mild aqueous conditions35,48. This approach ensures efficient surface modification without compromising encapsulated Hb function.

Building on our previously reported HP-UiO-66 NPs35, which focused on structural design and biofunctional validation, the present protocol emphasizes methodological transparency and reproducibility by providing a detailed, step-by-step experimental workflow with defined parameter ranges and critical handling considerations. Troubleshooting guidance is included for key stages of synthesis, Hb encapsulation, and PEGylation via SPAAC chemistry, ultimately yielding stable Hb@HP-UiO-66-PEG NPs (see Figure 1).

The workflow involves: (i) defect engineering using DA as a modulator to synthesize amino-modified HP-UiO-66-NH2 NPs; (ii) azide functionalization of surface amine groups; (iii) encapsulation of Hb under mild, aqueous conditions; (iv) covalent PEG conjugation through SPAAC.

For each major step, this protocol identifies common experimental challenges, such as particle aggregation, pore collapse, or inefficient surface functionalization, and provides practical troubleshooting strategies to improve inter-laboratory reproducibility. Importantly, this article extensively demonstrates that the synthesis of HP-UiO-66 NPs is readily scalable, enabling reliable gram-scale production per batch.

The final PEGylated nanocarriers exhibit high colloidal stability in saline, 4-(2-hydroxyethyl)-1-piperazineethanesulfonic acid (HEPES), and serum, while maintaining Hb’s oxygen-binding and release capacity35. Analytical methods such as Fourier transform infrared spectroscopy (FTIR), nuclear magnetic resonance (NMR), powder X-ray diffraction (PXRD), nitrogen (N2) sorption, zeta (ζ)-potential analysis, dynamic light scattering (DLS), scanning electron microscopy (SEM), and transmission electron microscopy (TEM) confirm chemical transformation, framework integrity, porosity, nanoscale morphology and successful surface functionalization.

By emphasizing reproducibility, scalability, and operational clarity, supported by video-based visualization of technically sensitive steps, this methodology provides a reproducible, robust, and accessible route to fabricate stable MOF-based oxygen carriers. Importantly, the approach is modular and readily adaptable to other biomacromolecules or enzymes that require a protected yet oxygen-permeable microenvironment to retain functionality. Accordingly, Hb@HP-UiO-66-PEG NPs represent a promising step toward the creation of HBOCs capable of delivering oxygen efficiently while resisting degradation in biological fluids.

From a practical perspective, this strategy can be extended to other biomacromolecules beyond Hb, provided that they possess dimensions below approximately 20 nm, corresponding to the maximum accessible mesopore size, and exhibit one or more of the following characteristics: (i) sensitivity to harsh chemical conditions, favoring mild synthesis and surface modification; (ii) a requirement for a hydrated, oxygen-permeable microenvironment to maintain functional activity; and (iii) susceptibility to phosphate-induced MOF degradation, necessitating surface stabilization. The approach relies on moderate defect densities to balance mesopore accessibility with framework integrity. When catalyst-free, site-specific surface modification is required to preserve biomolecule function, SPAAC-based PEGylation offers clear advantages. Consequently, this protocol is well-suited for efficient biomacromolecule loading, functional preservation, and long-term colloidal stability under physiological conditions, whereas substantially larger biomolecules or systems demanding extreme thermal or acid stability may require alternative MOF platforms.

PROTOCOL:

- Hb extraction (Expected duration: 2 days):

NOTE: The extraction of Hb from commercially available bovine blood (SSI Diagnostica A/S, Hillerød, DK) is conducted following our previously reported protocol49.

  - Transfer bovine blood (typically 10 mL per extraction) into a 50 mL conical centrifuge tube. Add an equal volume of 0.9% (w/v) saline solution to dilute the blood and reduce viscosity. Gently invert several times until a homogeneous suspension is obtained.

  - Centrifuge the diluted blood at 1500 x g for 20 min at 4 °C. After centrifugation, a clear yellow supernatant (plasma) appears above a dense red pellet (RBCs).

  - Carefully decant and discard the plasma layer without disturbing the pellet. Add fresh saline (10 mL), resuspend the RBC pellet by gentle inversion, and repeat the centrifugation step under identical conditions. Perform two washing cycles in total to remove plasma proteins and platelets.

NOTE: The washing process is crucial for obtaining pure RBCs free of plasma contaminants that could interfere with the downstream toluene extraction.

  - After the second wash, discard the supernatant and resuspend the cleaned RBC pellet in a mixture of toluene, ultrapure water, and saline at the volume ratio of 1:0.4:1. For example, mix 10 mL toluene, 4 mL ultrapure water, and 10 mL saline in a 50 mL separatory funnel before adding the pellet.

  - Seal the funnel and invert gently several times to mix the phases. Place the funnel at 4 °C overnight (~12–16 h) to allow complete phase separation. The toluene phase extracts the lipid membranes, leaving a clear aqueous phase enriched in Hb.

  - The following day, carefully collect the lower aqueous layer using the stopcock of the separatory funnel. This layer appears transparent and dark red.

  - Transfer the aqueous Hb solution into centrifuge tubes and spin at 8,000 x g for 20 min at 4 °C to remove any remaining cell debris or emulsified material.

  - Filter the supernatant through a diatomaceous earth frit using a vacuum filtration setup. Apply gentle vacuum only (avoid high negative pressure that can cause foaming or protein denaturation).

  - Collect the filtrate containing stroma-free Hb. Store this stock on ice if used immediately, or aliquot (~1 mL) and freeze at −80 °C for long-term use.

NOTE: Hb aliquots may be stored frozen for several months without detectable degradation.

  - Quantify Hb concentration using a bicinchoninic acid protein assay according to the manufacturer’s protocol, using pure bovine Hb as a calibration standard. Adjust final Hb stock to the desired working concentration (typically 20 mg mL-1 in HEPES buffer, pH 7.4).

NOTE: Expect a clear, red Hb solution free of cellular material.

  - Optional: measure the UV-vis absorption spectra (375–700 nm) of extracted Hb to assess its oxygen binding and releasing ability. Record the initial UV-vis spectrum. Add sodium dithionite and purge with N₂ gas for 10 min to release oxygen, and record the spectrum of deoxygenated Hb. Subsequently, purge with air for 10 min to allow reoxygenation and re-record the spectrum.

NOTE: The presence of characteristic Soret and Q-bands of oxygenated (~412 nm for Soret band; ~541 and ~577 nm for Q-bands) and deoxygenated (~430 nm for Soret band; ~557 nm for Q-band) Hb after purging with air and N2, respectively, indicates that Hb remains functional (Supplementary Figure 1).

CAUTION: Handle toluene with care in a fume hood while wearing appropriate personal protective equipment, including a lab coat, safety glasses, and gloves. In the present study, all toluene-containing waste liquids were collected and disposed of as organic chemical liquid waste in designated containers (Group C), labeled “toluene and mixture,” in accordance with the guidelines of the Danish Ministry of the Environment. The corresponding waste-disposal documentation is provided in Supplementary File 1.

- Fabrication of HP-UiO-66-NH₂ NPs (Expected duration: 3 days):

CAUTION: HCl (37%) is corrosive; handle inside a fume hood with a lab coat, acid-resistant gloves, and appropriate eye protection. Add acid slowly and avoid inhalation of vapors.

  - Weigh ZrCl₄ (400 mg, 1.715 mmol) into a 250 mL round-bottom flask. Slowly add DMF (80 mL) and ultrasonicate for 5 min to completely dissolve ZrCl₄. Add DA (12 g, 60 mmol) and ultrasonicate for an additional 30 min until a clear, homogeneous solution is obtained. Measure the total solution volume using a graduated cylinder and add DMF as needed to adjust the final volume to 100 mL. DA acts as a competitive modulator, introducing controlled lattice defects.

NOTE: Add DMF slowly when dissolving ZrCl₄, as the coordination process is exothermic and may cause slight fuming; perform all operations in a fume hood. Account for the volume increase caused by the incorporation of DA by using an initial volume of 80 mL DMF to dissolve ZrCl₄ and DA, and adjust the final volume to 100 mL only after complete dissolution to ensure accurate concentration control.

  - Add 2-amino-1,4-benzenedicarboxylic acid (BDC-NH2, 150 mg, 0.857 mmol) to the same flask and sonicate for an additional 30 min. Continue until a homogeneous yellow suspension is obtained.

  - Transfer the mixture into a 150 mL Teflon-lined autoclave or round-bottom pressure vessel suitable for solvothermal synthesis. Seal tightly and place in a preheated muffle furnace at 120 °C for 18 h. Do not disturb the reactor during heating to avoid uncontrolled nucleation.

  - After the reaction, turn off the furnace and allow the vessel to cool naturally to room temperature (~2 h). Observe a fine yellow suspension appearing uniformly throughout the medium.

  - Collect the product by centrifugation (11,000 x g, 10 min, room temperature) and discard the supernatant. Wash the pellet with DMF (100 mL) three times using the same centrifugation conditions to remove excess ligand and unreacted DA.

  - Prepare an acid removal solution consisting of DMF (100 mL) containing 0.5 mL concentrated HCl (37%). Immerse the washed pellet completely in this solution and heat it in a muffle furnace at 90 °C for 12 h. This step removes the DA modulator, creating mesopores within the structure.

  - After 12 h, centrifuge (11,000 x g, 5 min, room temperature) and discard the supernatant. Repeat the acid treatment using a fresh batch of DMF containing 0.5 mL HCl to ensure complete removal of DA.

  - Wash the pellet sequentially with DMF, acetone, and ethanol (100 mL, 11,000 x g, 5 min, room temperature) to remove any residual acid and solvent. Between each wash, gently redisperse the pellet by vortexing or mild sonication.

  - Dry the product in a vacuum oven (140 °C) overnight. The final materials (HP-UiO-66-NH₂ NPs) appear as fine yellow powder. Expect a typical yield of ~428 mg.

  - Confirm that PXRD peaks at 2θ = 7.3°, 8.4°, 25.7° remain sharp (Figure 2A), indicating retention of crystalline UiO-66 topology. Verify complete removal of DA by 1H NMR after alkaline (2 M KOH in D₂O) digestion (no methylene peaks at 1.2 ppm) (Figure 2B).

  - Characterize porosity by N₂ adsorption–desorption at 77 K50. The curve displays a combined Type I/IV isotherm with a hysteresis loop (Figure 2C), confirming mesopore formation.

NOTE: Incomplete high-temperature drying or insufficient sample mass leads to inaccurate porosity measurements using N2 adsorption–desorption isotherms; repeat the vacuum-drying process at 140 °C and ensure the sample weight exceeds 100 mg if the Brunauer−Emmett−Teller (BET) area is < 1,000 m2 g-1.

  - Optional: assess NPs' morphology using SEM. HP-UiO-66-NH₂ NPs exhibit spherical morphology with a narrow size distribution of ~100 nm (Figure 2D).

NOTE: Expect HP-UiO-66-NH₂ NPs possessing both micro- and mesopores, suitable for subsequent Hb encapsulation.

  - The synthesis of HP-UiO-66-NH₂ NPs can be scaled up by proportionally increasing all reagents and solvents used in steps 2.1–2.9 and performing multiple parallel solvothermal reactions. By maintaining the same molar ratios, concentrations, reaction temperature, and reaction time, and increasing reagent and solvent quantities by 1.5-fold, two or more 150 mL Teflon-lined autoclaves or round-bottom pressure vessels can be operated simultaneously, enabling gram-scale production of HP-UiO-66-NH₂ NPs (Figure 3).

NOTE: Handle all organic solvents inside a fume hood while wearing a lab coat, acid-resistant gloves, and appropriate eye protection. Collect all DMF-containing waste liquids and dispose of them as organic chemical liquid waste in designated containers (Group C) labeled “DMF and mixture” in accordance with the guidelines of the Danish Ministry of the Environment. Waste solutions containing trace amounts of ZrCl₄ and HCl were handled accordingly. The corresponding waste-disposal documentation is provided in Supplementary File 1.

- Azide functionalization of HP-UiO-66-NH2 NPs (HP-UiO-66-N3 NPs) (Expected duration: 1 day):

CAUTION: Trimethylsilyl azide (TMSN₃) and tert-butyl nitrite (tBuONO) are toxic and volatile. Conduct all steps in a fume hood wearing a lab coat, nitrile gloves, and eye protection.

  - Accurately weigh out HP-UiO-66-NH2 NPs (50 mg) into a 25 mL round-bottom flask. Add THF (10 mL) and ultrasonicate for 5 min to fully disperse the particles. The suspension turns opaque but uniform.

NOTE: Uniform dispersion of NPs after sonication is critical for effective contact with reactants. If the dispersion is incomplete, increase the solution volume and repeat sonication.

  - Place the flask in an ice-water bath (0 °C) on a magnetic stirrer set to 400 rpm. Using a glass syringe, add tBuONO (0.4 mL) dropwise over 5 min while maintaining the temperature below 5 °C.

  - Immediately add TMSN₃ (0.35 mL) dropwise under the same cold conditions. A mild effervescence may appear, indicating nitrosation progress. Wrap the ice–water bath in aluminum foil to keep the reaction protected from light.

  - Continue stirring the reaction mixture overnight (~16 h) at 0 °C to allow complete azide substitution of amine groups on the MOF surface.

  - Remove the ice bath and allow the suspension to reach room temperature. Centrifuge (11,000 x g, 5 min, room temperature) to collect the modified NPs.

  - Wash the pellet thoroughly with THF (10 mL) three times, collecting it by centrifugation (11,000 x g, 5 min, room temperature) after each wash. Follow this with three additional washes using acetone (10 mL), centrifuging under the same conditions, to remove residual reagents and by-products.

  - Dry the product (HP-UiO-66-N3 NPs) using an oven (140 °C) under vacuum overnight. Expect a typical yield of ~45 mg.

  - Verify the chemical transformation by FTIR spectroscopy: a sharp asymmetric azide stretch appears at ~2122 cm-1 (Figure 4A). The band intensity correlates with surface substitution efficiency.

NOTE: Incomplete conversion can impair subsequent click conjugation. If the azide band is weak, repeat the functionalization using 2-fold quantities of tBuONO and TMSN₃. Expect azido-terminated HP-UiO-66-N3 NPs with preserved crystallinity and mesoporous architecture (Figure 4B–D), ready for Hb encapsulation and SPAAC PEGylation.

CAUTION: Collect all waste liquids containing TMSN₃, including mixtures with other reagents (e.g., tBuONO and THF), in designated Group O hazardous waste containers in accordance with the guidelines of the Danish Ministry of the Environment. Clearly label all containers to indicate the presence of TMSN₃ and its mixtures. Relevant waste disposal documentation is provided in Supplementary File 1.

- Hb Loading and Quantification (Expected duration: 2 days):

  - Transfer HP-UiO-66-N₃ NPs (5 mg) into a 2 mL centrifuge tube. Add 0.5 mL of HEPES buffer (20 mM, pH 7.4) containing Hb (10 mg mL-1) and gently invert the tube several times to fully disperse the NPs.

  - Place the tube on a thermoshaker and incubate at room temperature (22–25 °C) under 1,000 rpm shaking for 2 h.

  - After incubation, centrifuge 13,500 x g for 5 min at room temperature to pellet the resulting Hb@HP-UiO-66-N3 NPs. Collect the supernatant carefully for quantification of unentrapped Hb.

  - Wash the pellet gently with ultrapure water (1 mL) to remove loosely adsorbed Hb, and repeat the wash twice. Carefully collect the supernatant of each wash for quantification of unentrapped Hb. Resuspend the final pellet in 0.5 mL ultrapure water for characterization. Keep samples in refrigerator (4 °C) to prevent oxidation.

  - Determine unentrapped Hb in the collected supernatants using the sodium lauryl sulfate (SLS)-Hb assay51. Pipette 10 µL of each supernatant into a 96-well microplate and add 100 µL of SLS reagent (0.6 mg mL-1 in ultrapure water). Incubate 5 min at room temperature in the dark, then read absorbance at 412 nm using a UV-vis spectrophotometer.

  - Prepare a standard curve using Hb stock solutions (0–10 mg mL-1) in HEPES buffer to convert absorbance values to concentration.

  - Calculate encapsulation efficiency (EE) and loading content (LC) as:

EE (%) = [(initial Hb – supernatant Hb)/initial Hb] × 100

LC (%) = [(encapsulated Hb)/(encapsulated Hb + NP mass)] × 100

  - To scale up the Hb encapsulation, incubate 100 mg of NPs with 10 mL of Hb solution (10 mg mL⁻¹). To assess the porosity after encapsulation, immerse the resulting NPs in ethanol for 24 h, then dry overnight in a vacuum oven at 140 °C, and record the N₂ adsorption–desorption isotherm to evaluate porosity. The red color of the powder indicates successful encapsulation of Hb (Figure 5A), and a marked decrease in N₂ uptake confirms the successful filling of mesopores with Hb (Figure 5B).

  - Measure ζ-potential using a Zetasizer Nano ZS. The surface charge decreases from ~+40 mV (HP-UiO-66-N3 NPs) to ~+20 mV (Hb@HP-UiO-66-N3 NPs), consistent with Hb adsorption (Figure 5C).

  - Optional: acquire SEM micrograph (Figure 5D) and PXRD diffractogram (Figure 5E) to ensure morphological integrity and structural stability.

  - Record UV-vis spectra (375–700 nm) to assess reversible oxygen binding of the NPs. Add sodium dithionite and purge with N₂ for 10 min to release oxygen, record the spectrum, then purge with air for 10 min to allow re-binding, and record the spectrum again (Figure 5F).

NOTE: The presence of characteristic Soret and Q-bands of oxygenated (~412 nm for Soret band; ~541 and ~577 nm for Q-bands) and deoxygenated (~430 nm for Soret band; ~557 nm for Q-band) Hb after purging with air and N2, respectively, indicates that Hb remains functional.

  - Expect uniform red Hb@HP-UiO-66-N3 NPs containing functionally active Hb with reversible oxygenation behavior.

- Fabrication of Hb@HP-UiO-66-PEG NPs: PEGylation via SPAAC (Expected duration: 2 days):

  - Prepare a 2 mL centrifuge tube containing Hb@HP-UiO-66-N₃ NPs (10 mg) dispersed in 0.5 mL ultrapure water. Add DBCO-PEG (Mw ~2,000, 10 mg in 0.5 mL ultrapure water) to achieve a 1:1 (w/w) ratio of PEG to NPs. Gently vortex until fully mixed.

  - Incubate the mixture on a thermoshaker at room temperature and 1,000 rpm for 1 h, then transfer to a refrigerator (4 °C) for overnight reaction (~16 h). This catalyst-free SPAAC click reaction covalently couples DBCO to the azide groups, forming stable triazole linkages.

  - Centrifuge at 12,000 x g for 5 min at room temperature to pellet the PEGylated Hb@HP-UiO-66-PEG NPs. Discard the supernatant containing unreacted PEG.

  - Wash the pellet three times with ultrapure water (2 mL each) under identical centrifugation conditions. This ensures the removal of free PEG chains and small molecular by-products.

  - Resuspend the purified Hb@HP-UiO-66-PEG NPs in 2 mL ultrapure water and store at 4 °C.

The colloid appears homogeneous and slightly opalescent, indicating good dispersion stability.

  - Confirm successful PEGylation by FTIR spectroscopy (Figure 6A): disappearance of the azide band at 2122 cm-1 and emergence of PEG-derived methylene (−CH2−) and ether bond (−O−) signals at 2883 and 1097 cm-1, respectively.

  - Verify by ¹H NMR (Figure 6B) after alkaline digestion in 2 M KOH/D₂O; PEGylated samples show an additional signal at δ = 3.53 ppm corresponding to the oxyethylene protons of PEG.

  - Measure hydrodynamic diameter and polydispersity index (PDI) by DLS (Figure 6C).

An increase from ~115 nm (PDI ~0.1) to ~140 nm (PDI ~0.17) confirms surface coating with a thin polymer layer.

  - Optional: measure surface ζ-potential using a Zetasizer Nano ZS to verify the negligible effect of surface PEGylation on the surface charge (Figure 6D). A minor change after PEGylation (from ~+20 mV to ~+17 mV) indicates minimal impact on surface charge.

  - Observe morphology by TEM (Figure 6E). TEM reveals a diffuse ~10–12 nm rim around the particle edge representing the PEG corona.

  - Perform oxygen-release assay using an oxygen electrode (Figure 6F) to show that Hb function remains intact after PEGylation, similar to the non-PEGylated form. Expect a stable, red-dispersed Hb@HP-UiO-66-PEG NPs exhibiting strong colloidal stability and retained oxygen-binding capacity, suitable for long-term physiological testing.

  - Optional: conduct oxygen dissociation curves (ODCs) of free Hb and Hb encapsulated in Hb@HP-UiO-66-PEG NPs to identify the partial oxygen pressure at 50% Hb oxygen saturation (p50) and Hill coefficient (Hill-n) (Figure 7).

- Assessment of NPs stability (Expected duration: 7 days):

  - Dispense Hb@HP-UiO-66-PEG NPs (~5 mg) into seven 2 mL centrifuge tubes. Add 2 mL of each medium: ultrapure water, saline (0.9% NaCl), HEPES (20 mM), mouse serum, human plasma, Dulbecco’s modified Eagle’s medium−high glucose (DMEM), and phosphate-buffered saline (PBS). Ensure that the centrifuge tube caps are tightly closed to prevent medium evaporation during incubation. CD-1 mouse serum and human plasma are purchased from Innovative Research (Novi, MI, US).

  - Incubate the samples at 37 °C in a thermoshaker at 1,000 rpm for up to 7 days.

  - After 3 and 7 days, collect particles by centrifugation (12,000 g, 10 min, room temperature), wash once with ultrapure water, and dry for PXRD (Figure 8A).

  - At 0, 24, 72, and 168 h, withdraw aliquots (~50 μL) and dilute them 20-fold for DLS analysis (size and PDI) (Figure 8B).

  - Optional: prepare SEM samples after 48 h in PBS to visually check morphology (Figure 8C).

NOTE: Diluting the NPs suspension (in PBS) with ultrapure water is essential for SEM analysis as it reduces background interference from buffer components and facilitates clear imaging of the NPs.

  - Compare to non-PEGylated Hb@HP-UiO-66-N3 NPs under identical conditions; rapid disappearance of characteristic crystalline reflections and emergence of broad amorphous scattering features indicate instability without PEG coating.

NOTE: PEGylated samples remain structurally intact after 7 days in all media, while non-PEGylated ones decompose within 24 h in PBS or DMEM.

- Physicochemical characterization (Expected duration: ~15 days):

NOTE: This section describes the analytical methods required to confirm NPs' morphology, composition, porosity, and Hb integrity. These characterization techniques collectively validate the reproducibility of the synthesis and modification processes.

  - PXRD

    - Dry ~5 mg of each NP sample in a freezer drier overnight before measurement to remove adsorbed moisture.

    - Load the sample onto a flat zero-background silicon holder and gently flatten using a clean glass slide to form a thin, uniform layer.

NOTE: Avoid pressing too hard, which can induce preferred orientation effects.

    - Record PXRD patterns using a benchtop diffractometer equipped with Cu Kα radiation (λ = 1.5418 Å). Set scanning parameters to 2θ = 4–50°, step size = 0.02°, dwell = 1 s per step.

    - Compare the obtained patterns with the simulated UiO-66 structure generated using Mercury software based on the previously reported crystal data31. The presence of sharp peaks at 7.3°, 8.4°, and 25.7° indicates crystalline integrity.

  - N2 Adsorption–Desorption (BET Surface Area and Pore Size Distribution)

    - Degas ~100 mg of each NP sample at 140 °C under vacuum for 24 h using a Micromeritics Smart VacPrep system with purging ultra-high-purity N2.

    - Immediately transfer the degassed samples to the analysis port of a Micromeritics Tristar surface area analyzer. Record N₂ isotherms at 77 K within a relative pressure range (P/P₀) of 0–1.

    - Compute surface areas using the BET method following the extended Rouquerol criteria52. Calculate pore size distribution via the non-local density functional theory implemented in the Micromeritics software.

    - Generate the adsorption/desorption curves and pore size distribution plots using Origin 2022. Import the adsorption/desorption data into the software, with relative pressure (P/P₀) on the X-axis and the adsorbed volume of N₂ on the Y-axis.

    - Plot the pore size distribution data with pore width as the X-axis and incremental pore volume as the Y-axis. Present the results as “Line + Symbol” graphs. Perform all analyses using the default software settings.

  - FTIR Spectroscopy

    - Grind ~5 mg of each dried sample into a fine powder using a clean agate mortar.
Place the powder on the ATR crystal of an ALPHA-P Bruker spectrometer53.

    - Collect spectra in the 4,000–400 cm-1 range with 2 cm-1 resolution.

    - Identify characteristic peaks: Appearance of N3 stretch at ~2122 cm-1 confirms azide formation (Figure 4A). After PEGylation, observe −CH2− and C–O–C bands at ~2883 cm-1 and ~1097 cm-1, respectively, along with the disappearance of N3 stretch (Figure 6A). Expect the disappearance of the azide band and emergence of PEG-derived methylene (−CH2−) and ether bone (−O−) signals confirm the completion of the SPAAC reaction.

    - Plot the FTIR curves using Origin 2022. Import the raw data into the software with X (wavenumber) and Y (transmittance) values. Generate graphs in “Line” format using the software's default settings.

  - ¹H NMR

    - Add ~10 mg of NPs to 0.5 mL of 2 M KOH in D₂O placed in a glass vial, and heat at 80 °C for 1–2 h to digest the framework.

    - Centrifuge (12,000 x g, 10 min, room temperature) to spin down the undissolved residues. Transfer the supernatant to a 5 mm NMR tube.

    - Record 1H NMR spectra on a 400 MHz Bruker Avance spectrometer.

    - Assign the characteristic chemical shifts, including the aromatic protons of BDC-NH₂ (δ = 6–8 ppm) and PEG signals (δ = ~3.53 ppm, Figure 6B), using the software of MestReNova. Analyze the NMR data using the software's NMR processing module with its default settings.

  - SEM

    - Deposit ~5 µL drop of NP suspension onto a silicon wafer; allow to air-dry in a dust-free cabinet. If charging occurs, sputter-coat with 5 nm gold.

    - Image on a FEI Quanta 200 SEM at 10–20 kV accelerating voltage.

    - Analyze the images using ImageJ. Import the original TIFF images with embedded scale information into the software and adjust brightness and contrast solely for visualization. Do not apply any additional processing that might affect the dimensional information. Generate the scale bars using the software's Analyze function.

  - TEM

    - Place ~5 µL of NP dispersion on a lacey-carbon Cu grid (300 mesh). Blot excess with filter paper and dry at ambient conditions.

    - Examine under a FEI Tecnai T20 G2 TEM at 200 kV.

    - Analyze the images using ImageJ. Image processing follows the procedures described in Section 7.5.3.

  - DLS and ζ-Potential

    - Dilute NPs ~20-fold in ultrapure water for size and PDI analysis; for ζ-potential, dilute in HEPES buffer (20 mM, pH 7.4).

    - Measure using a Zetasizer Nano ZS at 25 °C. At least three independent samples (n = 3) are prepared, and each sample is measured in triplicate.

    - Plot the DLS and ζ-potential data using Origin 2022. Present the intensity-weighted particle size distribution as “Line + Symbol” graphs and ζ-potential results as “Column” graphs. Report the data as mean ± standard deviation (n = 3). Perform all analyses using the software's default settings.

  - UV-vis Spectroscopy for Hb functionality

    - Disperse 3 mg of free Hb or a corresponding amount of NPs (i.e., containing an equivalent amount of encapsulated Hb) in 3 mL ultrapure water. Transfer the suspensions to a quartz cuvette. Record UV-vis spectra from 375–700 nm using a UV-2600 UV−vis spectrophotometer.

    - Add a pinch of sodium dithionite to the Hb solution or NP suspension and purge with N2 for 10 min to deoxygenate the sample. Record the shift of the Soret band from 412 to 430 nm and the merging of Q-bands. Reoxygenate the sample by air bubbling for 10 min to observe reversal (Figure 5F and Supplementary Figure 1).

    - Plot the data using Origin 2022. Present the spectra as “Line” graphs, with wavelength on the X-axis and absorbance on the Y-axis.

  - Quantification of released oxygen

    - Disperse 1 mg of free Hb or a corresponding amount of Hb@HP-UiO-66-N3 and Hb@HP-UiO-66-PEG NPs (containing an equivalent amount of encapsulated Hb) in 600 μL ultrapure water. Transfer the suspensions into 2 mL glass vials and seal them tightly with rubber caps.

    - Carefully insert the needle probe of the oxygen electrode through the rubber cap into the suspension to monitor the dissolved oxygen level using a needle-type O2 meter.

    - Quickly add K3[Fe(CN)6] solution (10 wt% in 100 μL ultrapure water) to the suspension and continuously record the change in oxygen level.

    - Calculate the released oxygen by subtracting the maximum value observed after K3[Fe(CN)6] addition from the initial value before its addition. Normalize the obtained values to the oxygen released from the same amount of free Hb (Figure 6F) using Origin 2022. Measure three independent samples (n = 3) for free Hb or each NP.

NOTE:  Hb@HP-UiO-66-N3 and Hb@HP-UiO-66-PEG NPs show comparable oxygen-release, each releasing ~60% of the oxygen released by free Hb.

  - Measurement of ODCs and calculation of p50 and Hill-n

    - Prepare free bovine Hb or Hb@HP-UiO-66-PEG NPs at a final Hb concentration of ~1 mg mL-1 in 3 mL HEPES–saline buffer (20 mM HEPES, 0.9% (w/v) NaCl, pH 7.4). Transfer the sample into a quartz cuvette equipped with a magnetic stirring bar.

    - Place the cuvette into a Hemox analyzer. Set the measurement temperature to 37 °C and allow the system to equilibrate under continuous stirring until a stable baseline is achieved.

    - Record ODCs by stepwise deoxygenation over a partial pressure range of 1–148 mmHg. Use the pre-installed Hemox analyzer software to measure ODCs and calculate p50 and Hill-n using the default analysis settings. Measure three independent samples (n = 3) for each sample.

    - Export the processed data and plot the ODCs using Origin 2022 in the default “Line” graph format.

- Reproducibility and statistical analysis

  - Perform all procedures described in this protocol at least three independent times. Report results as typical outcomes observed across these repetitions. Maintain the key experimental parameters, such as reaction times and temperatures, within a reasonable range to ensure reproducibility and reliability for other users.

  - Present structural characterization (PXRD, N₂ adsorption–desorption, FTIR, ¹H NMR, SEM, UV–vis) as representative data for qualitative confirmation without performing statistical analysis.

  - Report quantitative measurements (DLS, ζ-potential, p50, Hill-n) as mean ± SD (n ≥ 3). Determine statistical significance using unpaired two-tailed Welch’s t-test or one-way ANOVA, as appropriate. Indicate significance levels as ns (no significant), * = p < 0.05, ** = p < 0.005, *** = p < 0.001, **** = p < 0.0001.

REPRESENTATIVE RESULTS:

Successful synthesis of HP-UiO-66-NH₂ NPs was verified by PXRD (Figure 2A), which exhibited the characteristic diffraction peaks at 2θ = 7.3°, 8.4°, and 25.7°, in good agreement with the simulated UiO-66 structure36. The slight peak broadening relative to the simulated pattern was attributed to the nanoscale dimensions of particles and confirmed preservation of crystalline integrity following synthesis54. Complete removal of DA was confirmed by 1H NMR spectroscopy after alkaline digestion, as evidenced by the absence of the characteristic methylene proton signal of DA at 1.2 ppm (Figure 2B). N2 adsorption–desorption isotherms displayed a combined type I and type IV profile, with a hysteresis loop observed between P/P₀ = 0.8–1.0 (Figure 2C), indicating coexistence of microporous and mesoporous domains55. Under optimal conditions, HP-UiO-66-NH2 NPs should exhibit an N₂ uptake of 700–800 cm3g-1 and a corresponding BET surface area of approximately 1070 m2g-1. Complete removal of DA and sufficient vacuum activation at 140 °C were identified as critical parameters for achieving these values; incomplete activation resulted in markedly reduced N2 uptake and BET surface area. Pore-size distribution analysis revealed a broad mesopore population ranging from 2 to 20 nm within HP-UiO-66-NH2 NPs (Figure 2C, inset), providing pore dimensions compatible with Hb encapsulation. SEM micrographs further revealed uniform, spherical NPs with an average diameter of approximately 100 nm (Figure 2D).

The synthetic protocol was further demonstrated to be scalable, as HP-UiO-66-NH₂ NPs were successfully produced using a 1.5-fold increase in reagent quantities via two parallel solvothermal reactions, yielding approximately 1.2 g of product with structural features comparable to those obtained at a smaller scale, thereby demonstrating practical gram-scale reproducibility (Figure 3).

Building on the well-defined hierarchical porosity and robust framework structure of HP-UiO-66-NH₂ NPs, the NPs were subsequently functionalized via a SPAAC-compatible azide modification prior to Hb encapsulation. Surface-exposed amino groups were selectively converted to azides through simultaneous treatment with tBuONO and TMSN₃, as confirmed by the appearance of a characteristic asymmetric azide stretching vibration at 2122 cm-1 in the FTIR spectrum of the resulting HP-UiO-66-N3 NPs, which was absent in the HP-UiO-66-NH₂ precursor (Figure 4A). PXRD patterns of HP-UiO-66-N₃ NPs were indistinguishable from those of HP-UiO-66-NH₂ NPs, indicating that framework crystallinity and structural integrity were preserved during azide functionalization (Figure 4B). Importantly, N₂ adsorption–desorption isotherms and corresponding pore size distribution analysis confirmed retention of the hierarchical micro–mesoporous architecture, with mesopore sizes remaining in the 2–20 nm range (Figure 4C,D), thereby maintaining pore accessibility for subsequent Hb encapsulation and functional performance.

Hb encapsulation was visually apparent from a distinct yellow-to-red color change (Figure 5A). The EE reached approximately 35%, corresponding to an LC of approximately 24 wt% at an input concentration of 10 mg mL-1 Hb. At lower input concentrations (<8 mg mL-1), Hb encapsulation was insufficient, resulting in reduced LC. Consistent with successful pore filling, a pronounced decrease in N2 uptake was observed after Hb loading (Figure 5B), confirming occupation of mesopores by Hb molecules. The ζ-potential decreased from 42.9 ± 1.4 mV (for HP-UiO-66-N3 NPs) to 20.4 ± 1 mV for Hb@HP-UiO-66-N3 NPs, indicating that electrostatic complementarity, in addition to physical confinement within the pores, contributed to Hb encapsulation (Figure 5C)56. The NPs retained their morphological and structural integrity following Hb encapsulation, as demonstrated by SEM micrographs showing intact spherical particles (Figure 5D) and unchanged PXRD pattern (Figure 5E). Furthermore, UV–Vis spectroscopy confirmed preservation of Hb functionality (Figure 5F): the presence of a Soret band at 412 nm and Q-bands at 541 and 577 nm indicated oxygenated Hb, while addition of an oxygen scavenger shifted the Soret band to 430 nm and merged the Q-bands into a single peak at 557 nm, consistent with formation of deoxygenated Hb. Subsequent air purging fully restored the original oxygenated spectrum, and these transitions were reversible over multiple oxygenation–deoxygenation cycles, demonstrating that encapsulated Hb retained its native oxygen-binding and release capability.

Following Hb loading, surface PEGylation of Hb@HP-UiO-66-N3 NPs was achieved via SPAAC chemistry, yielding Hb@HP-UiO-66-PEG NPs. The disappearance of the azide stretching vibration at 2122 cm-1 in the FTIR spectrum, together with the emergence of PEG-associated methylene (−CH2−) and ether bond (−O−) signals, confirmed successful PEGylation (Figure 6A). 1H NMR analysis further verified PEG conjugation by the appearance of a resonance at 3.53 ppm, corresponding to the oxyethylene protons of DBCO-PEG (Figure 6B). DLS analysis showed an increase in hydrodynamic diameter from 114.9 ± 1.8 to 140.8 ± 8.3 nm, while maintaining a low PDI (≤ 0.17) (Figure 6C). A slight decrease in ζ-potential was observed (from 20.4 ± 1 to 17.3 ± 0.3 mV), indicating a minor change of surface charge upon PEGylation (Figure 6D). TEM micrographs revealed a faint corona of up to ~12 nm surrounding each NP, consistent with the presence of a PEG coating layer (Figure 6E).

Importantly, surface PEGylation did not compromise Hb functionality. Oxygen release measurements using an oxygen electrode showed that the encapsulated Hb in Hb@HP-UiO-66-N₃ NPs remained 64.3 ± 8.3% activity relative to free Hb (100 ± 10.5%), while Hb@HP-UiO-66-PEG NPs retained 61.9 ± 7.2% (Figure 6F). ODC analysis revealed a modest rightward shift for Hb@HP-UiO-66-PEG, with p50 increasing from 27.4 ± 2.6 to 33.9 ± 2.7 mmHg, indicating slightly reduced oxygen affinity and enhanced oxygen release (Figure 7A). Notably, Hill-n remained 1.8 ± 0.1, lower than free Hb (2.4 ± 0.1) but still above 1 (Figure 7B), demonstrating preserved positive cooperatively essential for physiological oxygen delivery57.

Stability studies indicated that PEGylated Hb@HP-UiO-66-PEG NPs maintained well-defined PXRD reflections for at least 7 days in all tested media (Figure 8A). In contrast, non-PEGylated NPs degraded rapidly in PBS and DMEM, losing crystallinity within 24 h (Supplementary Figure 2). DLS analysis further confirmed excellent colloidal stability of PEGylated NPs, with a stable, monomodal size distribution in PBS and DMEM (Figure 8B). SEM micrographs showed that Hb@HP-UiO-66-PEG NPs preserved their spherical morphology after 48 h incubation in PBS (Figure 8C), confirming that surface-grafted PEG chains provided an effective steric barrier against aggregation and enhanced colloidal stability40.

Collectively, these results establish a clear structure–function relationship, demonstrating that hierarchical porosity of HP-UiO-66 NPs enables efficient Hb encapsulation, while SPAAC-mediated PEGylation confers physiological stability without compromising oxygen-binding, release, or cooperative transport. This work therefore establishes a robust, generalizable protocol for PEGylated hierarchically porous MOF NPs suitable for stabilizing and transporting Hb and other biomacromolecules.

FIGURE AND TABLE LEGENDS:

Figure 1. Schematic representation of the synthesis and modification workflow. (A) Formation of DA-UiO-66-NH2 NPs via solvothermal reaction. (B) Conversion to HP-UiO-66-NH2 NPs after DA removal. (C) Azide substitution, Hb encapsulation, and surface PEGylation by SPAAC. Figures were adapted with permission from Jin et al.35; Copyright 2025 American Chemical Society.

Figure 2. Structural validation of HP-UiO-66-NH2 NPs. (A) PXRD patterns of UiO-66-NH2 and HP-UiO-66-NH2 NPs show identical peak positions. (B) 1H NMR reveals the incorporation and complete removal of DA. (C) N2 isotherms and pore-size distributions (inset) confirm mesoporosity. (D) SEM micrograph illustrates spherical morphology. Figures A and B were reprinted with permission from Jin et al.35. Figure C was adapted with permission from Jin et al.35; Copyright 2025 American Chemical Society.

Figure 3. Scale-up synthesis of HP-UiO-66-NH2 NPs. (A) Pale yellow solution prior to synthesis. (B) Heating in a muffle furnace. (C) A yellow suspension formed after synthesis. (D) Yellow solid powder (gram-scale) obtained.

Figure 4. Structural validation of HP-UiO-66-N3 NPs. (A) FTIR spectra of HP-UiO-66-N3 NPs confirm the presence of azido groups. (B) PXRD patterns of UiO-66-NH2 and HP-UiO-66-N3 NPs show identical peak positions. (C) N2 isotherms and (D) pore-size distributions of HP-UiO-66-N3 NPs confirm mesoporosity. Figures A and B were adapted with permission from Jin et al.35. Figures C and D were reprinted with permission from Jin et al.35; Copyright 2025 American Chemical Society.

Figure 5. Hb encapsulation and functional assessment. (A) Photographs showing the color change of NP powders before/after Hb loading. (B) N2 adsorption decreases upon Hb loading. (C) ζ-potential decreases upon Hb loading. Data is presented as mean ± SD (n ≥ 3). **** = p < 0.0001 (unpaired two-tailed Welch’s t-test). (D) SEM micrograph reveals intact spherical morphology. (E) PXRD pattern remains intact after Hb encapsulation within Hb@HP-UiO-66-N3 NPs. (F) UV-vis spectra of Hb@HP-UiO-66-N3 NPs demonstrate their ability to reversibly bind and release oxygen. Figures B and E were adapted with permission from Jin et al.35; Copyright 2025 American Chemical Society.

Figure 6. Confirmation of PEGylation. (A) FTIR spectra show the disappearance of azide stretch and the appearance of PEG-derived methylene (−CH2−) and ether bond (−O−) signals. (B) 1H NMR spectrum displays PEG signature at 3.53 ppm. (C) DLS distribution plot of NPs and corresponding mean sizes (inset) presented as mean ± SD (n = 3). * = p < 0.05 (unpaired two-tailed Welch’s t-test). (D) ζ-potential before and after PEGylation, presented as mean ± SD (n ≥ 3). *p < 0.05 (unpaired two-tailed Welch’s t-test). (E) TEM micrograph of Hb@HP-UiO-66-PEG NPs displays a surface PEG layer. (F) Oxygen released by Hb@HP-UiO-66-PEG NPs relative to Hb@HP-UiO-66-N3 NPs and free Hb. Data is presented as mean ± SD (n = 3). * = p < 0.05 (one-way ANOVA); ns, not significant (unpaired two-tailed Welch’s t-test). Figures A and E were adapted with permission from Jin et al.35. Figure B was reprinted with permission from Jin et al.35; Copyright 2025 American Chemical Society.

Figure 7. Preserved oxygen affinity and cooperativity of Hb. (A) ODCs of free Hb and Hb@HP-UiO-66-PEG NPs. (B) p50 and Hill-n of free Hb and Hb@HP-UiO-66-PEG NPs. Data is presented as mean ± SD (n = 3). * = p < 0.05 compared with Hb (unpaired two-tailed Welch’s t-test).

Figure 8. Long-term colloidal stability of Hb@HP-UiO-66-PEG NPs. (A) Identical PXRD patterns of NPs after 3 and 7 days in different media. (B) Stable DLS spectra of NPs after incubation (i) in PBS and (ii) in DMEM for up to 7 days. (C) SEM micrograph shows preserved morphology of NPs after 48 h PBS incubation. Figures were reprinted with permission from Jin et al.35; Copyright 2025 American Chemical Society.

Supplementary Figure 1: UV-vis spectra of free Hb following purging with N2 and compressed air for several cycles, demonstrating the ability to reversibly bind and release oxygen.

Supplementary Figure 2: PXRD patterns of Hb@HP-UiO-66-N3 NPs after incubation in various physiologically relevant solutions for 24 h at 37 °C. The NPs retained their PXRD patterns in ultrapure water but lost crystallinity when exposed to DMEM or PBS. The simulated PXRD pattern of UiO-66 is shown for comparison.

Supplementary Figure 3: Pore size distribution of Hb@HP-UiO-66-PEG NPs, showing that the porous structure is retained, allowing oxygen diffusion.

Supplementary Figure 4: N2 isotherms of Hb@HP-UiO-66-PEG NPs, confirming that the porous structure is retained.

Supplementary Figure 5: The identical PXRD patterns of Hb@HP-UiO-66-N3 and Hb@HP-UiO-66-PEG NPs indicate preserved structural stability. The simulated PXRD pattern of UiO-66 is shown for comparison.

Supplementary File 1: Waste disposal of Toluene, DMF, ZrCl4, low concentration HCl, TMSN3, tBuONO, and THF, respectively.

DISCUSSION:

This protocol presents a robust and reproducible route to synthesize, functionalize, and characterize HP-UiO-66 NPs designed for Hb encapsulation and oxygen-delivery applications. By adopting an inside-out engineering approach, this work establishes a generalizable framework for the design of MOF NPs for biomedical applications, including biomacromolecule and drug delivery. While the underlying material concept and functional performance have been reported previously, the present protocol significantly extends prior work by emphasizing operational clarity, reproducibility, and scalability. Detailed handling steps, troubleshooting guidance for sensitive stages, and practical instructions for gram-scale synthesis are provided to facilitate reliable implementation across laboratories. Several steps are particularly critical for achieving consistent NPs with optimal structural and functional properties, as discussed below.

Defect synthesis of HP-UiO-66-NH₂ NPs.

The defect-engineering strategy relies on introducing DA as a modulator that competes with the primary linker, BDC-NH₂, for coordination to Zr4+ nodes. This competition intentionally generates “missing-linker” defects that are subsequently converted into accessible mesopores upon removal of the modulator. The molar ratio of Zr:BDC-NH₂:DA is a key determinant of the balance between framework integrity and mesopore density. As optimized in the prior study by Cai, a ratio of 1:0.5:35 provides sufficient defect formation to generate mesoporosity while maintaining crystal ordering and structural stability36. Deviations from this ratio disrupt this balance: excess linker or insufficient modulator yields inadequate mesoporosity, whereas insufficient linker or excessive DA compromises crystallinity and framework stability.

Complete removal of DA is essential to ensure pore accessibility. Acidic washing in DMF containing ~0.5% HCl at 90 °C for 12 h must be repeated to fully extract residual DA. Lower temperatures or shorter extraction times result in incomplete removal, leading to pore blockage and reduced accessibility. Conversely, excessively high acid concentrations (≥6 M HCl) can degrade the NPs. This mild acid-mediated treatment effectively removes DA while preserving framework integrity, yielding an optimal mesoporous structure suitable for Hb loading. Successful DA removal and structural preservation are confirmed by unchanged PXRD patterns (Figure 2A) and the disappearance of the characteristic methylene signal at 1.2 ppm in ¹H NMR spectra (Figure 2B), respectively.

Compared with alternative template-assisted approaches for HP-MOF synthesis—such as block-copolymer soft templates or sacrificial silica NPs—the DA-mediated method offers several practical advantages58,59. DA is inexpensive, commercially available, and requires no additional template synthesis. Its removal under mild conditions reliably generates accessible mesopores while preserving crystallinity and structural stability (Figure 2A), making this strategy broadly applicable across MOF systems.

Importantly, these defect-engineering conditions remain robust upon scale-up. The synthesis of HP-UiO-66-NH₂ NPs can be readily scaled by proportionally increasing reagent quantities while maintaining identical molar ratios, concentrations, reaction temperature, and reaction time. Owing to furnace size constraints that limit the use of pressure vessels larger than 150 mL (Figure 3A,B), parallel solvothermal reactions offer a practical and reliable scaling strategy. Using two parallel reactions, a 1.5-fold scale-up readily affords gram-scale product with excellent reproducibility (Figure 3C,D), thereby demonstrating the scalability and practicality of this protocol.

Azide functionalization.

Surface modification of HP-UiO-66-NH₂ NPs was achieved through azide installation, enabling subsequent SPAAC chemistry. Prior to Hb loading, surface amine groups were converted to azides using tBuONO and TMSN₃. Successful conversion is confirmed by the appearance of the characteristic asymmetric azide stretching vibration at 2122 cm⁻¹ in FTIR spectra, which is absent in the amine-functionalized precursor (Figure 4A).

Unreacted reagents were removed by simple soaking and washing, preventing pore blockage and preserving mesopore accessibility. N2 adsorption–desorption isotherms and pore size distribution analyses show that HP-UiO-66-N₃ NPs retain their hierarchical porosity, with no significant change in mesopore size following azide functionalization (Figure 4C,D).

Hb loading.

Activated HP-UiO-66-N₃ NPs were incubated with an Hb solution, allowing Hb to diffuse into the mesoporous cavities. Successful loading is evidenced by the characteristic red coloration of the NPs (Figure 5A) and reduced N₂ adsorption capacity (Figure 5B). Complete solvent evacuation prior to incubation is critical for maximizing loading efficiency, as residual solvent within the pore network impedes protein access. In addition to providing sufficient pore volume, favorable electrostatic interactions between negatively charged Hb and the positively charged MOF framework promote protein uptake (Figure 5C).

Through defect engineering, HP-UiO-66-N₃ NPs overcome the size limitations of intrinsic UiO-66 micropores, achieving Hb encapsulation with an EE of ~35% and a LC of ~24 wt%. Although these values are moderate compared with some MOF-based HBOCs—such as one-pot synthesized Hb@ZIF-8 or Hb@ZIF-90 systems, which can achieve EE values exceeding 80–90%—the differences primarily arise from the loading strategy employed60,61. One-pot in situ encapsulation, as used for ZIF-8 and ZIF-90, typically yields higher EE but can interfere with crystal growth and compromise particle stability. By contrast, post-adsorption inherently limits total loading because Hb preferentially occupies surface and near-surface mesopores, restricting access to deeper pore volume, a phenomenon also observed in large-pore MOFs such as PCN-33318,62. Nevertheless, post-adsorption offers distinct advantages by better preserving framework integrity and colloidal stability63. Consistent with this, HP-UiO-66-N₃ NPs retain their morphology and crystallinity after Hb loading, as confirmed by SEM (Figure 5D) and PXRD (Figure 5E). In contrast, high-concentration one-pot encapsulation can increase EE but may disrupt crystal growth and particle stability28. Moreover, the encapsulated Hb remains functional, exhibiting reversible UV–Vis spectral transitions between oxygenated and deoxygenated states (Figure 5F). These results highlight the importance of balancing loading efficiency with structural and functional stability when selecting MOF platforms and loading strategies.

PEGylation and functionality.

Surface PEGylation was employed to enhance biocompatibility and impart stealth properties while preserving Hb functionality35. To avoid chemical perturbation of encapsulated Hb, SPAAC-mediated PEGylation was used, enabling selective conjugation between surface azides and DBCO-functionalized PEG under mild, copper-free conditions48. Unlike Cu(I)-catalyzed azide–alkyne click chemistry, SPAAC eliminates the risk of copper-induced protein deactivation64,65. Simple incubation of Hb@HP-UiO-66-N₃ NPs with aqueous DBCO–PEG at room temperature yielded PEGylated Hb@HP-UiO-66-PEG NPs.

Successful PEGylation was confirmed by multiple techniques. FTIR spectra show the disappearance of the azide vibration and the emergence of PEG-related methylene (−CH2−) and ether bond (−O−) signals at 2883 and 1097 cm⁻¹, respectively (Figure 6A). ¹H NMR spectra exhibit a distinct PEG signal at 3.53 ppm attributable to the oxyethylene units of PEG (Figure 6B). DLS measurements reveal good colloidal dispersion with an increased hydrodynamic diameter of ~140.8 nm (Figure 6C) and only minor changes in ζ-potential (Figure 6D). TEM images further show a uniform PEG coating of ~12 nm thickness surrounding the NPs (Figure 6E), confirming surface-localized modification.

Functional evaluation demonstrates that PEGylation does not compromise oxygen transport. Approximately 64% of encapsulated Hb in Hb@HP-UiO-66-N₃ NPs remains functional, and Hb@HP-UiO-66-PEG NPs retains 61.9 ± 7.2%, with no statistically significant difference (Figure 6F). This outcome is expected, as PEG is intrinsically highly permeable to small gas molecules such as molecular oxygen owing to its hydrated, flexible polymer chains and high free-volume fraction66,67. Moreover, the PEG coating in this system is surface-localized and relatively thin (~12 nm), well below the diffusion length scales that would impose a significant resistance to oxygen transport. Finally, PEGylation occurs exclusively on the external surface of the NPs and does not penetrate or occlude the mesoporous MOF framework, as supported by the preservation of pore accessibility, gas adsorption capacity, and unchanged PXRD patterns following PEGylation (Supplementary Figures 3–5).

ODCs further confirm functional integrity. Compared with free Hb, Hb@HP-UiO-66-PEG NPs exhibit a modest rightward shift, with p50 increasing from 27.4 ± 2.6 to 33.9 ± 2.7 mmHg (Figure 7A,B), indicating slightly reduced oxygen affinity and enhanced release. This behavior is consistent with previous reports on post-loaded Hb@PCN-333(Al) NPs, suggesting a general feature of MOF-confined Hb systems68. The Hill-n (1.8 ± 0.1) remains greater than 1, demonstrating preserved positive cooperativity and intact quaternary structure69,70. These modest changes are attributed to spatial confinement and surface modification effects that slightly perturb the oxygenated-deoxygenated equilibrium without compromising structural integrity, consistent with previous reports for MOF-confined Hb systems71,72.

NPs stability.

Stability studies underscore the protective role of PEGylation. Non-PEGylated NPs rapidly degrade in PBS and DMEM, losing crystallinity within 24 h (Supplementary Figure 2). In contrast, Hb@HP-UiO-66-PEG NPs maintain distinct PXRD reflections for at least 7 days (Figure 8A), stable hydrodynamic size (Figure 8B), and preserved spherical morphology as observed by SEM (Figure 8C). The PEG shell provides a steric barrier against aggregation and environmental stress, significantly enhancing colloidal and structural stability40. As a result, PEGylated Hb@HP-UiO-66 NPs are well suited for biomedical applications that require prolonged stability under physiological conditions, such as oxygen delivery and drug transport.

Troubleshooting.
(1) To remove residual DMF, which is a high-boiling and toxic solvent, from the pores and maximize pore accessibility, the NPs can be soaked and ultrasonically dispersed in ethanol, a low-boiling and less-toxic solvent that facilitates efficient solvent exchange. Subsequent vacuum-drying at 140 °C for 12 h effectively removes ethanol from the pores. Thorough pore evacuation is essential for maximizing Hb loading capacity.

(2) NP aggregation may occur during the Hb loading if the particles are not fully dispersed in the buffer. Short sonication pulses applied immediately before Hb incubation help redisperse the NPs. Brief, low-intensity sonication is sufficient to break up loosely associated aggregates without compromising the integrity of the framework, ensuring a homogeneous suspension and promoting efficient Hb diffusion into the mesopores.

(3) Prior to the final surface-PEGylation step, loss of crystallinity is often observed when PBS is used for washing, due to phosphate-induced framework erosion. To prevent this degradation, repeated washing with ultrapure water or physiological saline rather than PBS is recommended. This prevents phosphate-mediated attack and preserves the structural integrity of the NPs prior to PEG conjugation.

Limitations.
Although the introduction of mesopores enables Hb loading, a key limitation of the current system is its relatively modest Hb loading capacity. Further improvements may require exploring alternative modulators or employing mixed-metal frameworks to generate higher defect densities and increase mesopore volume.

Importance and potential applications of the method.

This work presents a novel synergistic inside-out engineering strategy for MOF NPs, offering a practical approach to simultaneously overcome intrinsic pore-size constraints and limited physiological stability. These advances provide valuable insights into the rational design of MOF-based nanocarriers and highlight their potential for efficient delivery of biomacromolecules. Beyond Hb, the integration of hierarchical porosity with modular PEGylated surface engineering provides a versatile platform for accommodating diverse therapeutic proteins (e.g., insulin) and enzymes (e.g., catalase or superoxide dismutase) while preserving their structural integrity and bioactivity in complex physiological environments. Such structural programmability further opens the door to multifunctional systems that integrate oxygen transport with complementary therapeutic modalities, enabling synergistic regulation of hypoxic or diseased microenvironments.

ACKNOWLEDGMENTS:

We gratefully acknowledge support from the European Research Council under the European Union’s Horizon 2020 Research and Innovation Program (Grant No. 101002060).

DISCLOSURES:

Authors declare no competing financial interest.

REFERENCES:

1.	Sen Gupta, A., Doctor, A. Oxygen Carriers. Damage Control Resuscitation. Springer International Publishing. Cham, Switzerland (2020).

2.	Sen Gupta, A. Bio‐inspired nanomedicine strategies for artificial blood components. WIREs Nanomedicine and Nanobiotechnology. 9 (6), e1464 (2017).

3.	Ding, L. et al. Engineered Red Blood Cell Biomimetic Nanovesicle with Oxygen Self-Supply for Near-Infrared-II Fluorescence-Guided Synergetic Chemo-Photodynamic Therapy against Hypoxic Tumors. ACS Applied Materials & Interfaces. 13 (44), 52435–52449 (2021).

4.	Sang, W. et al. Oxygen‐Enriched Metal‐Phenolic X‐Ray Nanoprocessor for Cancer Radio‐Radiodynamic Therapy in Combination with Checkpoint Blockade Immunotherapy. Advanced Science. 8 (4), 2003338 (2021).

5.	Kim, H. et al. Upconverting nanoparticle-containing erythrocyte-sized hemoglobin microgels that generate heat, oxygen and reactive oxygen species for suppressing hypoxic tumors. Bioactive Materials. 22, 112–126 (2023).

6.	Wang, Y. et al. Highly Penetrable and On-Demand Oxygen Release with Tumor Activity Composite Nanosystem for Photothermal/Photodynamic Synergetic Therapy. ACS Nano. 14 (12), 17046–17062 (2020).

7.	Liu, L. et al. Hemoglobin based oxygen carrier and its application in biomedicine. Coordination Chemistry Reviews. 532, 216508 (2025).

8.	Mohanto, N., Mondal, H., Park, Y.-J., Jee, J.-P. Therapeutic delivery of oxygen using artificial oxygen carriers demonstrates the possibility of treating a wide range of diseases. Journal of Nanobiotechnology. 23 (1), 25 (2025).

9.	Cao, M. et al. Hemoglobin-Based Oxygen Carriers: Potential Applications in Solid Organ Preservation. Frontiers in Pharmacology. 12, 760215 (2021).

10.	Jansman, M.M.T., Hosta-Rigau, L. Recent and prominent examples of nano- and microarchitectures as hemoglobin-based oxygen carriers. Advances in Colloid and Interface Science. 260, 65–84 (2018).

11.	Coll-Satue, C., Bishnoi, S., Chen, J., Hosta-Rigau, L. Stepping stones to the future of haemoglobin-based blood products: clinical, preclinical and innovative examples. Biomaterials Science. 9 (4), 1135–1152 (2021).

12.	Taguchi, K., Maruyama, T., Otagiri, M. Pharmacokinetic properties of hemoglobin vesicles as a substitute for red blood cells. Drug Metabolism Reviews. 43 (3), 362–373 (2011).

13.	Rameez, S., Alosta, H., Palmer, A.F. Biocompatible and Biodegradable Polymersome Encapsulated Hemoglobin: A Potential Oxygen Carrier. Bioconjugate Chemistry. 19 (5), 1025–1032 (2008).

14.	Pacheco, M.O. et al. Silk Fibroin Particles as Carriers in the Development of Hemoglobin‐Based Oxygen Carriers. Advanced NanoBiomed Research. 3 (9), 2300019 (2023).

15.	Taguchi, Y., Cu, Y., Fei, J., Dai, L., Li, J., Yang Construction and Evaluation of Hemoglobin-BasedCapsules as Blood Substitutes. Advanced Functional Materials. 22, 1446–1453 (2012).

16.	Coll-Satue, C., Jansman, M.M.T., Thulstrup, P.W., Hosta-Rigau, L. Optimization of Hemoglobin Encapsulation within PLGA Nanoparticles and Their Investigation as Potential Oxygen Carriers. Pharmaceutics. 13 (11), 1958 (2021).

17.	Jansman, M.M.T. et al. Hemoglobin-based oxygen carriers camouflaged with membranes extracted from red blood cells: Optimization and assessment of functionality. Biomaterials Advances. 134, 112691 (2022).

18.	Liu, X., Jansman, M.M.T., Hosta-Rigau, L. Haemoglobin-loaded metal organic framework-based nanoparticles camouflaged with a red blood cell membrane as potential oxygen delivery systems. Biomaterials Science. 8 (21), 5859–5873 (2020).

19.	Grzegorzewski, W. et al. Hemoglobin-Based Oxygen Carriers: Selected Advances and Challenges in the Design of Safe Oxygen Therapeutics (A Focused Review). International Journal of Molecular Sciences. 26 (19), 9775 (2025).

20.	Zhu, K. et al. Nanomaterial-related hemoglobin-based oxygen carriers, with emphasis on liposome and nano-capsules, for biomedical applications: current status and future perspectives. Journal of Nanobiotechnology. 22 (1), 336 (2024).

21.	Lawson, H.D., Walton, S.P., Chan, C. Metal–Organic Frameworks for Drug Delivery: A Design Perspective. ACS Applied Materials & Interfaces. 13 (6), 7004–7020 (2021).

22.	Lian, X., Chen, Y.-P., Liu, T.-F., Zhou, H.-C. Coupling two enzymes into a tandem nanoreactor utilizing a hierarchically structured MOF. Chemical Science. 7 (12), 6969–6973 (2016).

23.	Jin, W. et al. Surface engineering of metal-organic framework nanoparticles-based miRNA carrier: Boosting RNA stability, intracellular delivery and synergistic therapy. Journal of Colloid and Interface Science. 677, 429–440 (2025).

24.	Wang, Q. et al. Hemoglobin coated oxygen storage metal–organic framework as a promising artificial oxygen carrier. Journal of Materials Chemistry B. 9 (19), 4002–4005 (2021).

25.	Jin, W. et al. Small RNA stabilization via non-covalent binding with a metalloporphyrin nanocage to accomplish synergistic gene and photodynamic therapy. Cell Reports Physical Science. 3 (12), 101187 (2022).

26.	Douka, D. et al. Hemoglobin-loaded ZIF-8 nanoparticles functionalized with human serum albumin as stealth, stable, and biocompatible oxygen carriers. Nanoscale Advances. 7 (24), 8058–8073 (2025).

27.	Liang, K. et al. Biomimetic mineralization of metal-organic frameworks as protective coatings for biomacromolecules. Nature Communications. 6 (1), 7240 (2015).

28.	Douka, D., Jin, W., Cantallops‐Iglesias, C., Dieste‐Izquierdo, A., Thulstrup, P.W., Hosta‐Rigau, L. Optimization and Investigation of Hemoglobin‐Loaded ZIF‐90 Metal–Organic Framework Nanoparticles as Artificial Oxygen Carriers. Particle & Particle Systems Characterization. 42 (3), 2400189 (2025).

29.	Noori, R., Maarof, N.N.N., Alshatteri, A.H. High-efficiency encapsulation and pH-triggered release of docetaxel from folic acid-functionalized ZIF-90 nanocarriers. RSC Advances. 15 (53), 45347–45358 (2025).

30.	Poryvaev, A.S. et al. Guest Leakage from ZIF-8 Particles under Drug Delivery Conditions: Quantitative Characterization and Guest-Induced Framework Stabilization. The Journal of Physical Chemistry C. 125 (28), 15606–15613 (2021).

31.	Valenzano, L. et al. Disclosing the Complex Structure of UiO-66 Metal Organic Framework: A Synergic Combination of Experiment and Theory. Chemistry of Materials. 23 (7), 1700–1718 (2011).

32.	Jarai, B.M., Stillman, Z., Attia, L., Decker, G.E., Bloch, E.D., Fromen, C.A. Evaluating UiO-66 Metal–Organic Framework Nanoparticles as Acid-Sensitive Carriers for Pulmonary Drug Delivery Applications. ACS Applied Materials & Interfaces. 12 (35), 38989–39004 (2020).

33.	Hou, T., Huang, Y., Wang, X., Hu, X., Guan, P. Preparation of lysozyme-imprinted mesoporous Zr-based metal-organic frameworks with remarkable specific recognition. Talanta. 265, 124896 (2023).

34.	Parvizifard, G., Alipour, E., Soltani, S., Hosseini, M.G. Development of a novel voltammetric aptasensor for highly sensitive detection of cardiac troponin I, utilizing ciprofloxacin-loaded UIO-66 metal-organic frameworks. Talanta. 296, 128471 (2026).

35.	Jin, W. et al. Biorthogonal PEGylation of Hierarchical Porous Metal–Organic Frameworks as Robust, Functional Nanocarriers for Hemoglobin-Based Oxygen Delivery. Journal of the American Chemical Society. 147 (47), 43400–43414 (2025).

36.	Cai, G., Jiang, H.-L. A Modulator-Induced Defect-Formation Strategy to Hierarchically Porous Metal–Organic Frameworks with High Stability. Angewandte Chemie International Edition. 56 (2), 563–567 (2017).

37.	Wen, S., Fu, Q., Yan, L., Zhao, X. Hierarchically porous three-dimensional-ordered macro-microporous metal-organic frameworks: Design, precise synthesis, and applications. Coordination Chemistry Reviews. 517, 215996 (2024).

38.	Bunzen, H. Chemical Stability of Metal-Organic Frameworks for Applications in Drug Delivery. ChemNanoMat. 7 (9), 998–1007 (2021).

39.	Bůžek, D., Adamec, S., Lang, K., Demel, J. Metal–organic frameworks vs. buffers: case study of UiO-66 stability. Inorganic Chemistry Frontiers. 8 (3), 720–734 (2021).

40.	Chen, X., Argandona, S.M., Melle, F., Rampal, N., Fairen-Jimenez, D. Advances in surface functionalization of next-generation metal-organic frameworks for biomedical applications: Design, strategies, and prospects. Chem. 10 (2), 504–543 (2024).

41.	Sanità, G., Carrese, B., Lamberti, A. Nanoparticle Surface Functionalization: How to Improve Biocompatibility and Cellular Internalization. Frontiers in Molecular Biosciences. 7, 587012 (2020).

42.	Chen, X. et al. Formulation of Metal–Organic Framework-Based Drug Carriers by Controlled Coordination of Methoxy PEG Phosphate: Boosting Colloidal Stability and Redispersibility. Journal of the American Chemical Society. 143 (34), 13557–13572 (2021).

43.	Wang, S., McGuirk, C.M., d’Aquino, A., Mason, J.A., Mirkin, C.A. Metal–Organic Framework Nanoparticles. Advanced Materials. 30 (37), 1800202 (2018).

44.	Abánades Lázaro, I., Haddad, S., Sacca, S., Orellana-Tavra, C., Fairen-Jimenez, D., Forgan, R.S. Selective Surface PEGylation of UiO-66 Nanoparticles for Enhanced Stability, Cell Uptake, and pH-Responsive Drug Delivery. Chem. 2 (4), 561–578 (2017).

45.	Verhoef, J.J.F., Anchordoquy, T.J. Questioning the use of PEGylation for drug delivery. Drug Delivery and Translational Research. 3 (6), 499–503 (2013).

46.	Sheffey, V.V., Siew, E.B., Tanner, E.E.L., Eniola‐Adefeso, O. PLGA’s Plight and the Role of Stealth Surface Modification Strategies in Its Use for Intravenous Particulate Drug Delivery. Advanced Healthcare Materials. 11 (8), 2101536 (2022).

47.	Suk, J.S., Xu, Q., Kim, N., Hanes, J., Ensign, L.M. PEGylation as a strategy for improving nanoparticle-based drug and gene delivery. Advanced Drug Delivery Reviews. 99, 28–51 (2016).

48.	Siegel, N. et al. Universal Click-Chemistry Approach for the DNA Functionalization of Nanoparticles. Journal of the American Chemical Society. 146 (25), 17250–17260 (2024).

49.	Chen, J., Liu, X., Jansman, M.M.T., Thulstrup, P.W., Hosta-Rigau, L. Metal-Phenolic Networks as Broad-Spectrum Antioxidant Coatings for Hemoglobin Nanoparticles Working as Oxygen Carriers. Chemistry of Materials. 34 (20), 9200–9211 (2022).

50.	Evans, T.G., Salinger, J.L., Bingel, L.W., Walton, K.S. Determining Surface Areas and Pore Volumes of Metal-Organic Frameworks. Journal of Visualized Experiments. (205), 65716 (2024).

51.	Coll-Satue, C., Jansman, M.M.T., Hosta-Rigau, L. Comparative Evaluation of UV-Vis Spectroscopy-Based Approaches for Hemoglobin Quantification: Method Selection and Practical Insights. Biomolecules. 14 (9), 1046 (2024).

52.	Osterrieth, J.W.M. et al. How Reproducible are Surface Areas Calculated from the BET Equation? Advanced Materials. 34 (27), 2201502 (2022).

53.	Drenchev, N.L. et al. In situ FTIR Spectroscopy as a Tool for Investigation of Gas/Solid Interaction: Water-Enhanced CO2 Adsorption in UiO-66 Metal-Organic Framework. Journal of Visualized Experiments. (156), 60285 (2020).

54.	Li, F., Zheng, K., Zhang, H., Duan, C., Xi, H. Nanoscale Hierarchically Porous Metal–Organic Frameworks: Facile Synthesis, Mechanism Research, and Application. ACS Sustainable Chemistry & Engineering. 7 (13), 11080–11087 (2019).

55.	Liu, G., Luo, Q., Wang, H., Zhuang, W., Wang, Y. In situ synthesis of multidentate PEGylated chitosan modified gold nanoparticles with good stability and biocompatibility. RSC Advances. 5 (86), 70109–70116 (2015).

56.	Ben‐Haim, A.E. et al. DNA Delivery to Intact Plant Cells by Casein Nanoparticles with Confirmed Gene Expression. Advanced Functional Materials. 34 (16), 2314756 (2024).

57.	Gourianov, N., Kluger, R. Conjoined Hemoglobins. Loss of Cooperativity and Protein−Protein Interactions. Biochemistry. 44 (45), 14989–14999 (2005).

58.	Doustkhah, E., Hassandoost, R., Khataee, A., Luque, R., Assadi, M.H.N. Hard-templated metal–organic frameworks for advanced applications. Chemical Society Reviews. 50 (5), 2927–2953 (2021).

59.	Li, C. et al. Metal Organic Framework Cubosomes. Angewandte Chemie International Edition. 62 (11), e202215985 (2023).

60.	Peng, S. et al. Metal–Organic Framework Encapsulating Hemoglobin as a High-Stable and Long-Circulating Oxygen Carriers to Treat Hemorrhagic Shock. ACS Applied Materials & Interfaces. 11 (39), 35604–35612 (2019).

61.	Gu, X., Palmer, A.F. ZIF-8 Metal–Organic Framework Nanoparticles Loaded with Hemoglobin as a Potential Red Blood Cell Substitute. ACS Applied Nano Materials. 5 (4), 5670–5679 (2022).

62.	Pullen, S., Clever, G.H. Mixed-Ligand Metal–Organic Frameworks and Heteroleptic Coordination Cages as Multifunctional Scaffolds—A Comparison. Accounts of Chemical Research. 51 (12), 3052–3064 (2018).

63.	Pastore, V.J., Cook, T.R., Rzayev, J. Polymer–MOF Hybrid Composites with High Porosity and Stability through Surface-Selective Ligand Exchange. Chemistry of Materials. 30 (23), 8639–8649 (2018).

64.	Obeso, J.L., Huxley, M.T., De Los Reyes, J.A., Humphrey, S.M., Ibarra, I.A., Peralta, R.A. Low‐Valent Metals in Metal‐Organic Frameworks Via Post‐Synthetic Modification. Angewandte Chemie International Edition. 62 (49), e202309025 (2023).

65.	Zhang, X., Malhotra, S., Molina, M., Haag, R. Micro- and nanogels with labile crosslinks – from synthesis to biomedical applications. Chemical Society Reviews. 44 (7), 1948–1973 (2015).

66.	Biscarat, J., Charmette, C., Masquelez, N., Sanchez, J., Pochat‐Bohatier, C. Crosslinking of gelatin membranes with ferulic acid or glutaraldehyde: Relationship between gas permeability and renaturation level of gelatin triple helices. Journal of Polymer Science Part B: Polymer Physics. 53 (4), 280–287 (2015).

67.	Kurian, P. et al. Synthesis, permeability and biocompatibility of tricomponent membranes containing polyethylene glycol, polydimethylsiloxane and polypentamethylcyclopentasiloxane domains. Biomaterials. 24 (20), 3493–3503 (2003).

68.	Liu, X., Jansman, M.M.T., Li, W., Kempen, P., Thulstrup, P.W., Hosta-Rigau, L. Metal–organic framework-based oxygen carriers with antioxidant protection as a result of a polydopamine coating. Biomaterials Science. 9 (21), 7257–7274 (2021).

69.	Srinivasu, B.Y., Bose, B., Mitra, G., Kurpad, A.V., Mandal, A.K. Adsorption Induced Changes of Human Hemoglobin on Ferric Pyrophosphate Nanoparticle Surface Probed by Isotope Exchange Mass Spectrometry: An Implication on Structure–Function Correlation. Langmuir. 33 (32), 8032–8042 (2017).

70.	Chen, K. et al. Low Modulus Biomimetic Microgel Particles with High Loading of Hemoglobin. Biomacromolecules. 13 (9), 2748–2759 (2012).

71.	Dyachenko, A., Gruber, R., Shimon, L., Horovitz, A., Sharon, M. Allosteric mechanisms can be distinguished using structural mass spectrometry. Proceedings of the National Academy of Sciences. 110 (18), 7235–7239 (2013).

72.	Coll-Satue, C. et al. A novel PEG-mediated approach to entrap hemoglobin (Hb) within ZIF-8 nanoparticles: Balancing crystalline structure, Hb content and functionality. Biomaterials Advances. 163, 213953 (2024).