# Submission ID #: 70553

# Scriptwriter Name: Sri Lakshmi Priya

# Project Page Link: https://review.jove.com/account/file-uploader?src=21518713

# Title: Medium-throughput Column DNA Extraction Using 24-well Racks

# Authors and Affiliations:

Denis Naberezhnov¹,², Olesya Lygun³, Tatyana Kalinina³, Ilya Nikiteev¹, Julia Kuzmina¹, Maria Drozhzhina¹, Grigory Kozhemyakin¹, Nikita Baraboshkin¹, Angelina Lebedeva¹, Galina Viryasova⁴, Oleg Fedorov¹

¹Research Institute of Systems Biology and Medicine of Rospotrebnadzor

²Engelhardt Institute of Molecular Biology, Russian Academy of Sciences

³Moscow Institute of Physics and Technology

⁴Belozersky Institute of Physico-Chemical Biology, Lomonosov Moscow State University

# Corresponding Authors:

Denis Naberezhnov		denis@naberezhnov.com

# Email Addresses for All Authors:

Olesya Lygun			lygun.oa@phystech.edu

Tatyana Kalinina		kalinina.tt@phystech.edu

Ilya Nikiteev			nikiteev_ia@sysbiomed.ru

Julia Kuzmina			kuzmina_je@sysbiomed.ru

Maria Drozhzhina		drozhzhina_ma@sysbiomed.ru

Grigory Kozhemyakin		kozhemyakin_gl@sysbiomed.ru

Nikita Baraboshkin		baraboshkin_nm@sysbiomed.ru

Angelina Lebedeva		lebedeva_aa@sysbiomed.ru

Galina Viryasova		galina.viryasova@yandex.ru

Oleg Fedorov			o.fedorov@sysbiomed.ru.

Denis Naberezhnov		denis@naberezhnov.com

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

☐ Incorrect

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 06/22/2026

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  15

Number of Shots:  44

# Interviews

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Tatyana Kalinina: Our research focuses on developing 3D-printed racks for parallel spin-column DNA and RNA extraction, bridging manual and automated methods.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Tatyana Kalinina: This protocol addresses limitations of labor-intensive manual spin-column methods by enabling simultaneous processing of up to 116 DNA or RNA samples.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Tatyana Kalinina: This protocol allows researchers to obtain high-quality plasmid DNA for downstream applications, including cloning, sequencing, and PCR analysis.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Tatyana Kalinina: The most important consideration when performing this protocol is preventing cross-contamination by maintaining watertight, non-overfilled 3D-printed reservoirs.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Are there any additional methods or analyses that can be performed following this procedure?

  - Tatyana Kalinina: Additionally, following this procedure, these racks can be used for any column-based nucleic acid extraction analysis.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

- Preparation of 3D-printed Equipment

Demonstrator: Olesya Lygun

NOTE: SCREEN timestamps for protocol were added at the postshoot stage. Please contact the postshoot note integrator (Sri Lakshmi Priya) for queries regarding lab media.

  - To begin, use a fused-filament 3D printer with a 0.4-millimeter nozzle to print the rack [1]. Use generic polylactide or PLA (P-L-A), polyethylene terephthalate glycol or PETG (P-E-T-G), or acrylonitrile butadiene styrene or ABS (A-B-S) filament with 100 percent infill [2]. Set the layer height to 0.1 millimeter and enable surface smoothing [3]. Adjust the print settings to ensure watertightness and mechanical stability [4].

    - WIDE: Talent standing near the fused filament 3D printer and preparing it with a 0.4 mm nozzle.

    - Close shot of the filament spool as the talent loads generic PLA, PETG, or ABS filament for printing.

    - SCREEN: 2.1.3 (ironing).png

    - SCREEN: 2.1.4.png

  - Test the racks for waterproofness before using them for the first time [1]. Fill the printed reservoir with 70 percent ethanol to test it before use [2]. Centrifuge for progressively increasing durations to confirm the absence of leakage or structural damage [3-TXT].

    - Talent adding water into racks and checking it.

    - Talent filling the printed reservoir with 70 percent ethanol.

    - Talent placing the reservoir into the centrifuge. TXT: 1 min → 2 min → 5 min → 10 min → 30 min

  - After successful testing using glue, join the Base Reservoir components together to form a sealed unit [1].

    - Talent joining the Base Reservoir components to form a sealed unit.

- Preparation of Bacterial Culture and Cell Lysate

Demonstrator: Maria Drozhzhina

  - Inoculate Escherichia coli or E. coli (E-Coli) transformants into 3 to 5 milliliters of lysogeny broth or LB (L-B) medium containing appropriate antibiotics [1]. Incubate the culture for 12 to 16 hours at 37 degrees Celsius with shaking [2].

    - Talent inoculating E. coli transformants into a tube containing lysogeny broth medium.

    - Talent placing the culture tubes in an incubator shaker set to 37 degrees Celsius.

  - Centrifuge at 2000 g for 10 minutes to harvest the cells [1]. Discard the supernatant [2].

    - Talent placing the culture tubes in the centrifuge.

    - Talent discarding the supernatant.

  - Add 250 microliters of resuspension buffer to the pellet [1] and resuspend the pellet completely by vortexing or pipetting [2].

    - Talent adding resuspension buffer to the cell pellet.

    - Talent resuspending the pellet by vortexing or pipetting until the suspension is uniform.

  - Add 250 microliters of lysis buffer [1] and mix gently by inverting the tube 4 to 6 times or by rocking the 24-well plate [2].

    - Talent adding lysis buffer to the resuspended pellet.

    - Talent gently inverting the tube 4 to 6 times to mix.

  - Add 350 microliters of neutralization buffer [1] and mix it [2]. Observe the formation of a white precipitate to confirm successful neutralization [3].

    - Talent adding neutralization buffer to the lysate.

    - Talent inverting the tube to mix.

    - Close shot of the tube showing the white precipitate after neutralization.

- Preparation of Lysate Using Titanium Dioxide Co-precipitation

Demonstrator: Maria Drozhzhina

  - Add lysis buffer into a tube containing titanium dioxide powder at a 2 to 1 volume-to-mass ratio [1]. Add 250 microliters of the titanium dioxide suspension to the lysate [2] and mix it by inverting the tube [3]. Add 350 microliters of neutralization buffer [4] and mix thoroughly [5].

    - Talent adding lysis buffer into a tube containing titanium dioxide powder.

    - Talent adding the titanium dioxide suspension to the lysate.

    - Talent inverting the tube to mix the suspension with the lysate.

    - Talent adding neutralization buffer to the mixture.

    - Talent inverting the tube to mix the contents.

  - Centrifuge at 2000 g for 10 minutes [1]. Observe the cell pellet completely precipitated in the presence of titanium dioxide and the supernatant appearing fully transparent [2]. In the absence of titanium dioxide, note the presence of residual sediment in the supernatant [3]. Collect the supernatant without disturbing the precipitate [4] and transfer it to the binding column [5].

    - Talent placing the tubes into the centrifuge.

    - Close-up of precipitated cell pellet and transparent supernatant.

    - Close-up of residual sediment in the supernatant.

    - Talent collecting the supernatant while leaving the precipitate undisturbed.

    - Talent transferring the supernatant to the binding column.

- Assembly of the Filtration Unit for Lysate Clearance and DNA Binding

Demonstrator: Tatyana Kalinina

  - Place the deoxyribonucleic acid or DNA (D-N-A) binding columns into the column holder layer of the rack [1]. Position the column lids into the designated slots to secure them during centrifugation [2]. Place the clarification or filter columns into the filter column layer directly above the binding columns [3].

    - Talent placing the DNA-binding columns into the column holder layer of the rack.

    - Talent positioning the column lids into the designated slots on the rack.

    - Talent placing the filter columns into the filter column layer above the binding columns.

  - Transfer the prepared neutralized lysate into the filtration columns [1]. Place the assembled rack into a swinging-bucket plate rotor [2]. Centrifuge at 2000 g for 1 minute at room temperature [3]. After removing the rack from the centrifuge, remove and discard the filter column layer [4].

    - Talent transferring the neutralized lysate into the filtration columns.

    - Talent placing the assembled rack into a swinging-bucket plate rotor.

    - Talent placing the rotor into the centrifuge.

    - Talent removing the filter column layer from the rack and discarding it.

- Washing and Elution of DNA

Demonstrator: Tatyana Kalinina

  - Discard the flow-through from the reservoir into a waste container [1]. Reassemble the rack with the binding columns [2]. Add 500 to 700 microliters of wash buffer to each binding column [3]. Centrifuge at 2000 g for 1 minute [4]. Discard the flow-through from the reservoir and repeat the washing step as previously demonstrated [5].

    - Talent pouring the flow through from the reservoir into a waste container.

    - Talent reassembling the rack with the binding columns.

    - Talent adding wash buffer to each binding column.

    - Talent placing the rack in the centrifuge.

    - Talent discarding the flow through from the reservoir.

  - Remove the binding columns from the rack [1] and transfer them to 2-milliliter collection tubes [2]. Centrifuge at 10,000 g or higher for 1 minute to dry the membrane [3]. Transfer the columns to clean 1.5-milliliter tubes [4].

    - Talent removing the binding columns from the rack

    - Talent placing the binding columns into 2-milliliter collection tubes.

    - Talent placing the collection tubes in the centrifuge.

    - Talent transferring the dried columns into clean 1.5 milliliter tubes.

  - Add 50 microliters of elution buffer or water directly to the membrane and incubate the columns for 1 minute at room temperature [1]. Centrifuge at 10,000 g or higher for 1 minute to elute the DNA [2].

    - Talent adding elution buffer or water directly onto the membrane of each column and keeping it aside for incubation.

    - Talent placing the columns into the centrifuge.

# Results

- Results

  - Total plasmid yield showed no significant difference between the conventional method and extraction using the rack configurations [1].

    - LAB MEDIA: Figure 5. Video editor: Emphasize the three bars labeled “Manual Extraction,” “Rack extraction,” and “Titanium dioxide”.

  - Plasmid conformation was preserved, with similar distributions of relaxed, linearized, and supercoiled forms [1].

    - LAB MEDIA: Figure 6. Video editor: Highlight the band patterns under Standard Manual Extraction (column 2,3,4) and Rack Extraction (column 6,7,8).

  - Dye distribution was confined to the waste area directly beneath the loaded columns [1], with no visible signal detected in adjacent wells or on neighboring column tips, indicating that the rack design reduces the likelihood of splashing and cross-column transfer [2].

    - LAB MEDIA: Figure 7. Video editor: Circle the wells with strong red dye.

    - LAB MEDIA: Figure 7. Video editor: Circle the wells with a small center tip and have no red staining.

  - Amplification was detected only in eluates from columns to which the amplicon had been added [1].

    - LAB MEDIA: Figure 8. Video editor: Highlight all the red curves that rise upward from cycle 20 and reach approximately 600 on the Y-axis by cycle 50

Pronunciation guides:

- Polylactide
Pronunciation link: https://www.merriam-webster.com/dictionary/polylactide
IPA: /ˌpɑː.liˈlæk.taɪd/
Phonetic Spelling: pah·lee·lak·tyde

- Polyethylene
Pronunciation link: https://www.merriam-webster.com/dictionary/polyethylene
IPA: /ˌpɑː.liˈeθ.əˌliːn/
Phonetic Spelling: pah·lee·eth·uh·leen

- Terephthalate
Pronunciation link: https://www.merriam-webster.com/dictionary/terephthalate
IPA: /təˈref.θəˌleɪt/
Phonetic Spelling: tuh·ref·thuh·layt

- Glycol
Pronunciation link: https://www.merriam-webster.com/dictionary/glycol
IPA: /ˈɡlaɪˌkɔːl/
Phonetic Spelling: gly·kawl

- Acrylonitrile
Pronunciation link: https://www.merriam-webster.com/dictionary/acrylonitrile
IPA: /əˌkrɪl.oʊˈnaɪ.trəl/
Phonetic Spelling: uh·kril·oh·nye·truhl

- Butadiene
Pronunciation link: https://www.merriam-webster.com/dictionary/butadiene
IPA: /ˌbjuː.təˈdaɪ.iːn/
Phonetic Spelling: byoo·tuh·dye·een

- Styrene
Pronunciation link: https://www.merriam-webster.com/dictionary/styrene
IPA: /ˈstaɪˌriːn/
Phonetic Spelling: sty·reen

- Filament
Pronunciation link: https://www.merriam-webster.com/dictionary/filament
IPA: /ˈfɪl.ə.mənt/
Phonetic Spelling: fil·uh·muhnt

- Watertightness
Pronunciation link: https://www.merriam-webster.com/dictionary/watertightness
IPA: /ˈwɔː.tɚˌtaɪt.nəs/
Phonetic Spelling: waw·ter·tyte·nuhs

- Ethanol
Pronunciation link: https://www.merriam-webster.com/dictionary/ethanol
IPA: /ˈeθ.əˌnɔːl/
Phonetic Spelling: eth·uh·nawl

- Centrifuge
Pronunciation link: https://www.merriam-webster.com/dictionary/centrifuge
IPA: /ˈsen.trəˌfjuːdʒ/
Phonetic Spelling: sen·truh·fyooj

- Reservoir
Pronunciation link: https://www.merriam-webster.com/dictionary/reservoir
IPA: /ˈrez.ɚˌvwɑːr/
Phonetic Spelling: rez·er·vwar

- Escherichia
Pronunciation link: https://www.merriam-webster.com/dictionary/Escherichia
IPA: /ˌeʃ.əˈrɪk.i.ə/
Phonetic Spelling: esh·uh·rik·ee·uh

- coli
Pronunciation link: https://www.merriam-webster.com/dictionary/coli
IPA: /ˈkoʊ.laɪ/
Phonetic Spelling: koh·lye

- Transformants
Pronunciation link: https://www.merriam-webster.com/dictionary/transformant
IPA: /trænsˈfɔːr.mənts/
Phonetic Spelling: trans·for·muhnts

- Lysogeny
Pronunciation link: https://www.merriam-webster.com/dictionary/lysogeny
IPA: /laɪˈsɑː.dʒə.ni/
Phonetic Spelling: lye·sah·juh·nee

- Antibiotics
Pronunciation link: https://www.merriam-webster.com/dictionary/antibiotic
IPA: /ˌæn.taɪ.baɪˈɑː.tɪk/
Phonetic Spelling: an·tye·bye·ah·tik

- Supernatant
Pronunciation link: https://www.merriam-webster.com/dictionary/supernatant
IPA: /ˌsuː.pɚˈneɪ.tənt/
Phonetic Spelling: soo·per·nay·tuhnt

- Resuspension
Pronunciation link: https://www.merriam-webster.com/dictionary/resuspension
IPA: /ˌriː.səˈspen.ʃən/
Phonetic Spelling: ree·suh·spen·shuhn

- Vortexing
Pronunciation link: https://www.merriam-webster.com/dictionary/vortex
IPA: /ˈvɔːr.teks/
Phonetic Spelling: vor·teks·ing

- Lysis
Pronunciation link: https://www.merriam-webster.com/dictionary/lysis
IPA: /ˈlaɪ.sɪs/
Phonetic Spelling: lye·suhs

- Neutralization
Pronunciation link: https://www.merriam-webster.com/dictionary/neutralization
IPA: /ˌnuː.trə.ləˈzeɪ.ʃən/
Phonetic Spelling: noo·truh·luh·zay·shuhn

- Lysate
Pronunciation link: https://www.merriam-webster.com/dictionary/lysate
IPA: /ˈlaɪ.seɪt/
Phonetic Spelling: lye·sayt

- Precipitate
Pronunciation link: https://www.merriam-webster.com/dictionary/precipitate
IPA: /prɪˈsɪp.ə.tət/ (noun), /prɪˈsɪp.əˌteɪt/ (verb)
Phonetic Spelling: prih·sip·uh·tuht; prih·sip·uh·tayt

- Titanium
Pronunciation link: https://www.merriam-webster.com/dictionary/titanium
IPA: /taɪˈteɪ.ni.əm/
Phonetic Spelling: tye·tay·nee·uhm

- Dioxide
Pronunciation link: https://www.merriam-webster.com/dictionary/dioxide
IPA: /daɪˈɑːk.saɪd/
Phonetic Spelling: dye·ok·side

- Co-precipitation
Pronunciation link: https://www.merriam-webster.com/dictionary/coprecipitation
IPA: /ˌkoʊ.prɪˌsɪp.əˈteɪ.ʃən/
Phonetic Spelling: koh·prih·sip·uh·tay·shuhn

- Clarification
Pronunciation link: https://www.merriam-webster.com/dictionary/clarification
IPA: /ˌkler.ə.fəˈkeɪ.ʃən/
Phonetic Spelling: clair·uh·fuh·kay·shuhn

- Deoxyribonucleic
Pronunciation link: https://www.merriam-webster.com/dictionary/deoxyribonucleic%20acid
IPA: /diˌɑːk.siˌraɪ.boʊ.nuːˈkliː.ɪk/
Phonetic Spelling: dee·ok·see·rye·boh·noo·klee·ik

- Centrifugation
Pronunciation link: https://www.merriam-webster.com/dictionary/centrifugation
IPA: /ˌsen.trə.fjuːˈɡeɪ.ʃən/
Phonetic Spelling: sen·truh·fyoo·gay·shuhn

- Rotor
Pronunciation link: https://www.merriam-webster.com/dictionary/rotor
IPA: /ˈroʊ.t̬ɚ/
Phonetic Spelling: roh·ter

- Elution
Pronunciation link: https://www.merriam-webster.com/dictionary/elution
IPA: /iˈluː.ʃən/
Phonetic Spelling: ee·loo·shuhn

- Membrane
Pronunciation link: https://www.merriam-webster.com/dictionary/membrane
IPA: /ˈmem.breɪn/
Phonetic Spelling: mem·brayn

- Plasmid
Pronunciation link: https://www.merriam-webster.com/dictionary/plasmid
IPA: /ˈplæz.mɪd/
Phonetic Spelling: plaz·mid

- Conformation
Pronunciation link: https://www.merriam-webster.com/dictionary/conformation
IPA: /ˌkɑːn.fɚˈmeɪ.ʃən/
Phonetic Spelling: kon·fer·may·shuhn

- Linearized
Pronunciation link: https://www.merriam-webster.com/dictionary/linearize
IPA: /ˈlɪn.i.əˌraɪzd/
Phonetic Spelling: lin·ee·uh·ryezd

- Supercoiled
Pronunciation link: https://www.merriam-webster.com/dictionary/supercoil
IPA: /ˌsuː.pɚˈkɔɪld/
Phonetic Spelling: soo·per·koyld

- Amplicon
Pronunciation link: https://www.merriam-webster.com/dictionary/amplicon
IPA: /ˈæm.plɪˌkɑːn/
Phonetic Spelling: am·pli·kon

- Eluates
Pronunciation link: https://www.merriam-webster.com/dictionary/eluate
IPA: /ˈel.juː.eɪt/
Phonetic Spelling: el·yoo·ayt