# Submission ID #: 70583

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/files_upload.php?src=21527863

Title: Focus Stacking Protocol for High-resolution Insect Photography

# Authors and Affiliations:

Nathanaël MAURY1, Somchit SUDAVANH1, Nita SOUN2, Reaksa SUN2, Sebastien BOYER2,3

1Toxo Inc.,

2Medical and Veterinary Entomology Unit, Institut Pasteur du Cambodge

3Ecology and emergence of arthropod-borne Pathogens Unit, Department of Global Health, Institut Pasteur

# Corresponding Authors:

Sebastien Boyer        (sboyer@paseur-kh.org)

# Email Addresses for All Authors:

Nathanaël Maury      (nathanael.maury@gmail.com)

Somchit Sudavanh    (somchit.soudavanh@gmail.com)

Nita Soun                    (nita@pasteur-kh.org)

Reaksa Sun                 (sunreaksa06@gmail.com)

Sebastien Boyer        (sboyer@paseur-kh.org)

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Interview statements: Which interview statement filming option is the most appropriate for your group? Please select one.

☒ 	Interviewees self-record interview statements. JoVE can provide support for this option.

3. Proposed interview filming date: Please indicate the proposed date that your group will self-film interviews: MM/DD/YYYY

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  17

Number of Shots:  25

# Interviews

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Sebastien Boyer: Our research investigates how anthropization, deforestation, climate change, and land-use changes affect mosquito vector dynamics, biodiversity, ecosystem health, and pathogen transmission.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Sebastien Boyer: Existing methods struggle with accurate insect identification and analysis of large datasets; This image-stacking protocol generates detailed diagnostic images, improving identification reliability.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera  Suggested B-roll: 4.3.1

CONCLUSION

Describe the most important consideration or challenge when performing this protocol.

  - Sebastien Boyer: The greatest challenge while performing this protocol was eliminating seismic vibrations and airflow to capture extremely high-resolution images of very small insects.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Are there any additional methods or analyses that can be performed following this procedure?

  - Sebastien Boyer: Following this procedure, additional morphometric analyses can identify subtle infection-associated differences, reveal visible pathogen indicators, and improve understanding of vector–pathogen interactions.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Sebastien Boyer: Future studies could combine these images with machine learning to discover new morphological traits and improve species identification.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

# Protocol

- Setting up the Imaging Workstation

Demonstrator: Nita Soun

  - To begin, place a vibration-damping table on a stable surface [1]. Position a camera mounting rail at the center of the table [2] and fix the digital camera body onto the rail using a rigid plate adapter [3].

    - WIDE: Talent placing a vibration-damping table on a stable surface.

    - Talent positioning a camera mounting rail at the center of the table.

    - Talent fixing the digital camera body onto the rail with a rigid plate adapter.

  - Attach the selected microscope objective from 5X (Five-X) to 20X or a macro lens via the appropriate tube adapter [1].

    - Talent attaching the selected microscope objective or macro lens using the appropriate tube adapter.

  - Install two rear and two frontal light sources equipped with diffusers to create uniform illumination [1].

    - Shot of the installed rear and frontal light sources equipped with diffusers.

  - Mount the subject support consisting of an XYZ (X-Y-Z) stage or rotary holder directly beneath the lens axis [1].

    - Talent mounting the subject support directly beneath the lens axis.

  - Ensure that the optical axis is perpendicular to the specimen plane [1].

    - Talent checking and aligning the optical axis perpendicular to the specimen plane.

  - Adjust the horizontal and vertical level using the stage micrometers until reflections are symmetrical [1].

    - Talent adjusting the horizontal and vertical level with the stage micrometers.

  - Confirm that illumination is homogeneous across the field [1]. Realign the diffusers as needed, followed by color calibration [2].

    - Talent checking the illumination across the field.

    - Talent realigning the diffusers to achieve homogeneous illumination.

- Specimen Mounting and Focusing

  - Fix dried or ethanol-preserved insects on a soft mounting base such as modeling clay [1]. Orient the specimen according to the desired view [2].

    - LAB MEDIA: 8.Mov: 01:16-01:30

    - LAB MEDIA: 11.Mov:  00:05-00:19

  - Position the support centered under the optical axis [1].

    - LAB MEDIA: 12.Mov: 00:00-00:12.

  - Define the beginning and end of the focal range with the nearest and farthest sharp plane [1].

    - LAB MEDIA:24.Mov  01:00-01:21

  - Program incremental movement between 0.2 micrometers and 2 micrometers depending on magnification [1]. Synchronize the capture trigger so that illumination fires simultaneously with each image exposure [2].

    - LAB MEDIA: 24.Mov  00:28-00:51

    - LAB MEDIA: 24. Mov: 00:10-00:26

  - Capture sequences of 50 to 2000 images according to specimen size and depth [1].

    - LAB MEDIA: 25.Mov: 00:00-00:09

- Image Processing

  - Transfer the files to a local computer immediately after acquisition [1]. Group each stack into a unique folder using incremental numbering [2].

    - LAB MEDIA: 26.Mov: 00:10-00:23

    - LAB MEDIA: 27.Mov: 00:00-00:09

  - Apply lens and color correction profiles [1] and export the images as 16-bit TIFF files [2].

    - LAB MEDIA: 27.Mov: 00:30-00:36

    - LAB MEDIA: 29.Mov: 00:11-00:16.

  - Open the TIFF stack in focus stacking software [1].

    - LAB MEDIA: 29.Mov: 0:17-0:20

  - Choose a depth map algorithm for the first composite [1].

    - SCREEN: Show the depth map algorithm selection and choose Method B.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/files_upload.php?src=21527863

  - Open the composite in an image editing program [1]. Remove background artifacts and dust using selection and healing tools [2].

    - LAB MEDIA: 38.Mov: 00:00-00:10

    - LAB MEDIA: 38.Mov: 00:51-01:10

# Results

- Results

  - Calculated pixel scales ranged from 0.76 micrometers at 5X magnification [1] to 0.19 micrometers at 20X magnification in object space, confirming adequate sampling for sub-micron structural details [2].

    - LAB MEDIA: Table 1. Video editor: Highlight the value 0.752 in the row 5× under “Scale per Pixel (Object Plane, micrometer)”.

    - LAB MEDIA: Table 1. Video editor: Highlight the value 0.188 in the row 20× under “Scale per Pixel (Object Plane, micrometer)”.

  - The effective resolving power was approximately 4 micrometers [1], matching the diffraction limit of the employed objectives [2].

    - LAB MEDIA: Table 2. Video editor: Highlight the Airy disk diameter values near 4 micrometers in the “Airy disk diameter (Object plane, micrometer)” column.

    - LAB MEDIA: Table 2. Video editor: Highlight the entries “Diffraction” in the “Limiting factor” column.

  - Expected depth of field values ranged from 5.05 micrometers [1] to approximately 0.1 micrometers [2].

    - LAB MEDIA: Table 2. Video editor: Highlight the value 5.05 in the “Depth of field (micrometer)” column.

    - LAB MEDIA: Table 2. Video editor: Highlight the value approximately 0.1 in the “Depth of field (micrometer)” column.