# Submission ID #: 70620

# Scriptwriter Name: Kathyayani Sridharan

# Project Page Link: https://review.jove.com/account/file-uploader?src=21539148

# Title: Cardiac Catheterization for Assessment of Pulmonary Hypertension in Preterm Lambs

# Authors and Affiliations:

# Yasmine N White*, Matthew Douglass*, Jakob Van Boerum, Andrew Rebentisch, Kurt H Albertine, Mar Janna Dahl

# Division of Neonatology, Department of Pediatrics, University of Utah

* These authors contributed equally

# Corresponding Authors:

# Yasmine N White              		(Yasmine.white@hsc.utah.edu)

# Email Addresses for All Authors:

# Matthew Douglass               		(matt.douglass@hsc.utah.edu)

# Jakob Van Boerum               		(Jakob.vanboerum@hsc.utah.edu)

# Andrew Rebentisch         		(Andrew.rebentisch@hsc.utah.edu)

# Kurt H Albertine               		(kurt.albertine@hsc.utah.edu)

# Mar Janna Dahl                 		(marjanna.dahl@hsc.utah.edu)

# Yasmine N White              		(Yasmine.white@hsc.utah.edu)

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

☐ Incorrect

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 05/12/2026

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps: 16

Number of Shots:  54

# Introduction

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context.
Note: The statements have been edited for length and clarity

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Yasmine White: Our research investigates pathophysiology and novel therapies for bronchopulmonary dysplasia-associated pulmonary hypertension in a neonatal lamb model.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Matthew Douglass: We developed a standardized, repeatable lamb cardiac catheterization protocol to address inconsistent pulmonary vascular resistance assessments in large-animal BPD-PH models.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Mar Janna Dahl: The cardiac catheterization allows researchers to measure cardiac pressures and calculate pulmonary vascular resistance in neonatal large animal models.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Yasmine White: The challenge of cardiac catheterization in neonatal lambs is guiding the catheter along the desired path into the right heart.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Are there any additional methods or analyses that can be performed following this procedure?

  - Matthew Douglass: Repeated cardiac catheterization allows for assessment of hemodynamic changes over time or in response to interventions.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Ethics Title Card

This research has been approved by the Institutional Animal Care and Use Committee (IACUC) at University of Utah

# Protocol

- Position Setup of Preterm Lamb for Right Heart Catheterization

Demonstrator: Dr. Mar Janna Dahl, Dr. Matthew Douglass

  - To begin, align the lamb in a prone position using a sling [1], then verify the functioning of existing umbilical or jugular central venous catheters for initial vascular access [2]. Infuse dexmedetomidine (dex-med-eh-TOH-mi-deen) and adjust the rate to achieve the desired sedation depth, while supplementing additional fast-acting sedatives and analgesics as needed[3-TXT].

    - WIDE: Talent positioning the lamb in prone position using a sling.

    - Talent checking the functioning of previously placed catheters.

    - IV line showing infusion of dexmedetomidine, analgesics and sedatives. TXT: Dex infusion: 0.2-1.0 mcg/kg/h; Supplement (IV bolus): pentobarbital (1 mg/kg/dose); buprenorphine (5 mcg/kg/dose)

  - Shave the right or left neck over the external jugular vein [1] and inject zero-point five milliliters of one percent lidocaine subcutaneously over the site [2]. Sterilize the site with three alternating betadine and alcohol scrubs [3].

    - Talent shaving the neck area of the lamb.

    - Talent injecting local anesthetic subcutaneously.

    - Talent sterilizing the site using alternate betadine and alcohol scrubs.

  - Wear appropriate personal protective equipment and sterile gloves [1]. Dispose of all needles and other sharps immediately after use in an approved sharps container [2].

    - Talent wearing PPE and gloves.

    - Talent disposing of all needles and sharps into designated container.

  - Prime a Swan-Ganz (Swahn GAHNZ) catheter with two-percent heparinized saline [1] and connect the distal port to a pressure transducer for continuous recording in LabChart [2]. Set the pressure transducer to zero [3]. Monitor [4] oxygen saturation, arterial blood pressure, and heart rate throughout the procedure [5].

    - Talent injecting fine continuous stream of saline into the catheter.

    - Talent connecting the distal port of the catheter to a pressure transducer.

    - Talent setting the transducer to zero.

    - SCREEN: Show the SpO2, HR and BP values.

    - TEXT ON A PLAIN BACKGROUND
Halt or abort the procedure if vitals destabilize despite ventilator and oxygen fraction adjustments. Criteria include:  
- SpO2: <90% for 2 min
- Heart rate: <60 beats/minute
- Respiratory rate: >100 breaths/ minute
- Arterial blood pressure decreased >15% from pre-procedure baseline

- Catheter Introduction and Advancement

Demonstrator: Dr. Mar Janna Dahl, Dr. Matthew Douglass

  - Use an eleven blade to make a one-centimeter incision over the external jugular vein to expose the vessel [1]. Dissect the external jugular vein [2] and pass a 3.0 (three-oh) silk suture beneath it to isolate the vein, maintain proximal control of the vessel and prevent bleeding during catheter introduction [3].

    - Talent making an incision over the external jugular vein using an 11 blade to expose the vessel.

    - Talent dissecting the external jugular vein.

    - Talent passing a 3-0 suture beneath the dissected jugular vein.

  - Introduce an 18 G (eighteen gauge), 2.5-inch (two-and-a-half inch) needle into the vessel [1]. Advance a guide wire through the needle using the Seldinger (sel-DING-er) technique [2]. Then, advance a 6 Fr (Six French) introducer filled with heparinized saline into the vessel over the guide wire [3].

    - Talent introducing an 18 G needle into the vessel.

    - Talent advancing a guide wire through the needle.

    - Talent advancing a 6 Fr introducer with saline over the guide wire.

  - Remove the guidewire while maintaining the introducer in place [1]. Use the existing silk tie to suture the introducer and secure it in the vessel at a depth of approximately three to three-point five centimeters [2]. Aspirate and flush the introducer with heparinized saline to confirm blood return [3].

    - Talent carefully removing the guide wire without disturbing the introducer.

    - Talent securing the introducer in the vessel using the existing 3-0 suture.

    - Talent aspirating and flushing saline into the introducer to remove any air and confirm blood return.

  - Re-verify transducer leveling and zeroing before catheter insertion [1-TXT]. Pass and gradually advance the previously prepared Swan-Ganz catheter through the introducer [2] and monitor the transduced pressure waveform [3].

    - Talent checking the pressure transducer levelling and zero calibration. TXT: Even 1–2 cm error significantly skews PA, PCWP, mPAP, and PVR values

    - Talent introducing and gradually advancing the Swan-Ganz catheter.

    - SCREEN: Shows the pressure waveform.

- Measurement of Pressures and Cardiac Output

Demonstrator: Dr. Mar Janna Dahl

  - Advance the Swan-Ganz catheter [1] until the typical venous pressure tracing is replaced by a typical right atrial pressure tracing and record transduced pressures [2]. Observe that the right atrial pressure does not typically reach zero, although this may occur in severe hypovolemia case [3].

    - Talent advancing the Swan-Ganz catheter further. Videographer: Obtain multiple reusable takes of 4.1.1 as these will be used later in the protocol

    - SCREEN: Show the monitor recording the venous pressure tracing changing to right atrial pressure tracing.

    - SCREEN: Display that the tracing does not typically return to zero.

  - Advance the Swan-Ganz catheter further until a typical right ventricular pressure tracing is seen and record transduced pressures [1]. Observe that right ventricular pressure reaches zero [2]. Check for catheter whip artifact caused by movement of the catheter tip with blood flow, resulting in spuriously high systolic spikes [3].

    - SCREEN: Show the monitor recording the changing of right atrial pressure tracing to right ventricular pressure tracing.

    - SCREEN: Show the typical right ventricular pressure tracing on the monitor returning to zero.

    - SCREEN: Show an example of catheter whip artifact on the waveform with falsely high systolic spikes.

  - Slowly advance the Swan-Ganz catheter further until a pulmonary arterial tracing is obtained and notice that pulmonary arterial pressure does not reach zero [1]. Record the transduced pressures [2]. If there is difficulty advancing the catheter into the pulmonary artery, briefly inflate the balloon to encourage the catheter tip to move toward the pulmonary valve [3]. Reposition the animal as another troubleshooting technique [4].

    - SCREEN: Show the monitor recording the changing of right ventricular pressure tracing to pulmonary arterial pressure tracing that does not return to zero.

    - SCREEN: Show the transduced pulmonary artery pressure values being recorded.

    - Talent briefly inflating the balloon to help move the catheter tip toward the pulmonary valve.

    - Talent repositioning the animal.

  - Inspect the waveform for over-dampening caused by kinks in the tubing [1]. Recognize that over-dampening can produce a falsely low pressure reading [2].

    - Talent checking the tubing for kinks while observing the waveform.

    - SCREEN: Show an over-dampened waveform with falsely low pressure reading.

  - Further advance the catheter [1] and inflate the balloon to create a wedge against the vessel wall, allowing measurement of pulmonary capillary wedge pressure [2]. Limit balloon inflation to a maximum of ten seconds once the waveform is confirmed [3]. Observe that balloon inflation should be accompanied by a feeling of resistance [4]. If no resistance is appreciated, consider that the balloon may have ruptured and do not instill any additional air [5].

    - Reuse 4.1.1.

    - Talent inflating the balloon to wedge the catheter against the vessel wall for PCWP measurement.

    - SCREEN: Show the confirmed wedge waveform while the balloon remains inflated briefly.

    - Talent feeling resistance while inflating the balloon.

    - Talent stopping air instillation after recognizing the absence of resistance.

  - Measure cardiac output via thermodilution using the designated machine and its specific instructions [1-TXT]. Connect the cardiac output machine to the port on the Swan-Ganz catheter [2]. Place the thermistor cable in a container of ice water [3]. Input the appropriate computational constant based on the catheter model and injectate temperature to compute cardiac output [4].

    - Talent preparing the cardiac output machine for thermodilution measurement.TXT: Exact instructions vary by model

    - Talent connecting the cardiac output machine to the port on the Swan-Ganz catheter.

    - Talent placing the thermistor cable in a container of ice water.

    - SCREEN: Show the computational constant being entered on the CO machine based on the catheter model and injectate temperature.

  - Place eight five milliliter syringes pre-filled with zero-point nine percent saline in ice water [1]. Instill three to five milliliters of cold injectate at zero to five degrees Celsius through the catheter, depending on the model of Swan-Ganz catheter used [2] and record the cardiac output [3-TXT].

    - Talent placing eight 5 milliliter syringes filled with saline in ice water.

    - Talent instilling the cold injectate through the catheter.

    - SCREEN: Show the cardiac output value displayed on the machine and being recorded. TXT: Follow the machine prompts; Repeat the cold injectate 5 to 6 times

  - Deflate the balloon [1] and withdraw the Swan-Ganz catheter [2]. If the cardiac catheterization is not done on the final day of the study, remove the introducer [3] and ligate the external jugular vein using 3.0 silk sutures [4]. Monitor the animal for complications [5].

    - Talent deflating the balloon.

    - Talent withdrawing the Swan-Ganz catheter.

    - Talent removing the introducer.

    - Talent ligating the external jugular vein with 3.0 silk sutures.

    - Talent monitoring the animal for complications.

# Results

- Results

  - Using cardiac catheterization technique, the hemodynamics in twelve neonatal lambs including both term and preterm animals was assessed, and the mean right ventricle pressure [1], pulmonary artery pressure [2], pulmonary capillary wedge pressure [3] and cardiac output are reported [4]. The pulmonary vascular resistance values are also calculated from mean pulmonary artery pressure, pulmonary capillary wedge pressure, and cardiac output measurements [5].

    - LAB MEDIA: Table 1: Video Editor: Please emphasize the column labelled as “Mean Right Ventricle Pressure (mmHg)”

    - LAB MEDIA: Table 1: Video Editor: Please emphasize the column labelled as “Mean Pulmonary Artery Pressure (mmHg)”

    - LAB MEDIA: Table 1: Video Editor: Please emphasize the column labelled as “Mean Pulmonary Capillary Wedge Pressure (mmHg)”

    - LAB MEDIA: Table 1: Video Editor: Please emphasize the column labelled as “Cardiac Output (L/min/kg BW)”

    - LAB MEDIA: Table 1: Video Editor: Please emphasize the column labelled as “Pulmonary Vascular Resistance (mmHg/[L/min]/kgBW)”