TITLE:

Microvascular Perfusion Monitored by Laser Speckle Contrast Imaging during Renal Ischemia-Reperfusion Injury in Mice

AUTHORS AND AFFILIATIONS:

Weibin Ye1#, Yukuan Chen1#, Jinwei Guo1, Huaying Zheng1, Yingbi Zhou1, Bin Liu1*

1Department of Cardiology, First Affiliated Hospital & Cardiovascular Research Center, Shantou University Medical College, Shantou, China

Email addresses of the co-authors:

Weibin Ye		(22wbye1@stu.edu.cn)

Yukuan Chen		(18ykchen1@stu.edu.cn)

Jinwei Guo		(19jwguo1@stu.edu.cn)

Huaying Zheng	(22hyzheng@stu.edu.cn)

Yingbi Zhou		(yz_sumc@163.com)

*Corresponding author

Bin Liu			(bliu@stu.edu.cn)

#These authors contributed equally to this work

SUMMARY:

A laser speckle contrast imaging system was used to monitor the changes in renal microvascular blood flow during unilateral renal ischemia-reperfusion after the resection of the right kidney in mice.

ABSTRACT:

Laser speckle contrast imaging (LSCI) is a non-invasive technique for measuring superficial blood perfusion over large areas. With its high spatiotemporal resolution, LSCI semi-quantifies heterogeneous blood flow changes across different regions within the imaged area. Here, we demonstrate the technique of LSCI to record blood flow during unilateral renal ischemia-reperfusion injury (IRI) in mice. Following right nephrectomy, the position of the mouse and the focal point of the objective of the LSCI system were adjusted to ensure the left kidney was in the field of view. LSCI scans were performed before, during, and after the ischemia of the kidney. Furthermore, by selecting regions of interest (ROIs), blood flow in the regions was simultaneously monitored. The method of LSCI visualizes microvascular perfusion of the kidney and allows for a real-time mapping of blood flow. It may be utilized to monitor the microvascular blood flow and evaluate the effect of an intervention on the reperfusion kinetics and spatial homogeneity during mouse renal IRI or other similar animal models.

INTRODUCTION:

Ischemia-reperfusion injury (IRI) is among the leading causes of acute kidney injury (AKI), which is associated with high morbidity and high mortality in patients1,2. The change of renal hemodynamics is one of the critical factors of IR-induced AKI1. IR-evoked microcirculatory dysfunction is a major reason for the development of injury, leading to oxygen deprivation and mitochondrial dysfunction of cells3,4. Therefore, monitoring blood flow changes in real time is an effective method for ensuring the successful ischemia or reperfusion of the kidney and assessing the degree of IRI in animal models for the disorder.

There are multiple techniques to measure the microvascular perfusion of tissues, including laser Doppler flowmetry (LDF)5, nicotinamide adenine dinucleotide dehydrogenase (NADH) autofluorescence imaging6–8, infrared imaging9, and indocyanine green (ICG) angiography10. Among them, infrared imaging and NADH autofluorescence imaging indirectly measure the microvascular perfusion. LDF, using an optical probe, only monitors regional blood flow5, and scanning LDF is not quick enough to monitor the real-time changes of tissue microvascular perfusion under I/R conditions11. ICG angiography can also identify renal blocked vessels10 but requires the administration of a contrast agent, which restricts its use to a limited number of measurements12. Laser speckle contrast imaging (LSCI) is an efficient non-invasive method for monitoring renal microperfusion in real-time13. It has been used to generate spatially resolved information related to blood flow dynamics. As a coherent beam is shone onto a rough surface, the reflected or scattered light waves will interfere with each other due to minute path differences, forming a random distribution of bright and dark spot patterns, known as "speckle". LSCI maps the speed of blood flow in superficial tissues by analyzing the degree of blurriness of the "dynamic speckle" produced when laser light irradiates the tissue12,14,15. In this study, the LSCI system operated at a laser wavelength of 785 nm with a spatial resolution of 3 μm, a field of view adjustable between 6.3 mm × 4.7 mm and 42 mm × 31 mm, and an imaging speed of up to 120 frames per second, enabling high-resolution, real-time mapping of cortical microvascular perfusion. In light of these advantages, LSCI has been applied in a range of experimental and clinical contexts for blood flow monitoring, including burn wounds16, retinal perfusion17, cerebral blood flow18, esophagus19, and the intestine20.

It should be noted that LSCI primarily assesses perfusion in superficial tissue, provides relative rather than absolute flow values, and can be influenced by tissue optical properties and motion artifacts. These inherent characteristics are considered when interpreting the imaging data.

This work, using a model involving right nephrectomy to facilitate exposure and imaging of the contralateral kidney, monitored the changes in microvascular perfusion before, during, and after the ischemia in a unilateral renal I/R mouse model using LSCI and demonstrated the spatial heterogeneity of blood flow on the surface of the kidney.

PROTOCOL:

All animal procedures complied with the NIH Guide for the Care and Use of Laboratory Animals and the Guide for the Care and Use of Laboratory Animals, and were approved by the Institutional Animal Care and Use Committee of Shantou University Medical College. To eliminate gender and strain-based differences in IRI effects, only male C57BL/6N mice were used in the study. All mice were matched in age and weight to produce comparable results. A sample size of n = 5 was used, consistent with prior methodological studies utilizing LSCI for technical validation21. The animals received humane care and were euthanized after the experiments by CO2 inhalation. The reagents and the equipment used are listed in the Table of Materials.

Before beginning the surgical procedure, turn on the laser speckle contrast imaging system. The system used in this study operates with a 785 nm laser. Set the working distance to approximately 15-20 cm above the surgical field. The system calculates blood flow in Laser Speckle Perfusion Units (LSPU) based on the spatial contrast algorithm. Ensure the instrument is warmed up and stable before proceeding.

- Animal preparation and laparotomy

1.1 Sterilize the surgical table before each procedure. Prepare and place all required materials (sterilized instruments and cotton swabs, gauze, pre-diluted anesthetics, heating pad, vascular clamp, saline solution, depilatory cream, cotton balls, stitches, skin antiseptics, and suture) on the surgical table.

1.2 Administer meloxicam (0.2 mg/kg body weight; subcutaneous injection) to the mouse 30 min in advance to alleviate pain.

1.3 Pour a certain amount of sterilized saline into a clean container. Then take a piece of gauze, and cut out a suitable opening in the middle.

1.4 Anesthetize a mouse by intraperitoneal injection of 2.5% Tribromoethanol (400 mg/kg body weight; previously diluted in sterile saline).

NOTE: Tribromoethanol was used as an anesthetic during the process of right nephrectomy. And 1% isoflurane was used for anesthesia during prolonged procedures of monitoring blood flow.

1.5 Place the mouse onto the heating pad with tape and use depilatory cream to remove the hair from the area on the mouse's abdomen. Throughout surgery, use the heating pad to maintain rectal temperature at 37 °C ± 0.5 °C.

1.6 Disinfect the skin in the surgical area with 75% alcohol first and then with iodophor using a cotton swab.

1.7 Use forceps to pinch the mouse's toe. Once the mouse shows no pain response, make a vertical incision along the midline of the abdomen using scissors. Gently separate the skin from the peritoneum, then cut open the peritoneum and enter the abdominal cavity.

1.8 Apply a retractor to the mouse to optimize visibility and cover the operative field with gauze, while aligning the gap on the gauze with the abdominal cavity, and moisten it with saline.

- Right nephrectomy

2.1 Use a cotton swab to gently push the intestine towards one side of the gauze, exposing the right kidney and the surrounding tissues. Fold the gauze to cover the intestine and moisten it with normal saline to prevent the intestine from drying out.

2.2 Use angled forceps to lift the right ureter. Pass two 6-0 suture threads through the ureter and perform ligation at two positions.

2.3 Transect the ureter between the two ligatures using microscissors.

2.4 After moistening the cotton balls with normal saline, use fine forceps to pick up the cotton balls, gently push the liver upwards, and fix it with the cotton balls, to expose the kidneys and the blood vessels on their right side.

2.5 Use angled forceps to bluntly separate the perinephric fat pads along the side of the right kidney until the renal artery and vein are fully exposed.

NOTE: Simultaneously separate the mass of adrenal tissue located above the kidney.

2.6 Carefully slide angled forceps underneath the renal artery and vein to create a channel. Then, pass a 6-0 silk braided suture around the vessels to ligate them together.

2.7 Cut the occluded renal artery and vein located close to the kidney. Remove the kidney along with the remaining adherent tissue.

2.8 Remove the cotton ball used to hold the liver.

- Monitoring renal blood flow with Laser Speckle Contrast Imaging (LSCI) during ischemia-reperfusion

3.1 Turn on the LSCI system and the instrument's dedicated software.

3.2 Click on Original Image and Preview to view the raw, unprocessed image of the surgical field.

3.3 Change the position of the mice, maintain anesthesia using 1% isoflurane delivered via a nose cone, and adjust the camera's magnification while observing the original image to achieve a clear view of the target kidney.

3.4 Click on Simultaneous Display to view the real-time blood flow image. Set magnification to 2.5× and select Automatic Blood Flow Adjustment to optimize the image. Then, lock the Blood Flow Parameters. Besides, set up to record one frame of the picture every 5000 ms.

NOTE: Wet cotton balls can be used to ensure the stable position of the kidney and keep the surrounding organs and tissues from it, enhancing the distinction between the kidney area and the background.

3.5 Set the size of the square area parameters to 112 and arrange the 5 square areas uniformly in a fixed orientation on the kidney view. Click on the Real-time Image Registration and Record to begin recording blood flow data.

NOTE: The machine will automatically record the laser speckle perfusion units (LSPU), which represent the real-time blood flow units in the ROIs and can be used for analyses.

3.6 After recording a 30 min baseline, separate the adipose tissue surrounding the left kidney vessels. Use the hemostatic forceps to grasp a micro-clamp and place it across the left renal artery and vein near the hilum. Continue real-time monitoring for another 30 min.

3.7 Release the micro-clamp to initiate reperfusion. Monitor and record the real-time recovery of renal microvascular blood flow.

3.8 Once the blood flow reperfusion is monitored for at least 60 min, stop monitoring and suture the abdominal incision with a 7-0 silk braided suture.

- Post-operative recovery and care

4.1 Administer fluids with an intraperitoneal injection of 1 mL saline to prevent dehydration following the surgery.

4.2 Keep the mice on the heat pad. Carefully monitor mice until they have recovered consciousness, appear alert, and are able to right themselves.

REPRESENTATIVE RESULTS:

This article aimed to detect changes in renal blood flow during IR-induced AKI. Therefore, we employed a common mouse model that involves right nephrectomy (Figure 1A) followed by an IR procedure on the contralateral kidney (Figure 1B). Because real-time monitoring of blood flow during IRI was required, all manipulations on the mice were performed at the instrument region throughout the entire process (Figure 1C).

After setting the instrument parameters and adjusting the field of view, we first conducted a 30 min blood flow measurement to monitor the baseline level of renal microvascular perfusion. During the initial 30 min, blood flow in the mouse renal microvasculature gradually became stable (Figure 2A–C). Notably, there existed heterogeneity in the perfusion level of different ROI before ischemia (Figure 2B). Following the clamping of the renal artery and vein, the blood flow dropped sharply, and the entire renal microcirculation immediately entered a state of no perfusion (Figure 2A–D). Upon the release of the clamp, the blood flow surged instantaneously and reached a peak (Figure 2B,C). Subsequently, the blood flow first decreased and then gradually recovered to a comparatively stable level (Figure 2B,C). By selecting an ROI, we noticed that microvascular perfusion after IR (during 60 min of reperfusion) was much lower than that before ischemia (Figure 2D).

FIGURE LEGENDS:

Figure 1: An overview of the experimental setup and procedure. (A) A schematic diagram showing the right nephrectomy. (B) A photograph showing the detection of microvascular blood flow and the ischemia-reperfusion (IR) surgical procedure. (C) A schematic diagram and a photograph demonstrating the ischemia surgery.

Figure 2: Monitoring of renal microvascular blood flow during IR. The mice experienced normal perfusion, ischemia, and reperfusion (30/30/60 min) of the kidney. (A) Representative pseudo-color images showing renal microvascular blood flow changes before, during, and after ischemia. Blood flow was monitored by laser speckle contrast imaging (LSCI). The region of interest (ROI) was marked by a square. Blue represents low speed of blood flow, and red represents high speed. (B) Laser speckle perfusion unit (LSPU) values of real-time blood flow in different ROIs. (C) LSPU values of real-time blood flow in the whole view of the renal microcirculation. (D) Normalized LSPU values of renal microvascular blood flow in an ROI before, during, and after ischemia. The selected ROI was located in the middle between the renal hilum and the periphery. The data were derived from the average values of the stable blood perfusion (5 min) during the three different periods and normalized to those of normal perfusion before ischemia (n = 5). Values are expressed as mean ± SD. Statistical analyses were performed with a paired Student's t-test. **P < 0.01.

DISCUSSION:

This work describes the application of an LSCI system for the real-time, high-resolution visualization of renal microvascular blood flow dynamics during IRI in mice. The protocol offers a significant advantage over traditional endpoint measurements or methods with limited spatial resolution, such as LDF, by providing a two-dimensional perfusion map that captures regional heterogeneity within the kidney microcirculation. The efficiency and usability of LSCI for intraoperative real-time monitoring are key strengths of this method.

By monitoring the changes in blood flow before, during, and after ischemia in male mice, we found that upon reperfusion after a 30 min period of ischemia, the renal microvascular blood flow in mice cannot return to the baseline level, and there exists heterogeneity in regions proximal or distal to the renal hilum. The results are consistent with those obtained with rats11. This pattern of initial hyperemia followed by sustained hypoperfusion shares similarities with microvascular responses observed in the cerebrum model21, suggesting common pathophysiological mechanisms across organs. The selection of appropriate ROIs enables the quantitative analysis of spatial differences in perfusion kinetics of the kidney. Also, LSCI can provide the real-time mapping of microvascular perfusion of the whole kidney. These are key strengths of the method. Our results indicate that the method can be used to: (1) monitor whether the surgery-inducing renal IRI is successful, (2) simultaneously detect the real-time perfusion of different ROIs in the renal microvasculature, and (3) assess whether an intervention can improve the microvascular perfusion after IR.

A critical step for the successful implementation of the measurement is the surgical preparation to ensure a stable and clear imaging field. Right nephrectomy can provide a good view and ensure typical signs of AKI after IR of the remaining kidney. Following right nephrectomy, the proper positioning of the left kidney and isolation of it from surrounding organs using saline-moistened cotton balls are essential to minimize motion artifacts and enhance contrast. If motion artifacts occur, ensure the kidney is stabilized and consider adjusting the depth of anesthesia. If image contrast is poor, verify laser focus and ensure the surgical field is free of excessive blood or fluid22. Furthermore, standardizing instrument parameters, such as the magnification, focus, and locking the blood flow adjustment settings, before initiating recording, is crucial for obtaining consistent and comparable data across experiments.

The primary limitation of LSCI is that it can only detect surface perfusion, meaning it captures blood flow in the superficial microvasculature but does not provide information on deep medullary or single-vessel hemodynamics. Secondly, the right nephrectomy adds to the surgical duration and physiological stress that may influence baseline and reperfusion perfusion dynamics. In addition, measurements are relative rather than absolute, and can be influenced by factors such as tissue optical properties and ambient light. Excessive bleeding or poor isolation of the kidney can also introduce artifacts that complicate interpretation. Furthermore, this study focused on validating the imaging technique itself and did not correlate LSCI measurements with functional endpoints (e.g., serum creatinine, BUN) or histopathological injury scores. This is a limitation as it fails to prove the success of the model construction from other physiological aspects. However, the significant reduction in perfusion volume during ischemic conditions can demonstrate the good completion of the surgery. Future studies integrating LSCI with such multimodal assessments would strengthen the physiological relevance of the perfusion data. Additionally, while the within-animal baseline–ischemia–reperfusion design provides internal control, the inclusion of a sham-operated group in future studies would help to clearly elucidate whether there is an involvement of surgical manipulation, anesthesia, or time-dependent drift. Therefore, the technique is best used as a complementary tool alongside other physiological and histological assessments of renal IRI.

The method described here represents an effective approach to mapping the real-time microvascular perfusion during renal IRI. It may be utilized to quantitatively evaluate the efficacy of therapeutic interventions aimed at mitigating IRI by assessing their impact on reperfusion kinetics and spatial homogeneity. Furthermore, the protocol may also be adapted to study blood flow dynamics in the surfaces of other organs exposed by laparotomy, such as the liver, spleen, and mesentery, during various ischemic or inflammatory insults.

ACKNOWLEDGMENTS:

This work was funded by the Innovative Team Research Program for Universities of Guangdong Province (2022KCXTD009).

DISCLOSURES:

The authors have no conflicts of interest to disclose.

REFERENCES:

- Weight, S C., Bell, P R., Nicholson, M L. Renal ischaemia-reperfusion injury. Br J Surg. 83 (2), 162–170 (1996).

- Schrier, R W., Wang, W., Poole, B., Mitra, A. Acute renal failure: Definitions, diagnosis, pathogenesis, and therapy. J Clin Invest. 114 (1), 5–14 (2004).

- Kosieradzki, M., Rowiński, W. Ischemia/reperfusion injury in kidney transplantation: Mechanisms and prevention. Transplant Proc. 40 (10), 3279–3288 (2008).

- Legrand, M., Mik, E. G., Johannes, T., Payen, D., Ince, C. Renal hypoxia and dysoxia after reperfusion of the ischemic kidney. Mol Med. 14 (7–8), 502–516 (2008).

- Zhu, X. et al. Ischemic preconditioning prevents in vivo hyperoxygenation in postischemic myocardium with preservation of mitochondrial oxygen consumption. Am J Physiol Heart Circ Physiol. 293 (3), H1442–1450 (2007).

- Coremans, J. M. C. C. et al. Pretransplantation assessment of renal viability with NADH fluorimetry. Kidney Int. 57 (2), 671–683 (2000).

- Fitzgerald, J. T., Demos, S., Michalopoulou, A., Pierce, J. L., Troppmann, C. Assessment of renal ischemia by optical spectroscopy. J Surg Res. 122 (1), 21–28 (2004).

- Raman, R. N., Pivetti, C. D., Matthews, D. L., Troppmann, C., Demos, S. G. A non-contact method and instrumentation to monitor renal ischemia and reperfusion with optical spectroscopy. Opt Express. 17 (2), 894 (2009).

- Gorbach, A. M. et al. Assessment of critical renal ischemia with real-time infrared imaging. J Surg Res. 149 (2), 310–318 (2008).

- Ietto, G. et al. Indocyanine green angiography for quality assessment of the kidney during transplantation: An outcome predictor prospective study. Transplant Proc. 53 (6), 1892–1896 (2021).

- Bezemer, R. et al. Real-time assessment of renal cortical microvascular perfusion heterogeneities using near-infrared laser speckle imaging. Opt Express. 18 (14), 15054 (2010).

- Dunn, A. K. Laser speckle contrast imaging of cerebral blood flow. Ann Biomed Eng. 40 (2), 367–377 (2012).

- Heeman, W. et al. Real-time visualization of renal microperfusion using laser speckle contrast imaging. J Biomed Opt. 26 (05), 056004 (2021).

- Briers, J. D. Laser speckle contrast analysis (LASCA): A nonscanning, full-field technique for monitoring capillary blood flow. J Biomed Opt. 1 (2), 174 (1996).

- Boas, D. A., Dunn, A. K. Laser speckle contrast imaging in biomedical optics. J Biomed Opt. 15 (1), 011109 (2010).

- Stewart, C. J. et al. A comparison of two laser-based methods for determination of burn scar perfusion: Laser Doppler versus laser speckle imaging. Burns. 31 (6), 744–752 (2005).

- Tamaki, Y., Araie, M., Kawamoto, E., Eguchi, S., Fujii, H. Noncontact, two-dimensional measurement of retinal microcirculation using laser speckle phenomenon. Invest Ophthalmol Vis Sci. 35 (11), 3825–3834 (1994).

- Kazmi, S. S., Richards, L. M., Schrandt, C. J., Davis, M. A., Dunn, A. K. Expanding applications, accuracy, and interpretation of laser speckle contrast imaging of cerebral blood flow. J Cereb Blood Flow Metab. 35 (7), 1076–1084 (2015).

- Ambrus, R. et al. Evaluation of gastric microcirculation by laser speckle contrast imaging during esophagectomy. J Am Coll Surg. 225 (3), 395–402 (2017).

- Knudsen, K. et al. Laser speckle contrast imaging to evaluate bowel lesions in neonates with NEC. Eur J Pediatr Surg Rep. 5 (1), e43–e46 (2017).

- Yin, L. et al. Laser speckle contrast imaging for blood flow monitoring in predicting outcomes after cerebral ischemia-reperfusion injury in mice. BMC Neurosci. 23 (1), 80 (2022).

- Hren, R., Brezar, S. K., Marhl, U., Sersa, G. Laser speckle contrast imaging of perfusion in oncological clinical applications: A literature review. Radiol Oncol. 58 (3), 326–334 (2024).