# Submission ID #: 70845

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/account/file-uploader?src=21607773

Title: A Low-Cost Markerless DeepLabCut-Based Workflow for Spontaneous Gait and Locomotion Analysis in Freely Moving Mice

# Authors and Affiliations:

Jesús Andrade-Guerrero1,2†, Alejandro Meda-Hernández3†, Valeria Sasia-Saldivar2, Sofía Y. Díaz- Miranda2*, Luis O. Soto-Rojas1

1Laboratorio de Investigación en Neurociencias y Enfermedades Neurodegenerativas (LINEN), Carrera de Médico Cirujano, Facultad de Estudios Superiores Iztacala, Universidad Nacional Autónoma de México

2Departamento de Neurobiología del Desarrollo y Neurofisiología, Instituto de Neurobiología, Universidad Nacional Autónoma de México

3Laboratorio de interacciones Neurona-Astrocito (FINA), Escuela Nacional de Estudios Superiores Juriquilla, Universidad Nacional Autónoma de México

†These authors contributed equally to this work.

# Corresponding Authors:

Sofía Y. Díaz- Miranda             (yoldi@unam.mx)

Luis O. Soto-Rojas                   (oskarsoto123@unam.mx )

# Email Addresses for All Authors:

Jesús Andrade-Guerrero         (jesusandrade1007@gmail.com)

Alejandro Meda-Hernández   (alexmdahdez98@gmail.com)

Valeria Sasia-Saldivar               (vsasia.14@gmail.com)

Sofía Y. Díaz- Miranda             (yoldi@unam.mx)

Luis O. Soto-Rojas                   (oskarsoto123@unam.mx )

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  YES

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21607773

3. Filming location: Will the filming need to take place in multiple locations?   NO

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. NO

Current Protocol Length

Number of Steps:  48

Number of Shots:  64  (48 SCs)

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Karen Arcia: Our research focuses on low-cost, markerless methods to quantify gait and locomotion in freely moving mice.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera Suggested B roll: 3.3.1

Describe the research contexts or systems in which this protocol can be applied.

  - Karen Arcia: This protocol enables natural, unrestricted assessment of movement and gait across diverse preclinical research applications.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Marcos Rojas: This protocol allows researchers to quantify spontaneous locomotion, gait dynamics, and limb coordination under natural, unconstrained conditions.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the most important consideration or challenge when performing this protocol.

  - Marcos Rojas: The most important challenge when performing this protocol is that it cannot directly measure biomechanical variables such as ground reaction forces or plantar pressure.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Marcos Rojas: Future studies can expand this workflow with 3D motion analysis and validation across diverse movement disorder models.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Videographer: Obtain headshots for all authors available at the filming location.

# Ethics Title Card

This research has been approved by the Bioethics Committee of the Institute of Neurobiology, National Autonomous University of Mexico

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Open Field Arena and Camera Preparation

Demonstrators: Karen Arcia, Marcos Villegas

  - Acclimate both the triple-transgenic Alzheimer's disease mice and non-transgenic control mice in the behavioral testing room for 60 minutes before the experiment [1].

    - Establishing shot of the talent with the mouse cages in the testing room. Videographer: Please obtain a CU shot as well for this shot

  - Clean the acrylic open field arena thoroughly with 70% ethanol to remove residual odors and scent cues and allow the surface to dry completely [1].

    - Talent cleaning the acrylic open field arena with 70% volume per volume ethanol.

  - Handle 70% ethanol in a well-ventilated area, away from heat or ignition sources. Allow the arena to dry completely before use [1].

    - Shot of 70% ethanol positioned away from heat or ignition sources.

  - Place the open field arena securely on the wooden platform and confirm that the structure is stable and free of vibrations [1]. Inspect the transparent acrylic walls and the matte floor surface to confirm they are free of dust, moisture, and reflective residues [2].

    - Talent places the open field arena on the wooden platform and inspects the arena setup for stability.

    - Talent inspecting the walls and floor for dust, moisture, or reflective residues.

  - Position the external lateral white light lamps on both sides of the arena at an approximate angle of 30 to 45 degrees relative to the arena plane to provide uniform illumination [1]. Adjust the lamp placement before the recording session [2] and verify that illumination at the arena level remains within the target range of 120 to 150 lux [3].

    - Talent positioning the external lateral white light lamps on both sides of the arena at an approximate angle of 30 to 45 degrees relative to the arena plane.

    - Talent adjusting the placement of the light lamps.

    - Talent checking illumination at the arena level.

  - Position the camera below the arena in a bottom view configuration at a fixed distance of approximately 80 centimeters from the arena plane, ensuring complete visibility of the recording area [1].

    - Talent positioning the camera below the arena in a bottom view configuration approximately 80 centimeters from the arena plane.

  - Connect the camera to the computer system [1] and initiate the video acquisition software before introducing the animal [2].

    - Talent connecting the camera to the computer system.

    - SCREEN: The video acquisition software being launched on the computer.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21607773

  - Adjust the camera focus and field of view to ensure that the arena surface is visible and sharply resolved [1]. Acquire a reference image for spatial calibration by placing the checkerboard grid or calibration pattern with known dimensions on the arena plane [2].

    - SCREEN: The camera focus and field of view is being adjusted for a sharp image of the arena surface.

    - Talent placing the checkerboard grid or calibration pattern with known dimensions on the arena.

- Recording Spontaneous Locomotor Behavior

Demonstrators: Alejandro Hernandez, Valeria Saldivar

  - Before placing the mouse into the arena, display the animal identifier in the video frame to ensure direct visual association between the recording and the corresponding experimental session [1].

    - Talent displaying the animal identifier within the video frame.

  - Initiate video recording before placing the mouse in the center of the open field arena to capture the initial exploratory behavior from the beginning of the session [1]. Gently place the mouse in the center of the open field arena [2].

    - SCREEN: The recording is being initiated in the video acquisition.

    - Talent gently placing the mouse in the center of the open field arena.

  - Continuously record spontaneous locomotion for 3 minutes without any visual, auditory, or tactile disturbances. Limit recording sessions to one per mouse [1]. Monitor the live video feed on the computer screen to confirm uninterrupted recording and proper capture of the animal’s movements [2].

    - SCREEN: Shot of the mouse moving freely in the open field arena during recording.

    - SCREEN: The live video feed is being monitored during recording.

  - Verify camera alignment and illumination uniformity between recording sessions to ensure consistent recording conditions across animals [1].

    - Talent checking camera alignment between recording sessions.

  - At the end of the recording period, stop video acquisition [1] and carefully remove the animal from the arena using the same handling procedure used previously [2].

    - SCREEN: The recording is being stopped.

    - Talent carefully removes the mouse from the open field arena.

  - Return the animal to its home cage and allow the animal to rest before any further procedures [1].

    - Talent returning the mouse to its home cage.

  - Clean the open-field arena with 70% ethanol after the session and allow it to dry completely before introducing the next animal [1]. Confirm that no residual ethanol or odor remains on the arena surface before introducing the next animal [2].

    - Talent cleaning the open field arena with 70% volume per volume ethanol.

    - Talent inspecting the arena surface to confirm the absence of residual ethanol or odor.

  - Document the animal identifier, session date and time, experimental condition, and the operator’s name immediately after recording [1].

    - Talent recording the animal identifier, session date and time, experimental condition and operator name.

  - Save the video file in a dedicated folder using a standardized naming convention [1].

    - SCREEN: The video file is being saved in a dedicated folder.

  - At the end of the experimental day, shut down the camera system and the lighting equipment to prevent drift in illumination settings across sessions [1].

    - Talent shutting down the camera system and lighting equipment.

- Video Processing and Kinematic Analysis

  - Acquire the raw videos in MOV (M-O-V) format [1]. Convert the videos to AVI format while maintaining a constant frame rate and resolution for compatibility with DeepLabCut (Deep-Lab-Cut) [2].

    - SCREEN: Show the raw video files in MOV format.

    - SCREEN: The video is being converted to AVI format with frame rate and resolution parameters maintained.

  - Back up all raw video files immediately after acquisition to a dedicated folder on the analysis computer and apply a standardized naming convention [2].

    - SCREEN: The raw video files are being copied to a dedicated folder on the analysis computer and named.

  - Review each video in its entirety to confirm that the recording is complete and free of interruptions, dropped frames, or corruption. Exclude videos with acquisition errors from analysis [1].

    - SCREEN: A video is being reviewed for completeness and image continuity.

  - Convert the videos to MP4 or uncompressed AVI format if the original format is not compatible with DeepLabCut [1].

    - SCREEN: The incompatible video files are being converted to MP4 or uncompressed AVI format.

  - Optionally, perform temporal trimming of the video footage to eliminate non-experimental intervals at the beginning and end of the session [1].

    - SCREEN: The beginning and end portions of a video are being trimmed to remove non-experimental intervals.

  - Confirm that the resolution and frame rate match the acquisition parameters used during recording and remain consistent across all sessions [1].

    - SCREEN: The video resolution and frame rate are being verified across multiple sessions.

  - Verify that the entire arena surface is visible within the video frame and no body parts are persistently occluded during locomotion [1]. Place all verified and processed videos into the designated project directory for DeepLabCut analysis [2].

    - SCREEN: A video frame with the entire arena surface and body parts visible is being shown.

    - SCREEN: The verified and processed video files are being organized into the DeepLabCut project directory.

  - Designate a video library for the project using either the videos intended for analysis or a representative subset for network training [1].

    - SCREEN: A video library is being designated for analysis videos or a representative training subset.

  - Next, launch the Python environment and import the DeepLabCut package [1]. Create a new DeepLabCut project using the suitable command [1].

    - SCREEN: The Python environment is being launched, and the DeepLabCut package is being imported.

    - SCREEN: The command deeplabcut.create_new_project('ProjectName','Author','VideoPath') is being entered and executed in the Python environment.

  - Then, define the anatomical landmarks or body parts of interest in the config.yaml (config-Y-A-M-L) file while ensuring that the definitions are clear and anatomically consistent [1]. Optimize the body-part definitions for the experimental question and maintain identical body-part definitions across all animals and sessions [2].

    - SCREEN: The anatomical landmarks or body parts are being entered. in the config.yaml file

    - SCREEN: The body part definitions are being defined according to the experimental objective.

- Frame Extraction and Body Part Labeling

  - Specify the start, stop, numframes2extract (Num-Frames-Two-Extract), and cropping_parameters in the config.yaml file according to the video length and spatial layout [1].

    - SCREEN: The start stop, numframes2extract, and cropping_parameters parameter being entered in the config.yaml file.

  - Using the appropriate command, extract representative frames [1]. Review the extracted frames to confirm the inclusion of variation in body posture, orientation, limb position, and gait-phase coverage across the recording [2].

    - SCREEN: The command deeplabcut.extract_frames(config_path, 'automatic', 'kmeans'/'uniform') is being entered and executed.

    - SCREEN: Show extracted frames displaying variation in body posture, orientation, limb position, and gait phases.

  - Extract frames manually using the suitable command when specific gait cycle phases are required [1].

    - SCREEN: The command deeplabcut.extract_frames(config_path, 'manual') is being entered and executed.

  - Next, launch the graphical labeling interface [1].

    - SCREEN: The command deeplabcut.label_frames(config_path) being entered and executed to open the graphical labeling interface.

  - Identify all predefined anatomical landmarks, including the representative video frame captured during acquisition [1] and the automated body point detection superimposed on the original frame [2]. Indicate the defined body landmarks and arena reference points used for analysis [3].

    - SCREEN: A representative video frame captured during acquisition with labeled landmarks is being shown.

    - SCREEN: The automated body point detection is being overlaid on the original frame.

    - SCREEN: The defined body landmarks and arena reference points used for analysis are being indicated.

  - Label all four limbs consistently by placing each landmark at the center of the paw over the plantar pads at ground contact [1].

    - SCREEN: The landmarks are being placed at the center of each paw over the plantar pads at ground contact.

  - Use the suitable command to save and inspect the labels and correct inaccurate annotations if needed [1].

    - SCREEN: The labels are being saved, and inaccurate annotations are being corrected

  - When feasible, have a second trained evaluator label a subset of frames to assess inter-rater consistency [1].

    - SCREEN: A subset of frames are being labeled by a second trained evaluator.

- Video Analysis and Gait Quantification

  - Use the appropriate command to run inference on the experimental videos [1]. Generate labeled videos to visually confirm tracking quality [2].

    - SCREEN: The command deeplabcut.analyze_videos(config_path, [video_path], save_as_csv=True) is being entered and executed.

    - SCREEN: The command deeplabcut.create_labeled_video(config_path, [video_path]) being entered and executed, followed by a labeled video displaying tracked landmarks.

  - Set the pcutoff (P-Cut-Off) parameter in the config.yaml file to 0.6 to apply a confidence threshold during tracking visualization and evaluation [1].

    - SCREEN: The pcutoff parameter is being set to 0.6 in the config.yaml file.

  - Import the output CSV files into the statistical computing environment [1] along with the corresponding analysis package and the custom scripts required for behavioral quantification [2].

    - SCREEN: The output CSV files are being imported into the statistical computing environment.

    - SCREEN: The analysis package and custom scripts are being loaded for behavioral quantification.

  - Convert frame indices to time using the original video sampling rate [1].

    - SCREEN: The frame indices are being converted to time values using the original video sampling rate.

  - To calibrate spatial dimensions, transform pixel coordinates into physical units using the known arena dimensions and the tracked arena corner landmarks [1].

    - SCREEN: The pixel coordinates are being transformed into physical units using arena dimensions and tracked arena corner landmarks.

  - Perform quality control on the tracked coordinates before metric extraction [1]. Consider points with a likelihood value below 0.3 as missing and replace missing points with linear interpolation when appropriate [2].

    - SCREEN: The tracked coordinates are being reviewed for quality control prior to metric extraction.

    - SCREEN: The tracked points with likelihood values below 0.3 are identified as missing and replaced.

  - Identify outlier trajectories during visual inspection and correct or exclude them when necessary [1].

    - SCREEN: The outlier trajectories are being identified and  corrected or excluded.

  - Define gait segments using synchronized kinematic criteria based on body center displacement and hind limb motion [1]. Exclude non-locomotor behaviors such as rearing, turning, and grooming [2].

    - ++SCREEN: The gait segments are being identified using body center displacement and hind limb motion criteria.

    - SCREEN: Behaviours like rearing, turning, and grooming are being excluded from the analysis.

  - Manually calibrate and refine the definition thresholds according to the experimental conditions and animal model to retain only robust, continuous locomotor bouts for análysis [1].

    - SCREEN: The locomotor definition thresholds are being adjusted and refined according to the experimental conditions and animal model.

  - Using the filtered continuous locomotor segments, compute gait metrics derived from geometric and temporal relationships among the tracked landmarks [1].

    - SCREEN: The filtered continuous locomotor segments being processed for gait metric extraction.

  - Extract open field metrics such as total distance traveled, average speed, time spent moving, percentage of time spent moving, number and duration of walking bouts, and cumulative distance covered in the center, periphery, and corner zones [1].

    - SCREEN: The calculated open field metrics including total distance traveled, average speed, time spent moving, percentage of time spent moving, number and duration of walking bouts, and cumulative distance covered in the center, periphery, and corner zones are being extracted.

  - Compile all outputs into a structured master file for downstream statistical analysis [1].

    - SCREEN: All extracted metrics and analysis outputs are being compiled into a structured master file.

  - Generate visual representations using the appropriate computational environments and the plotting scripts described previously [1].

    - SCREEN: The generated visual outputs are being shown.

# Results

- Results

  - In a representative non-transgenic mouse recording, the reconstructed body center trajectory showed continuous displacement across the arena [1]. The time-density and speed-coded visualizations highlighted regions of prolonged occupancy [2] and fluctuations in movement velocity during exploration [3].

    - LAB MEDIA: Figure 3A.

    - LAB MEDIA: Figure 3B. Video editor: Highlight the bright occupancy region represented at the corners.

    - LAB MEDIA: Figure 3B. Video editor: Highlight the tracks shown in the grey color range

  - A representative triple transgenic-Alzheimer mouse recording showed more spatially constrained trajectories and reduced arena coverage [1], accompanied by a more localized occupancy distribution and extended periods of lower movement speed [2].

    - LAB MEDIA: Figure 3C.

    - LAB MEDIA: Figure 3D.

  - The reconstructed limb pattern of the non-transgenic mouse recording showed a regular alternation between forelimbs and hindlimbs, with a consistent temporal sequence of stance and swing phases across the recording period [1].

    - LAB MEDIA: Figure 4A. Video editor: Highlight the alternating colored bars for forelimbs and hindlimbs.

  - The gait map showed aligned and parallel limb trajectories reflecting a relatively homogeneous spatial organization of limb movement [1].

    - LAB MEDIA: Figure 4B.

  - A representative triple transgenic-Alzheimer mouse recording showed greater temporal variability in stance phase duration and limb engagement [1]. The limb trajectories appeared more dispersed and less parallel, indicating greater spatial variability during stepping [2].

    - LAB MEDIA: Figure 4C.

    - LAB MEDIA: Figure 4D.