# Submission ID #: 70891

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=21621803

# Title: A Streamlined Protocol for Single-Molecule Localization Microscopy in Arabidopsis Nuclei

# Authors and Affiliations:

# AUTHORS AND AFFILIATIONS:

# Salim El-Khoury1,2, Oleksandr Glushonkov3, Cristel C. Carles2, Jean-Philippe Kleman1

# 1Institut de Biologie Structurale, CEA, CNRS, Université Grenoble Alpes

# 2Plant and Cell Physiology Lab (LPCV), Bioscience and Biotechnology Institute of Grenoble, Grenoble Alpes University – CNRS – INRAE – CEA

# 3Integrated Structural Biology Grenoble (ISBG) CNRS, CEA, Université Grenoble Alpes, EMBL

# Corresponding Authors:

# Cristel C. Carles		christel.carles@univ-grenoble-alpes.fr

# Jean-Philippe Kleman		jean-philippe.kleman@ibs.fr

# Email Addresses for All Authors:

# Salim El-Khoury			salim.el-khoury@univ-grenoble-alpes.fr

# Oleksandr Glushonkov 		Oleksandr.glushonkov@ibs.fr

# Cristel C. Carles		christel.carles@univ-grenoble-alpes.fr

# Jean-Philippe Kleman		jean-philippe.kleman@ibs.fr

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  YES

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://review.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

As these files are necessary for finalizing your script, please upload all screen-captured video files to your project page as soon as possible.

Videographer: Please record the computer screen for the shots labeled as SCREEN as back-up

3. Filming location: Will the filming need to take place in multiple locations?   YES

If Yes, how far apart are the locations? 800m

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps:  23

Number of Shots:  53 (9 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

Answers will be slightly edited in accordance with journal guidelines.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Salim El-Khoury: My research focuses on epigenome editing and the visualization of chromatin organization in Arabidopsis nuclei using super-resolution microscopy.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Salim El Khoury: Existing methods are not adapted to STORM imaging; this protocol improves nuclei purity, labeling consistency, sample mounting and imaging stability.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Salim El Khoury: This protocol enables nanoscale visualization of chromatin structure, histone modifications, and nuclear organization in individual plant nuclei.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the most important consideration or challenge when performing this protocol.

  - Salim El Khoury: Maintaining nuclear integrity while minimizing debris and background fluorescence is critical for achieving high-quality single-molecule imaging.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Salim El Khoury: Future studies can link nuclear architecture to gene regulation under developmental or environmental conditions.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Fixation of Arabidopsis Seedlings

Demonstrator: Salim El-Khoury

  - To begin, place a 6-well culture plate on ice under the fume hood [1]. Mix the the fixation solution containing 4% volume by volume methanol stabilized formaldehyde, 0.1% Triton X-100 and PBS [2]. Add approximately 4 milliliters of ice-cold fixation solution to each well under the fume hood [3].

    - WIDE: Talent placing a 6-well culture plate on ice under the fume hood.

    - Talent inverting the container with the fixation solution under the fume hood.

    - Talent adding fixation solution to each well of the 6-well culture plate under the fume hood.

  - Using clean forceps, remove the Arabidopsis seedlings that are between 8 and 15 days after germination from solid half-strength Murashige and Skoog or MS medium [1]. Immediately submerge the seedlings in the fixation solution [2] and place approximately 20 seedlings per well [3]. Use 3 wells to obtain sufficient material per sample [4].

    - Talent removing seedlings from the solid medium with forceps.

    - Talent submerging the seedlings in the fixation solution.

    - Talent placing approximately 20 seedlings into 1 well.

    - Shot of seedlings added into 3 wells.

  - Now, place the plate in the vacuum chamber filled with ice [1] and apply a vacuum cycling between 0.2 bar and 0.8 bar for 15 minutes to remove trapped air and allow deep penetration of the fixative into the plant tissues [2]. Then, slowly release the vacuum [3]. Stir the sample and apply the vacuum for a further 15 minutes [4].

    - Talent placing the plate in the vacuum chamber filled with ice.

    - Talent adjusting the vacuum settings.

    - Talent slowly releasing the vacuum from the chamber.

    - Talent stirring the sample in the plate and placing the plate back gain in the chamber.

  - Under the fume hood, remove the fixation solution with a pipette [1]. Wash each well 3 times for 10 minutes each under gentle agitation using 5 milliliters of ice-cold PBS  [2].

    - Talent removing the fixation solution from each well with a pipette under the fume hood.

    - Talent placing the plate added with PBS under gentle agitation.

  - After the washes, remove the PBS [1]. Using forceps, transfer the fixed tissue to a 120 by 120 millimeter Petri dish [2] and add 50 microliters of nuclei isolation buffer to the fixed seedlings [3].

    - Talent removing PBS  from the fixed tissue.

    - Talent transferring the fixed tissue to a 120 millimeter by 120 millimeter Petri dish with forceps.

    - Talent adding nuclei isolation buffer to the fixed seedlings.

- Isolation of Nuclei from Fixed Seedlings

  - Then, with a blade, chip the fixed tissue [1]. Using the same blade, scrape the chopped tissue to one edge of the Petri dish [2]. Add 600 microliters of nuclei isolation buffer and wash off any tissue stuck to the blade [3].

    - Talent chipping the fixed tissue with a blade.

    - Talent scraping the chopped tissue to one edge of the Petri dish with a blade.

    - Talent adding nuclei isolation buffer to the dish while rinsing tissue from the blade.

  - Transfer the tissue slurry into a 2 milliliter Dounce homogenizer tube [1] and stroke 5 to 10 times with the smaller pestle [2]. Then, stroke with the larger pestle until it moves smoothly inside the tube, typically requiring 5 to 10 strokes [3].

    - Talent transferring the tissue slurry into a 2 milliliter Dounce homogenizer tube.

    - Talent homogenizing the sample with the smaller pestle.

    - Talent homogenizing the sample with the larger pestle.

  - Now, filter the homogenized sample through a 30-micrometer mesh filter into a 1.5-milliliter microtube [1].

    - Talent filtering the homogenized sample through a mesh filter into a microtube.

  - Centrifuge the suspension for 10 minutes at 800 g using the lowest ramp for both acceleration and deceleration [1]. After centrifugation, note a small white pellet is visible among the green tissue debris [2].

    - Talent placing the microtube into the centrifuge.

    - Shot of a small white pellet visible among green tissue debris in the tube. Authors: If possible, please prepare the end product of this sample

  - Discard the supernatant [1] and resuspend the pellet in 150 microliters of nuclei isolation buffer [2].

    - Talent removing and discarding the supernatant from the microtube.

    - Talent resuspending the pellet in nuclei isolation buffer by pipetting up and down.

  - Centrifuge again for 10 minutes at 800 g and remove the supernatant [1].

    - Talent placing the microtube into the centrifuge.

  - After completely washing off the green debris, store the isolated nuclei suspension at 4 degrees Celsius for a maximum of 1 week [1] and immuno-stain the nuclei with desired antibodies [2].

    - Talent placing the microtube containing nuclei suspension into cold storage.

    - Shot of the adding the antibody mix to the sample. Authors: This is a representative shot for immunostaining, we do not have to show the entire procedure

- Sample Mounting in STORM Buffer and Agarose

  - Heat the 1.5% weight by volume low-melting agarose solution at 90 degrees Celsius for 1 minute to dissolve the powder and obtain a clear solution [1]. Cool the solution and maintain it at 37 degrees Celsius before proceeding to avoid heat denaturation of the glucose oxidase buffer [2].

    - Talent placing the solution into a heating device.

    - Talent placing the solution at 37 degrees Celsius in an incubator.

  - After the low-melting agarose solution reaches 37 degrees Celsius, add 100 millimolar mercaptoethylamine and glucose oxidase to achieve a 1x final concentration and prepare the STORM (storm) buffer [1-TXT] and mix the solution gently [2].

    - Talent adding mercaptoethylamine and glucose oxidase to the warm agarose solution. TXT: STORM: Stochastic Optical Reconstruction Microscopy

    - Talent gently mixing the solution.

  - Deposit 10 microliters of the mixture onto an ozone-treated 1.5-H coverslip that has been treated for 20 minutes in an ultraviolet ozone cleaning system [1]. Wait for 3 minutes to allow the nuclei to deposit on the coverslip [2].

    - Talent pipetting the sample onto a coverslip.

    - Shot of the sample resting on the coverslip during deposition.

  - Then, remove the excess liquid and wait for 2 minutes [1]. Add 10 microliters of the prepared low-melting agarose to the coverslip and immediately cover with a microscope slide [2].

    - Talent removing excess liquid from the coverslip.

    - Talent adding agarose and placing a microscope slide over the coverslip.

  - Seal the coverslip using silicone sealant [1]. Mix equal amounts of solution A and solution B in a small Petri dish using a micropipette tip [2]. Apply the mixture over the edges of the coverslip to seal the sample and allow it to harden for 5 minutes [3].

    - Talent applying silicone sealant around the coverslip.

    - Talent mixing solution A and solution B in a Petri dish using a micropipette tip.

    - Talent applying the sealant mixture along the edges of the coverslip.

- STORM Image Acquisition and Reconstruction

  - Mount the sample slide on the microscope and find focus [1]. Locate and focus on a nucleus [2] Activate a focus-maintaining module if available to ensure stability during long acquisitions [3].

    - Talent placing the sample slide onto the microscope stage and adjusting focus.

    - SCREEN: View of the sample as the nucleus is brought into focus.

    - Talent pointing to the focus-maintaining module.

# Authors: Please create screen capture videos of the shots labeled as SCREEN, write a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21621803

  - Select a region where one or a few nanodiamonds are positioned near the nucleus to improve drift correction during reconstruction [1].

    - SCREEN: View showing nanodiamonds positioned near the nucleus of interest. Authors: Please let us know after the shoot if this could be filmed

  - For two-color imaging, begin acquisition with the longer wavelength channel [1] and then switch to the shorter wavelength channel to complete imaging [2].

    - SCREEN: Show acquisition settings selecting the longer wavelength channel.

    - SCREEN: Show switching to the shorter wavelength channel in the acquisition settings.

  - Process the raw image stacks in Thunder-STORM using Gaussian fitting with Weighted Least-Squares estimation and generate reconstructed images [1]. Apply filtering based on sigma and uncertainty below 30 nanometers to retain accurate localizations and remove noise [2].

    - SCREEN: Show raw image stacks being loaded into ThunderSTORM.

    - SCREEN: Apply sigma and uncertainty filtering below 30 nanometers in ThunderSTORM. Authors: We need only 20-second clips for each shot labelled SCREEN.

As we are already at the 55-shot limit, this section has been summarised into 2 steps

  - Finally, perform sample drift correction using ThunderSTORM cross-correlation or the COMET (comet) algorithm [1] and confirm minimal drift in the reconstructed images [2].

    - SCREEN: Run algorithm for  drift correction using ThunderSTORM cross-correlation or the COMET algorithm.

    - SCREEN: Show reconstructed images with minimal drift.

# Results

- Results

  - Nuclei were isolated from 15-day-old wild-type seedlings, immunolabeled with AF647-conjugated secondary antibodies, and counterstained with Hoechst H33258 (H-3-3-2-5-8) [1].

    - LAB MEDIA: Figure 1

  - Confocal imaging showed multiple isolated and well-preserved nuclei with minimal debris [1], with chromocenters and the nucleolus clearly distinguishable [2].

    - LAB MEDIA: Figure 1. Video editor: Highlight Hoechst column images

    - LAB MEDIA: Figure 1. Video editor: Zoom in on the top image in Hoechst and AF647 columns and and point to the regions indicated by arrows

  - RNA Polymerase II (2) signal was absent from chromocenters and the nucleolus [1].

    - LAB MEDIA: Figure 1. Video editor: Highlight the areas marked by arrows where the red signal is missing

  - Diffraction-limited maximum intensity projections and corresponding SMLM reconstructions revealed structural details of RNA Polymerase II and DNA labeling [1], including enlarged regions of interest showing finer structures [2].

    - LAB MEDIA: Figure 2. Video editor: Highlight the AF647 and JF549 columns images

    - LAB MEDIA: Figure 2. Video editor: Highlight the boxed inset regions after each image

  - Localization filtering based on sigma and uncertainty improved the definition of both RNA Polymerase II and DNA signals compared to detected localizations [1], and further refinement after drift correction maintained this improved structure [2].

    - LAB MEDIA: Figure 3A. Video editor: Show the top row labeled “detected localizations”

    - LAB MEDIA: Figure 3A. Video editor: Move to the middle and bottom rows and

  - Filtering based on sigma values removed localizations with excessively small or large fitted widths [1].

    - LAB MEDIA: Figure 3B.

  - Fourier Ring Correlation analysis showed that resolution in the AF647 channel improved from 90.1 to 47.7 nanometers after filtering [1], while the JF549 channel improved from 56.6 to 27.9 nanometers [2].

    - LAB MEDIA: Figure 4. Video editor: Highlight the resolution value labeled 90.1 nanometer in panel A and then 47.7 nanometer in panel C for the left column

    - LAB MEDIA: Figure 4. Video editor: Highlight the resolution value labeled 56.6 nanometer in panel A and then 27.9 nanometer in panel C for the right column

NOTE to the Authors:

For submitting the SCREEN footage:

Upload Separate Clips (Highly Recommended)

Please upload a separate video file for each SCREEN shot (tagged as SCREEN), naming the file according to the shot number (e.g., 2.4.3.mp4, 1.5.1, 3.2.1, 3.1.3, etc.).

- Each clip must be a maximum of 20 to 25 seconds to match the voice narration.

- If an action takes longer, provide only the best, most representative 20-25 second segment.

- Please do not provide longer clips as we won’t be able to use them.