# Submission ID #: 70979

# Scriptwriter Name: Praveen Kumar

# Project Page Link: https://review.jove.com/files_upload.php?src=21648643

# Title: Sex Hormone Detection in Human Tear Film using Enzyme-linked Immunosorbent Assays

# Authors and Affiliations:

Nutchaarpa Puangmalai1,2, Anna A. Tichenor1

1School of Optometry, Indiana University

# 2Faculty of Optometry, Ramkhamhaeng University

# Corresponding Authors:

Anna A. Tichenor               annatich@iu.edu

# Email Addresses for All Authors:

# sapuang@iu.edu

# annatich@iu.edu

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps: 14

Number of Shots: 35

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Nutchaarpa Puangmalai: Our research focuses on women’s health in ocular surface diseases and adaptation of ELISA for sex hormone quantification in basal tear fluid.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Nutchaarpa Puangmalai: Traditional ELISAs require large sample volumes, making tear fluid analysis challenging. This protocol enables hormone or protein quantification from limited tear samples.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Anna Tichenor: This protocol allows researchers to measure sex hormone concentration by ELISA in basal tear fluid collected by using micro-capillary tubes.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Anna Tichenor: The tear film is easily accessible for ocular surface research, but limited tear volume often requires sample collection across multiple visits.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Anna Tichenor: Future studies can use this protocol to investigate how tear hormone levels correlate with diseases or monitor treatment effectiveness.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Ethics Title Card

This research has been approved by the Institutional Review Board (IRB) at Indiana University

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Tear Film Collection and Storage

Demonstrator: Anna Tichenor

  - Begin by obtaining appropriate microcapillary tubes to collect tears and a 1.5-milliliter microcentrifuge tube for tear film storage [1].

    - WIDE: Talent walking into the room with microcapillary tubes placed in a rack.  Videographer: Please obtain CU shot as well for this shot.

  - Take the rubber squeeze bulb from the microcapillary tube kit to facilitate transfer of tear fluid into a microcentrifuge tube [1].

    - Talent taking the rubber squeeze bulb out from the box for tear fluid collection.  NOTE: Not filmed

  - To collect the basal tears, instruct the participant to look straight ahead or slightly upward [1]. Then, gently place the microcapillary tube opening in the tear meniscus near the lateral canthus, avoiding lid and conjunctival contact to minimize reflex tearing [2]. Fill the tube with tears until it’s full [3].

    - Talent is instructing the participant to look straight ahead or slightly upward.

    - Talent placing the microcapillary tube opening in the tear meniscus near the lateral canthus.

    - Close-up of the tube being filled with tears until it's full.

  - Occlude the hole on the rubber squeeze bulb [1], insert the microcapillary tube into the bulb [2], and place the open end into the microcentrifuge tube [3]. Gently squeeze the bulb to release the tear into the microcentrifuge tube [4]. Close the microcentrifuge tube cap to prevent evaporation [5].

    - Talent blocking the hole of the rubber squeeze bulb with a finger. NOTE: 2.4.1 to 2.4.5 are combined in a single shot

    - Talent inserting the microcapillary tube in the rubber squeeze bulb.

    - Talent placing open end of microcapillary tube in the 1.5 mL microcentrifuge tube.

    - Talent squeezing the bulb and tear is getting collected in 1.5 mL microcentrifuge tube.

    - Talent closing the cap of the microcentrifuge tube.

  - Spin the tube at 2,000 g for 3 to 5 seconds to collect the sample at the bottom [1]. Then, store the sample at minus 80 degrees Celsius for preserving analyte stability prior to analysis [2].

    - Talent placing the tubes in a benchtop microcentrifuge device and closing the lid. Videographer: If possible, film the setting/parameters of the device.

    - Talent placing the samples at -80 °C.

- Estradiol Assay

Demonstrator: Nutchaarpa Puangmalai

  - Obtain all required reagents and materials and let them reach room temperature [1].

    - WIDE: Talent placing required reagents on the workbench.

  - Aliquot only the required amounts of ready-to-use standards and controls to minimize contamination and repeated temperature changes [1].

    - Talent dispensing the reagent into a small 1.5 mL microcentrifuge tube.

  - Prepare six standard solutions labeled as number zero to five [1]. Include one control sample provided with the kit [2]. Keep the frozen samples on ice for thawing [3]. Spin down the tear samples [4] to ensure samples are at the bottom of the tube [5].

    - Talent preparing standard solution in a vial labeled as No.5 and other standards labeled vials labeled from 0 to 4 are in the background

    - Talent placing control sample on the workbench.

    - Talent placing samples on ice.

    - Talent placing the sample in spinner

    - Shot of samples at bottom after spinning

  - Mix all standards, controls, and samples prior to pipetting into the wells for homogeneous distribution [1]. Add 20 microliters of each standard, control, and sample into wells [2]. Add 160 microliters of 17 beta Estradiol-HRP (H-R-P) Conjugate to the testing wells [3].

    - Representative shot of talent pipetting a standard solution up and down.

    - Talent adding 20 μL of sample into wells.

    - Talent adding 160 μL of 17β-Estradiol-HRP Conjugate to the testing wells.

  - Then, cover the plate with aluminum foil or use the provided cover foil from the kit [1] and incubate it at thirty-seven degrees Celsius for two hours [2].

    - Talent covering plate with aluminum foil.

    - Talent placing the covered plate in the incubator

  - In the meantime, prepare wash buffer from 10X (ten-x) washing solution [1].

    - Talent adding deionized water to a container with 10x washing solution

  - After incubation, remove the aluminum foil from plate [1] and aspirate the liquid from the wells [2]. Wash each well three times with the wash buffer for at least 5 seconds each [3].  Add 100 microliters of TMB (T-M-B) substrate solution into all wells [4]. Cover the whole plate with aluminum foil and incubate at room temperature for thirty minutes [5].

    - Talent removing the aluminum foil.

    - Talent taking the liquid out from the wells.

    - Talent adding wash buffer to wells

    - Talent adding 100 μL of TMB Substrate Solution into wells.

    - Talent covering wells with aluminum foil and keeping aside on the workbench.

  - Post incubation, add one hundred microliters of Stop Solution into the wells [1] and observe the reagent color change from blue to yellow [2]. Then, shake the microplate gently [3]. NOTE: The VO is edited for the rearranged shots

    - Talent adding 100 μL of Stop Solution into all wells. NOTE: 3.8.1. and 3.8.3 are filmed in a single shot

3.8.3      Shot of color changing from blue to yellow in plate  NOTE: This shot is moved  here as per the author's request

    - Talent shaking the plate and placing back on workbench.

  - Measure the absorbance at 450 nanometers within 30 minutes of adding the Stop Solution [1]. Then, calculate the concentration of 17 beta Estradiol using appropriate tools [2].

    - Talent placing the plate in the spectrophotometer and closing the lid.

    - Representative shot of talent at the computer doing statistical analysis or generating a graph. NOTE: able to show a quick demo when getting results/ measurement from spectrophotometer rather than showing the graph

# Results

- Results

  - In the present study, twenty women aged between forty-three and sixty-six years with and without dry eye disease or DED (D-E-D) were included [1]. The average tear film 17 beta Estradiol concentrations in older women without DED were approximately 599.3 (five hundred-ninety-nine-point-three) picograms per milliliter [2] and 423.6 (four hundred-twenty-three-point-six) picograms per milliliter in older women with DED [3].

    - LAB MEDIA: Table 2 Video Editor: Please do not show the column called “Testosterone Concentrations (ng/mL)” as it is not included in the protocol.

    - LAB MEDIA: Table 2 Video editor: Please emphasize the row named “non-DED” and then the value 599.30 in it.

    - LAB MEDIA: Table 2 Video editor: Please emphasize the row named “DED” and then the value 423.60 in it.

  - The percent coefficient of variation for estradiol measurements was 1.0% for non-DED samples [1] and 6.65% for DED samples, confirming assay reproducibility [2].

    - LAB MEDIA: Table 2 Video Editor: Please emphasize the row named “non-DED” and then the value 1.00 in it.

    - LAB MEDIA: Table 2 Video Editor: Please emphasize the row named “DED” and then the value 6.65 in it.

- Microcapillary
Pronunciation link: https://www.merriam-webster.com/dictionary/microcapillary
IPA: /ˌmaɪ-kroʊ-ˈkæp-ə-ˌler-i/
Phonetic Spelling: my·kroh·kap·uh·lair·ee

- Microcentrifuge
Pronunciation link: https://www.merriam-webster.com/dictionary/microcentrifuge
IPA: /ˌmaɪ-kroʊ-ˈsen-trə-ˌfyüj/
Phonetic Spelling: my·kroh·sen·truh·fyooj

- Meniscus
Pronunciation link: https://www.merriam-webster.com/dictionary/meniscus
IPA: /məˈnɪskəs/
Phonetic Spelling: muh·nis·kuhs

- Lateral
Pronunciation link: https://www.merriam-webster.com/dictionary/lateral
IPA: /ˈlæt-rəl/, /ˈla-tə-rəl/
Phonetic Spelling: lat·ruhl; la·tuh·ruhl

- Canthus
Pronunciation link: https://www.merriam-webster.com/dictionary/canthus
IPA: /ˈkan-thəs/
Phonetic Spelling: kan·thuhs

- Conjunctival
Pronunciation link: https://www.merriam-webster.com/dictionary/conjunctiva
IPA: /kənˌdʒʌŋ(k)ˈtaɪ-vəl/
Phonetic Spelling: kuhn·jungk·tye·vuhl

- Occlude
Pronunciation link: https://www.merriam-webster.com/dictionary/occlude
IPA: /əˈkluːd/
Phonetic Spelling: uh·klood

- Centrifuge
Pronunciation link: https://www.merriam-webster.com/dictionary/centrifuge
IPA: /ˈsen-trə-ˌfyüj/
Phonetic Spelling: sen·truh·fyooj

- Analyte
Pronunciation link: https://www.merriam-webster.com/dictionary/analyte
IPA: /ˈænəˌlaɪt/
Phonetic Spelling: an·uh·lyte

- Estradiol
Pronunciation link: https://www.merriam-webster.com/dictionary/estradiol
IPA: /ˌes-trə-ˈdī-ˌȯl/
Phonetic Spelling: es·truh·dye·awl

- Aliquot
Pronunciation link: https://www.merriam-webster.com/dictionary/aliquot
IPA: /ˈa-lə-ˌkwät/
Phonetic Spelling: al·uh·kwaht

- Reagent
Pronunciation link: https://www.merriam-webster.com/dictionary/reagent
IPA: /rē-ˈā-jənt/
Phonetic Spelling: ree·ay·juhnt

- Pipetting
Pronunciation link: https://www.merriam-webster.com/dictionary/pipette
IPA: /pī-ˈpet/
Phonetic Spelling: pie·pet·ing

- Conjugate
Pronunciation link: https://www.merriam-webster.com/dictionary/conjugate
IPA: /ˈkän-jə-gət/, /ˈkän-jə-ˌgāt/
Phonetic Spelling: kon·juh·guht; kon·juh·gate

- Incubate
Pronunciation link: https://www.merriam-webster.com/dictionary/incubate
IPA: /ˈɪŋ-kyə-ˌbāt/
Phonetic Spelling: ing·kyuh·bate

- Deionized
Pronunciation link: https://www.merriam-webster.com/dictionary/deionize
IPA: /dē-ˈī-ə-ˌnīz/
Phonetic Spelling: dee·eye·uh·nyzd

- Aspirate
Pronunciation link: https://www.merriam-webster.com/dictionary/aspirate
IPA: /ˈas-pə-ˌrāt/
Phonetic Spelling: as·puh·rate

- Substrate
Pronunciation link: https://www.merriam-webster.com/dictionary/substrate
IPA: /ˈsəb-ˌstrāt/
Phonetic Spelling: sub·strate

- Microplate
Pronunciation link: https://www.merriam-webster.com/dictionary/microplate
IPA: /ˈmaɪ-kroʊ-ˌplāt/
Phonetic Spelling: my·kroh·plate

- Absorbance
Pronunciation link: https://www.merriam-webster.com/dictionary/absorbance
IPA: /əbˈzɔːr.bəns/
Phonetic Spelling: ub·zor·buhns

- Nanometer
Pronunciation link: https://www.merriam-webster.com/dictionary/nanometer
IPA: /ˈna-nō-ˌmē-tər/
Phonetic Spelling: nan·oh·mee·tur

- Spectrophotometer
Pronunciation link: https://www.merriam-webster.com/dictionary/spectrophotometer
IPA: /ˌspek-trō-fə-ˈtä-mə-tər/
Phonetic Spelling: spek·troh·fuh·tom·uh·tur

- Coefficient
Pronunciation link: https://www.merriam-webster.com/dictionary/coefficient
IPA: /ˌkō-ə-ˈfi-shənt/
Phonetic Spelling: koh·uh·fish·uhnt

- Reproducibility
Pronunciation link: https://www.merriam-webster.com/dictionary/reproducibility
IPA: /ˌrē-prə-ˌdü-sə-ˈbi-lə-tē/
Phonetic Spelling: ree·pruh·doo·suh·bil·uh·tee

- Picograms
Pronunciation link: https://www.merriam-webster.com/dictionary/picogram
IPA: /ˈpī-kə-ˌgram/
Phonetic Spelling: pie·kuh·gram

- Testosterone
Pronunciation link: https://www.merriam-webster.com/dictionary/testosterone
IPA: /tes-ˈtäs-tə-ˌrōn/
Phonetic Spelling: tes·tahs·tuh·rohn

- Evaporation
Pronunciation link: https://www.merriam-webster.com/dictionary/evaporation
IPA: /i-ˌva-pə-ˈrā-shən/
Phonetic Spelling: ee·vap·uh·ray·shuhn

- Stability
Pronunciation link: https://www.merriam-webster.com/dictionary/stability
IPA: /stə-ˈbi-lə-tē/
Phonetic Spelling: stuh·bil·uh·tee

- Concentration
Pronunciation link: https://www.merriam-webster.com/dictionary/concentration
IPA: /ˌkän-sən-ˈtrā-shən/
Phonetic Spelling: kon·suhn·tray·shuhn

- Thawing
Pronunciation link: https://www.merriam-webster.com/dictionary/thaw
IPA: /ˈθɔːɪŋ/
Phonetic Spelling: thaw·ing