# Submission ID #: 71048

# Scriptwriter Name: Sri Lakshmi Priya

# Project Page Link: https://review.jove.com/account/file-uploader?src=21669688

# Title: Characterizing Riboglow Probes In Vitro as the Basis for Fluorescence Lifetime Imaging In Live Mammalian Cells and Three-Dimensional Cellular Models

# Authors and Affiliations:

Gareth M. Francis*, Luke K. Shafik*, Olayinka Barnes*, Kosonike Barnes*, Giulia Chitu*, Sydney Darensburg*, Isabel Early*, Jordan Elasfari*, Elana Elder*, Hayden Giles*, Nathan Ho*, Manya Reddy*, Andrew Shen*, Adelle Snider*, Erin N.L. Tran*, Esther Braselmann

Department of Chemistry, Georgetown University

* These authors contributed equally

# Corresponding Authors:

Esther Braselmann 		esther.braselmann@georgetown.edu

# Email Addresses for All Authors:

Gareth M. Francis		gf373@georgetown.edu

Luke K. Shafik			lks60@georgetown.edu

Olayinka Barnes		odb7@georgetown.edu

Kosonike Barnes		kdb109@georgetown.edu

Giulia Chitu			gvc16@georgetown.edu

Sydney Darensburg		srd93@georgetown.edu

Isabel Early			ise8@georgetown.edu

Jordan Elasfari			jse44@georgetown.edu

Elana Elder			eae58@georgetown.edu

Hayden Giles			heg34@georgetown.edu

Nathan Ho			ndh29@georgetown.edu

Manya Reddy			mmr181@georgetown.edu

Andrew Shen 			als433@georgetown.edu

Adelle Snider			ars381@georgetown.edu

Erin N.L. Tran			ent19@georgetown.edu

Esther Braselmann 		esther.braselmann@georgetown.edu

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

If Yes, can you record movies/images using your own microscope camera?

N/A

If your protocol involves microscopy but you are not able to record movies/images with your microscope camera, JoVE will need to use our scope kit.

If your microscope does not have a camera port, the scope kit will be attached to one of the eyepieces and you will have to perform the procedure using one eye.

Nikon Ti2 Eclipse

If a dissection or stereo microscope is required for your protocol, please list all shots from the script that will be visualized using the microscope (shots are indicated with the 3-digit numbers, like 2.1.1, 2.1.2, etc.).

Scope shots: 3.7.2, 3.9.1, 3.9.2

Videographer: Please film above mentioned shots using a scope kit.

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done!

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. Yes

If Yes, please provide the full name and position (e.g., Director of [Institute Name], Senior Researcher [University Name], etc.) of the author willing to participate.

Luke Shafik (Graduate Student Researcher, Georgetown University), Esther Braselmann (Assistant Professor, Department of Chemistry, Georgetown University).

Current Protocol Length

Number of Steps:  24

Number of Shots:  55

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Answers are slightly edited in accordance with journal guidelines. Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Gareth M. Francis: Our research focuses on visualizing RNA in live cells to investigate the spatiotemporal dynamics of RNA.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research contexts or systems in which this protocol can be applied.

  - Gareth M. Francis: This Riboglow platform uses quantitative fluorescence lifetime imaging microscopy in multicellular systems applicable to biological systems in health and disease.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Luke Shafik: This protocol allows researchers to determine the fluorescence lifetime of fluorescent molecules in different biological contexts.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Are there any additional methods or analyses that can be performed following this procedure?

  - Luke Shafik:  Researchers can perform quantitative data analysis and assess variables of interest following this procedure, including the addition of RNA ligands.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Gareth M. Francis: Future studies can apply Riboglow-FLIM to disease models, expand multiplexing capabilities, and track RNA dynamics in live animal systems.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# `Testimonial Questions (OPTIONAL):

# Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

- Testimonial statements will not appear in the video but may be featured in the journal’s promotional materials.

- Provide the full name and position (e.g., Director of [Institute Name], Senior Researcher [University Name], etc.) of the author delivering the testimonial.

- Please answer the testimonial question live during the shoot, speaking naturally and in your own words in complete sentences.

What value do you expect this video article to have for other researchers working in your field?

  - Enter author name, Enter author title: (authors will present their testimonial statements live)

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

How does the video format of your article help you to share your work with the broader scientific community?

  - Enter author name, Enter author title: (authors will present their testimonial statements live)

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

NOTE: Timestamps for SCREEN shots were added at the postshoot stage. Please contact post shoot integrator (Sri Lakshmi Priya) for any discrepancies

- FLIM Imaging System Setup and Initialization

Demonstrator: Luke K. Shafik

  - To begin, obtain the RNA (R-N-A) samples and bead-loaded live cells with Riboglow probes [1]. Switch on the confocal laser scanning microscope system [2], followed by the microscope control unit [3]. Power on the time-correlated single photon counting based fluorescence lifetime imaging microscopy or FLIM (flim) system, including the laser combining unit, the photomultiplier detector, and the event timer [4].

Videographer’s NOTE: The first wide take, talent did a bit at the end of miminig working at the computer console, figured it would be good for b-roll if needed. Also did some closer options of turning on the pieces of equipment.

    - WIDE: Talent walking into the room with a tube rack containing RNA samples and bead-loaded live cells. Videographer: Please obtain CU shot as well for this shot.

    - Talent powering on the confocal laser scanning microscope.

    - Talent powering on the microscope control unit.

    - Talent powering on the TCSPC based FLIM system components.

  - After opening the image acquisition and analysis computer software, under the boot-up options, select AX with PicoQuant (A-X) [1].

    - SCREEN: 71048_2.2.1-and-2.3.1_t1.mp4: 0:08-0:16

  - Once the software has booted up, select AX FLIM and FCS (F-C-S) [1].

    - SCREEN: 71048_2.2.1-and-2.3.1_t1.mp4:  1:09 – 1:29

  - To set up the microscope for fluorescence lifetime data acquisition, confirm that Galvano is selected in the AX pad section [1]. Set the averaging value to 1 [2] and dwell time to 2 [3].

    - SCREEN:71048_2.4,-2.5,-and-2.6_t1.mp4: 0:01-0:08

    - SCREEN:71048_2.4,-2.5,-and-2.6_t1.mp4: 0:09-0:15

    - SCREEN:71048_2.4,-2.5,-and-2.6_t1.mp4: 0:15-0:23

  - In the laser power section, lower the individual laser sliders to 3 to 4 percent [1]. Set the All Lasers slider to zero [2].

    - SCREEN:71048_2.4,-2.5,-and-2.6_t1.mp4: 0:22-0:33

    - SCREEN:71048_2.4,-2.5,-and-2.6_t1.mp4: 0:33-0:40

  - On the TI2 Pad (T-twelve), after selecting the 20 times objective, in the scan area settings, select 512 by 512 [1] and turn on the detectors [2].

    - SCREEN:71048_2.4,-2.5,-and-2.6_t1.mp4: 0:41-0:48

    - SCREEN:71048_2.4,-2.5,-and-2.6_t1.mp4: 0:49-0:57

  - To define the data file path, go to the bottom of the controller section and select Configure [1]. After clicking the PicoQuant or FLIM FCS tab, under Workspace path, select the folder symbol on the right [2]. At storage location, create a new folder for the experiments for the day and close the configure window [3].

    - SCREEN: 71048_2.7_t1.mp4: 0:03-0:06

    - SCREEN: 71048_2.7_t1.mp4: 0:07-0:11

    - SCREEN: 71048_2.7_t1.mp4: 0:12-0:18

  - Once Live is selected and the image acquisition window appears, select Live again to pause photon collection [1].

    - SCREEN: 71048_2.8-and-2.9_t1.mp4: 0:02-0:08

  - To set up fluorescence lifetime imaging microscopy or FLIM data collection windows, right-click on the live image window and select PicoQuant Plugin [1]. Enable Show Decay Curve [2], Show Time trace [3] and Show FLIM Statistics [4].

    - SCREEN: 71048_2.8-and-2.9_t1.mp4: 0:09-0:14

    - SCREEN: 71048_2.8-and-2.9_t1.mp4: 0:15-0:16

    - SCREEN: 71048_2.8-and-2.9_t1.mp4: 0:17-0:28

    - SCREEN: 71048_2.8-and-2.9_t1.mp4: 0:29-0:41

- 2D Cellular FLIM Data Collection and Acquisition Setup

Demonstrator: Gareth M. Francis

  - To set up laser arrangement for the cellular experiment, go to Experiment Settings at the top right of the page next to the lasers [1]. After clicking the Add next to the multichannel dye box, add the desired dyes [2].

    - SCREEN: 71048_3.1,-3.2,-and-3.3_t1.mp4: 0:01-0:07

    - SCREEN: 71048_3.1,-3.2,-and-3.3_t1.mp4: 0:08-0:029

  - Select Expert mode [1]. Adjust the detector assignments for each emission spectrum [2]. Route overlapping excitation and emission spectra to different detectors [3].

    - SCREEN: 71048_3.1,-3.2,-and-3.3_t1.mp4: 0:29-0:34

    - SCREEN: 71048_3.1,-3.2,-and-3.3_t1.mp4: 0:34-0:39

    - SCREEN: 71048_3.1,-3.2,-and-3.3_t1.mp4: 0:40 – 0:57

  - On the left panel, adjust the pass order to minimize crosstalk between dyes with overlapping excitation and emission spectra [1]. After saving and naming the new laser configuration, select the saved configuration for the experiment [2].

    - SCREEN: 71048_3.1,-3.2,-and-3.3_t1.mp4: 0:58-1:07

    - SCREEN: 71048_3.1,-3.2,-and-3.3_t1.mp4: 1:08-1:20

  - To open the XYZ (X-Y-Z) Overview window, select View, choose Acquisition Controls, and then select XYZ Overview [1].

    - SCREEN: 71048_3.4_t1.mp4: 0:02-0:14

  - Add live-cell nuclear stain to the cells [1].

    - Talent adding live-cell nuclear stain to the cells.

  - After 30 seconds, place the imaging dish onto the microscope stage [1]. Push the dish toward the bottom left corner of the microscope stage [2]. Tape the sample to the stage when using non-air objectives to prevent xy drift [3].

    - Talent placing the imaging dish onto the microscope stage.

    - Talent positioning the dish at the bottom left of the stage.

    - Talent taping the imaging dish to the microscope stage.

  - Switch to the 4′ 6 diamidino 2 phenylindole or DAPI (Dapi) channel [1]. Using the knob on the side of the controller, slowly raise the Z height until the cell nuclei come into focus [2]. Adjust the microscope controller carrots to modify the sensitivity of the Z height knob [3] and if cells do not appear in view, adjust the XY coordinates [4].

    - SCREEN: 71048_3.7_t1.mp4: 0:02-0:13

    - SCOPE: Show the cell nuclei come into focus.

    - Talent adjusting the microscope controller carrots.

    - SCREEN: 71048_3.7-and-3.8_t1.mp4: 0:20-0:24

  - On the microscope control panel, after enabling the Perfect Focus System or PFS (P-F-S) automatic focus, on the right side of the AX (A-X) panel, select the magnifying glass next to the PFS button [1]. Once the microscope has automatically normalized the Z height, confirm that the PFS button illuminates and displays in focus [2]. NOTE: The VO is adjusted as per SC.

    - SCREEN: Show the cursor selecting PFS automatic focus On button.

    - SCREEN: 71048_3.7-and-3.8_t1.mp4: 0:24-0:28. NOTE: This shot number is changed from 3.8.2 to 3.8.1

    - SCREEN: 71048_3.7-and-3.8_t1.mp4: 0:29-0:34 NOTE: This shot number is changed from 3.8.3 to 3.8.2

  - After searching the cells using AX detector, find the areas of the cells using the DAPI channel [1]. Switch to the ATTO 590 (AT-TOH-five ninety) channel to effectively identify bead-loaded cells [2].

    - SCOPE: Show cells visualized in the DAPI channel.

    - SCOPE: Show bead-loaded cells visualized in the ATTO 590 channel.

SCREEN: 71048_3.9,-3.10,-3.11,-and-3.12_t1.mp4: 0:03-0:08 NOTE: This scope shot is changed to SC.

  - Once a cell is identified and centered by double clicking, switch to AX FLIM and FCS [1].

    - SCREEN: 71048_3.9,-3.10,-3.11,-and-3.12_t1.mp4: 0:09-0:18

  - Ensuring that 512 by 512 is selected, adjust the frame size by clicking and dragging the corners to fit the cell [1]. Allow the resized frame to turn red [2]. Right-click inside the frame or select Live until the frame turns green to confirm the adjustment [3].

    - SCREEN: 71048_3.9,-3.10,-3.11,-and-3.12_t1.mp4: 0:19-0:22

    - SCREEN: 71048_3.9,-3.10,-3.11,-and-3.12_t1.mp4: 0:23-0:24

    - SCREEN: 71048_3.9,-3.10,-3.11,-and-3.12_t1.mp4: 0:25-0:27

  - After adjusting the number of captures to the desired value, select Capture [1]. Observe the new capture window and the capture progress box on the right side of the screen [2].

    - SCREEN: 71048_3.9,-3.10,-3.11,-and-3.12_t1.mp4: 0:28-0:30

    - SCREEN: 71048_3.9,-3.10,-3.11,-and-3.12_t1.mp4: 0:31-0:40

  - Once the FLIM analysis software is opened, navigate to the correct file path [1]. Observe the acquisitions automatically populating the analysis software window [2]. Rename the acquisitions as they are collected [3].

    - SCREEN: 71048_3.13_t1.mp4: 0:01-0:13

    - SCREEN: 71048_3.13_t1.mp4: 0:14-0:32

    - SCREEN: 71048_3.13_t1.mp4: 0:33-0:48

  - When the capture is complete, switch back to the AX detector [1]. Select Live to search for a new cell [2]. Adjust the size of the AX FLIM and fluorescence correlation spectroscopy or FCS (F-C-S) window as needed for larger or smaller cells [3].

    - SCREEN: 71048_3.14_t1.mp4: 0:02-0:15

    - SCREEN: 71048_3.14_t1.mp4: 0:16-0:18

    - SCREEN: 71048_3.14_t2.mp4: 0:41-0:48

  - After acquiring new cells during the 3-hour imaging period, export the created .sptw (dot-S-P-T-W) file [1].

    - SCREEN: 71048_3.15_t1.mp4: 0:09-0:28

  - Proceed with FLIM data processing and analysis to extract fluorescence lifetime measurements from the collected samples [1].

    - SCREEN: 71048_3.16_t1.mp4: 0:20-0:23, 0:32-0:50

# Results

- Results

  - In vitro fluorescence lifetime measurements demonstrated that Riboglow probes Cbl-3xGly-ATTO590 or 3xGly (three-X-gly) and Cbl-5xGly-ATTO590 or 5xGly (Five-X-gly) were identical except for the number of glycine repeats in their linker segments [1], with 3 repeats in 3xGly [2] and 5 repeats in 5xGly [3].

    - LAB MEDIA: Figure 3D.

    - LAB MEDIA: Figure 3D. Video editor: Emphasize the pink structure labelled as 3xGly.

    - LAB MEDIA: Figure 3D. Video editor: Emphasize the blue structure labelled as 5xGly.

  - Representative FLIM acquisition windows with normalized false color scales [1] aligned with the extracted τAvAmp (Tau-average-amplitude) values for each sample type [2].

    - LAB MEDIA: Figure 2A.

    - LAB MEDIA: Figure 2B.

  - Representative decay curves showed that the presence of the RNA A (R-N-A-A) Tag increased the extracted fluorescence lifetime in vitro [1], while samples without the RNA A Tag showed left-shifted decay curves indicating shorter average lifetime values [2].

    - LAB MEDIA: Figure 2C. Video editor: Emphasize the pink curve labeled as 3xGly + RNA and the green curve as labeled 5xGly + RNA.

    - LAB MEDIA: Figure 2C. Video editor: Emphasize the purple curve labelled as 3xGly and the light blue curve labelled as 5xGly.

  - Cellular fluorescence lifetimes of cells bead-loaded with 3xGly [1] were significantly lower than those loaded with 5xGly [2].

    - LAB MEDIA: Figure 3D. Video editor: Emphasize the pink dots labelled as 3xGly.

    - LAB MEDIA: Figure 3D. Video editor: Emphasize the blue dots labelled as 5xGly.

  - Representative decay curves showed that cells loaded with 3xGly had left-shifted decay curves [1] compared to cells loaded with 5xGly, indicating lower average fluorescence lifetime values [2].

    - LAB MEDIA: Figure 3C. Video editor: Emphasize the pink curve labelled as 3xGly.

    - LAB MEDIA: Figure 3C. Video editor: Emphasize the blue curve labelled as 5xGly.

  - The nucleus of cells loaded with Riboglow probe consistently showed higher fluorescence lifetimes [1] than the cytosol of the same cell [2].

    - LAB MEDIA: Figure 5F. Video editor: Emphasize the light blue dots and light pink dots along with the horizontal line labelled as nucleus in 3xGly and 5xGly.

    - LAB MEDIA: Figure 5F. Video editor: Emphasize the dark blue and dark pink dots along with the horizontal line labelled as cytosol in 3xGly and 5xGly.

  - The false color scale acquisition window [1], decay curves [2], and extracted τAvAmp values all showed the same fluorescence lifetime trend [3].

    - LAB MEDIA: Figure 5C. Video editor: Emphasize the bright area in the middle of the first image labelled as 5xGly.

    - LAB MEDIA: Figure 5D and 5E. Video editor: Emphasize the light blue curve in 5D and light pink curve in 5E.

    - LAB MEDIA: Figure 5F. Video editor: Highlight the light pink and light blue dots labeled as nucleus.

# Pronunciation guides:

- Riboglow
Pronunciation link: No confirmed link found
IPA: /ˈraɪboʊˌɡloʊ/
Phonetic Spelling: rye·boh·gloh

- Confocal
Pronunciation link: https://www.merriam-webster.com/dictionary/confocal
IPA: /kənˈfoʊkəl/
Phonetic Spelling: kuhn·foh·kuhl

- Fluorescence
Pronunciation link: https://www.merriam-webster.com/dictionary/fluorescence
IPA: /flʊˈresəns/
Phonetic Spelling: floor·eh·suhns

- Microscopy
Pronunciation link: https://www.merriam-webster.com/dictionary/microscopy
IPA: /maɪˈkrɑːskəpi/
Phonetic Spelling: my·krah·skuh·pee

- Photomultiplier
Pronunciation link: https://www.merriam-webster.com/dictionary/photomultiplier
IPA: /ˌfoʊtoʊˈmʌltəˌplaɪər/
Phonetic Spelling: foh·toh·muhl·tuh·ply·uhr

- Detector
Pronunciation link: https://www.merriam-webster.com/dictionary/detector
IPA: /dɪˈtektər/
Phonetic Spelling: dih·tehk·tuhr

- PicoQuant
Pronunciation link: No confirmed link found
IPA: /ˈpiːkoʊˌkwɑːnt/
Phonetic Spelling: pee·koh·kwahnt

- Fluorophore
Pronunciation link: https://www.merriam-webster.com/dictionary/fluorophore
IPA: /ˈflʊrəˌfɔːr/
Phonetic Spelling: floor·uh·for

- Lifetime
Pronunciation link: https://www.merriam-webster.com/dictionary/lifetime
IPA: /ˈlaɪfˌtaɪm/
Phonetic Spelling: lyfe·tyme

- Calibration
Pronunciation link: https://www.merriam-webster.com/dictionary/calibration
IPA: /ˌkælɪˈbreɪʃən/
Phonetic Spelling: kal·uh·bray·shuhn

- Emission
Pronunciation link: https://www.merriam-webster.com/dictionary/emission
IPA: /ɪˈmɪʃən/
Phonetic Spelling: ih·mish·uhn

- Excitation
Pronunciation link: https://www.merriam-webster.com/dictionary/excitation
IPA: /ˌeksaɪˈteɪʃən/
Phonetic Spelling: ehk·sy·tay·shuhn

- Photon
Pronunciation link: https://www.merriam-webster.com/dictionary/photon
IPA: /ˈfoʊˌtɑːn/
Phonetic Spelling: foh·tahn

- Cytoplasm
Pronunciation link: https://www.merriam-webster.com/dictionary/cytoplasm
IPA: /ˈsaɪtəˌplæzəm/
Phonetic Spelling: sy·tuh·plaz·uhm

- Autofluorescence
Pronunciation link: https://www.merriam-webster.com/dictionary/autofluorescence
IPA: /ˌɔːtoʊflʊˈresəns/
Phonetic Spelling: aw·toh·floor·eh·suhns

- Nanoparticle
Pronunciation link: https://www.merriam-webster.com/dictionary/nanoparticle
IPA: /ˌnænoʊˈpɑːrtɪkəl/
Phonetic Spelling: nan·oh·par·tuh·kuhl

- Spectroscopy
Pronunciation link: https://www.merriam-webster.com/dictionary/spectroscopy
IPA: /spekˈtrɑːskəpi/
Phonetic Spelling: spek·trah·skuh·pee

- Photobleaching
Pronunciation link: https://www.merriam-webster.com/dictionary/photobleaching
IPA: /ˌfoʊtoʊˈbliːtʃɪŋ/
Phonetic Spelling: foh·toh·blee·ching

- Cytotoxicity
Pronunciation link: https://www.merriam-webster.com/dictionary/cytotoxicity
IPA: /ˌsaɪtoʊtɑːkˈsɪsɪti/
Phonetic Spelling: sy·toh·tok·siss·uh·tee

- Quantification
Pronunciation link: https://www.merriam-webster.com/dictionary/quantification
IPA: /ˌkwɑːntəfəˈkeɪʃən/
Phonetic Spelling: kwahn·tuh·fuh·kay·shuhn