# Submission ID #: 71195

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=21714523

# Title: Demonstration of a Training Protocol for Individuals with Tetraplegia and High Paraplegia Using a Self-Balancing Exoskeleton

# Authors and Affiliations:

# Gail F. Forrest*1,2, David H. Kim3, Michelle V. Agostini1, Ashley A. Pauly3, Michael Elliott3, Steven Kirshblum1,2, Stephen L. Kornfeld1,4, Lynda M. Murray3,5, Michelle K. Zeleny1, Michael R. Kashetta1, Jonathan E. Becerril1, Erica L. Garbarini1, Francisco E. Castano3, Saikat Pal3,6, Ann M. Spungen*3

# 1Kessler Foundation

# 2Department of Physical Medicine and Rehabilitation, Rutgers New Jersey Medical School

# 3Spinal Cord Damage Research Center, James J. Peters Veterans Affairs Medical Center

# 4Spinal Cord Injury and Disorders Service, James J. Peters Veterans Affairs Medical Center

# 5Department of Rehabilitation and Human Performance, Icahn School of Medicine at Mount Sinai

# 6Department of Biomedical Engineering, New Jersey Institute of Technology

# *These authors contributed equally

# Corresponding Authors:

Ann M. Spungen 		ann.spungen@va.gov

# Email Addresses for All Authors:

Gail F. Forrest 			gforrest@kesslerfoundation.org

David H. Kim 			david.kim12@va.gov

Michelle V. Agostini 		magostini@kesslerfoundation.org

Ashley A. Pauly 		ashley.pauly@va.gov

Michael Elliott 		michael.elliott2@va.gov

Steven Kirshblum 		skirshblum@selectmedical.com

Stephen L. Kornfeld 		stephen.kornfeld@va.gov

Lynda M. Murray 		lynda.murray@va.gov

Michelle K. Zeleny 		mzeleny@kessler-rehab.com

Michael R. Kashetta 		mkashetta@kesslerfoundation.org

Jonathan E. Becerril 		jbecerril@kesslerfoundation.org

Erica L. Garbarini 		egarbarini@kesslerfoundation.org

Francisco E. Castano 		francisco.castano@va.gov

Saikat Pal 			saikat.pal@njit.edu

Ann M. Spungen 		ann.spungen@va.gov

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   Filming will occur at one location, however both inside and outside the building

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. If user/companion open to it, yes.

Current Protocol Length

Number of Steps:  25

Number of Shots:  49

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

Authors: Seven statements were supplied; the video carries five (one required plus one more in the Introduction, and any three in the Conclusion). We kept the research focus, the research gap, the safety consideration, the follow-on methods and the future directions, and edited all five lightly for length and spoken flow.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Michelle V. Agostini: Our focus was on evaluating a self-balancing exoskeleton in home environments and the user's ability to operate it.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Michelle V. Agostini: Exoskeletons need trunk and arm strength to maneuver; this self-balancing device brings walking benefits to people with less strength.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

Describe the most important consideration or challenge when performing this protocol.

  - Michelle V. Agostini: A key consideration during the procedure is user safety while screening and after use. The risks of robotic devices for people with spinal cord injury must be carefully evaluated.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Are there any additional methods or analyses that can be performed following this procedure?

  - Michelle V. Agostini: Additional methods can explore the real-life use of this self-balancing device in users' homes over time.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Michelle V. Agostini: Future studies can explore the benefits of robotic walking for people with higher cervical lesions.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Videographer: Obtain headshots for all authors available at the filming location.

Testimonial Questions (OPTIONAL):

Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

Testimonial statements will not appear in the video but may be featured in the journal's promotional materials.

Provide the full name and position of the person delivering the testimonial.

Please answer the testimonial question live during the shoot, speaking naturally and in your own words in complete sentences.

What problem in your field of research does your JoVE article address?

  - (name and position to follow): (the testimonial statement will be delivered live during the shoot)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

What is the key takeaway you hope viewers remember after watching your video article?

  - (name and position to follow): (the testimonial statement will be delivered live during the shoot)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

# Ethics Title Card

This research has been approved by the Institutional Review Boards at the James J. Peters Veterans Affairs Medical Center and the Kessler Foundation

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Exoskeleton Setup and Donning

Demonstrator: 1 device trainer; 1 user; 1 companion

Authors: Thank you — noted that the device user and companion cannot be named until the filming date is set. Please send the names after the shoot, and we will drop them into the Demonstrator lines.

Authors: I have introduced a talent (the research author), a user, and a companion in the script. Please let me know if each action is correctly mapped to talent, user and companion. Please let me know if the shot description needs to be changed for any user with tracking mode on

  - To begin, explain the device features to the participant with a spinal cord injury, or SCI [1]. Inform them that four trainer-led sessions, each lasting 1 to 2 hours, will be conducted before the initial independent certification assessment, and additional training can be offered if required [2]. Instruct the user and the companion to perform independent skin inspections before and after each exoskeleton use [3].

    - WIDE: Talent showing and explaining the device features to the User.

    - Talent gesturing 4 and pointing to the user and the companion.

    - User and companion performing skin inspection before the exoskeleton use.

  - Position the exoskeleton on the designated bench in the seated position [1]. Raise both armrests and the HALO (halo) [2] and activate INSTALLATION (installation) mode to open the device legs [3].

    - WIDE: Talent positioning the exoskeleton on the designated bench in the seated position.

    - Talent raising both armrests and the HALO.

    - Talent activating INSTALLATION mode to open the device legs.

Authors: Thank you — confirmed as physical buttons on the device.

Videographer: Please capture the physical buttons in close-up so that they are clearly visible.

  - Transfer the SCI device User into the exoskeleton using thier preferred transfer method. Use a transfer aid when required [1].

    - Talent transferring the User into the exoskeleton using the participant’s preferred transfer method.

  - Secure the pelvic band, the lower-extremity straps, the shoulder straps, the chest straps. Then fasten the tibial interfaces, the HALO, and the armrests before standing [1]. Ensure that the armrests are separated by at least 90 centimeters during the donning and doffing procedures [2].

    - Talent securing the pelvic band, the lower-extremity straps, the shoulder straps, the chest straps, the tibial interfaces, the HALO, and the armrests. Videographer: Please provide verbal cues when each of the straps is being secured so that our video editors can recognize them accurately

    - Talent measuring the separation between the armrests to confirm at least 90 centimeters before donning.

Authors: Figure 1A labels the Halo, Headrest, Vest, Shoulder straps, Shoulder pads, Controller, Armrests, Belt, Thigh straps, Knee straps and Feet straps — there is no “pelvic band”, no “chest straps” and no “tibial interfaces”. Can you please label these in Figure 1A or please let us know if we need to modify some names in the narration above.

- Standing and Balance Exercises

Demonstrator: 1 device trainer; 1 user; 1 companion

Authors: I have instructed the Talent to operate the device during the early standing training in chapter 3 and then User operates it from chapter 5 onward, as competence builds — please confirm that this progression is correct.

  - Explain the standing procedure to the User and the companion [1]. Ensure that a bench is positioned behind the exoskeleton before initiating any maneuver [2]. Press and hold the sit-to-stand button to place the device in READY-TO-STAND mode [3]. Instruct the User to push and hold the joystick toward the green indicator to initiate standing [4].

    - WIDE: Talent explaining the standing procedure to the User and the companion.

    - Talent pointing to the bench behind the exoskeleton. Authors: This step is edited and is now placed here. Please confirm if the flow is correct

    - Talent pressing and holding the sit-to-stand button to activate READY-TO-STAND mode.

    - Talent instructing and the User pushing and holding the joystick toward the green indicator to initiate standing.

  - Monitor the User for orthostatic symptoms during the sit-to-stand transition [1]. Confirm that the joystick indicators illuminate white, indicating STANDING mode [2].

    - Talent monitoring the User during the sit-to-stand transition.

    - Talent pointing out the joystick indicators illuminated white, indicating STANDING mode.

  - Activate FREE mode by pressing the top-right button on the remote-control unit [1].

    - Talent pressing the top-right button on the remote-control unit to activate FREE mode.

  - Instruct the User to perform a semi-squat while in FREE mode [1]. Then, let them perform a left and a right weight shift.  FREE mode may also be used during the activities of daily living to facilitate reaching and opening doors or cabinets [2].

    - User performing a semi-squat while in FREE mode.

    - User performing a left and then a right weight shift while in FREE mode.

- Sitting and Doffing

Demonstrator: 1 device trainer; 1 user; 1 companion

  - Press and hold the stand-to-sit button to place the device in READY-TO-SIT mode [1].

    - Talent pressing and holding the stand-to-sit button to activate READY-TO-SIT mode.

  - Instruct the User to pull and hold the joystick toward the green indicator to initiate sitting [1]. Confirm that the User is safely seated before proceeding with doffing [2].

    - Talent instructing and the User pulling and holding the joystick toward the green indicator to initiate sitting.

    - Talent checking and examining that the User is safely seated in the exoskeleton.

  - Release all the restraints and open the tibial interfaces [1]. Then, position the wheelchair and transfer the User out of the exoskeleton [2].

    - Talent releasing all the restraints and opening the tibial interfaces.

    - Talent positioning the wheelchair beside the exoskeleton and transferring the User out of the exoskeleton.

  - Next, conduct the certification evaluation session to verify proficiency in all required basic skills, critical tasks, safety procedures, and theoretical knowledge, in accordance with the certification checklist [1]. Record the companion's level of assistance at each evaluation visit, using the three-level assistance scale [2].

    - User and companion performing a sit-to-stand transfer while the Talent looks at them.

    - Talent ticks items off the certification checklist or writes notes in a notepad.

- Indoor Mobility Training

Demonstrator: 1 device trainer; 1 user; 1 companion

  - After the User dons the device, instruct them and the companion to avoid collisions with walls, objects, or other obstacles while walking indoors [1]. Let the companion remain within an arm’s reach of the User at all times [2].

    - WIDE: Talent instructing the User and the companion to avoid walls, objects, and obstacles with actions and gestures.

    - Companion moving into position within an arm’s reach of the User.

  - Activate PRECISION mode by pressing the large center button on the remote-control unit [1]. Perform short forward, backward, and lateral steps as needed [2].

    - User pressing the large center button on the remote-control unit to activate PRECISION mode.

    - User performing short forward, backward, and lateral steps in PRECISION mode.

  - Instruct the companion to maintain both hands on the rear handle while navigating doorways, thresholds, crowded hallways, or other potentially unstable environments [1].

    - Companion maintaining both hands on the rear handle while assisting the User through a doorway.

  - Move the joystick in the desired direction to initiate walking while in WALKING mode [1].

    - User moving the joystick to initiate walking in WALKING mode.

  - Move and hold the joystick left or right to make directional changes, including 180- and 360-degree turns [1].

    - User moving and holding the joystick to perform directional changes, including 180-degree and 360-degree turns.

  - Activate FREE mode by pressing the top-right button on the remote-control unit [1]. Perform lateral weight shifts, forward reaching, weight transfers, or semi-squat movements [2].

    - User pressing the top-right button on the remote-control unit to activate FREE mode.

    - User performing lateral weight shifts, forward reaching, weight transfers, and semi-squat movements in FREE mode.

  - Then, use the FREE mode to reach and operate the door handles, open the manual doors and navigate the thresholds [1]. Maintain both hands on the rear handle while assisting with doorway, elevator, or threshold navigation [2].

    - User reaching for and operating a door handle while in FREE mode.

    - Close up of the companion maintaining both hands on the rear handle while assisting the User into an elevator.

  - Press the PAUSE button if an obstacle or a hazardous situation is encountered during walking [1]. Release the joystick immediately to stop the exoskeleton when an immediate hazard is identified [2].

    - Talent/companion pressing the PAUSE button during walking.

    - User releasing the joystick immediately to stop the exoskeleton. Authors: please check if this is correct and if the user releases the joystick

- Activities of Daily Living

Demonstrator: 1 device trainer; 1 user; 1 companion

  - Use PRECISION and FREE modes to position the User appropriately and facilitate the reaching activities [1]. Perform bathroom activities while using the exoskeleton, including handwashing, toothbrushing, and hair grooming, while standing at a sink or a mirror [2].

    - User using PRECISION and FREE modes to position themselves at the sink for hand washing.

    - WIDE: User washing their hands or grooming their hair at the sink and mirror, with the Talent and the companion alongside.

  - Perform the kitchen activities using the FREE mode to access the lower cabinets [1]. Move around the countertops and carry the lightweight objects. Do not handle loads of 3 kilograms or more [2].

    - User accessing the lower cabinets while in FREE mode.

    - User moving around the countertops and carrying the lightweight objects while in FREE mode.

  - Use the seating that supports the combined weight of the User and the exoskeleton [1-TXT]. Ensure that the armrests are separated by at least 60 centimeters during sitting and standing activities [2].

    - Talent pointing out a seating surface that supports the combined weight of the User and the exoskeleton. TXT: Height: ~38 - 56 cm; Minimum depth: 40 cm; Minimum width: 40 cm

    - User sitting down and standing up while the armrests remain separated by at least 60 centimeters.

- Community Mobility Training

Demonstrator: 1 device trainer; 1 user; 1 companion

  - Use only the stable, cushioned seating surfaces that are not mounted on wheels [1]. Use FREE mode to reach the objects [2]. Use PRECISION mode to maneuver safely when approaching or leaving a seating surface [3].

    - Talent pointing out a stable, cushioned seating surface that is not mounted on wheels.

    - User reaching for an object while in FREE mode.

    - User maneuvering safely when approaching a seating surface and leaving the seating surface in PRECISION mode.

  - Walk outdoors on cement, asphalt, flat grass, and the surface transitions with slopes of 2 percent grade or less and the surface irregularities of 0.5 inches or less [1-TXT]. Walk a minimum distance of 50 meters outdoors while being timed [2]. Encourage continuous walking but allow the User to pause or stop as needed. Do not expose the exoskeleton to water, excessive heat, excessive cold, or direct sunlight [3]. Finally, analyze the outcomes after completing the training, certification, and assessment procedures [4].

    - WIDE: User walking outdoors on cement, asphalt, flat grass or the surface transitions while using the exoskeleton.

    - User walking while the talent holds and watches a timer.

    - User pausing during the timed walk before resuming walking.

    - Talent writing in a notepad or typing on a computer.

# Results

- Results

  - Sixteen SCI device user–companion pairs completed the study. After the 4 training sessions, all 16 device users and companions completed the timed-up-and-go test within the prespecified goal of 3 minutes or less [1] and achieved the 6-minute walk test performance goal of 40 meters or more [2]. Eleven of the 16 participants achieved the prespecified 10-meter walk test performance goal of 0.20 meters per second or more [3]. Fifteen of the 16 participants donned the exoskeleton within the target time of 10 minutes or less [4], and all 16 doffed the exoskeleton within the target time [5].

    - LAB MEDIA: Table 2. Video editor: Highlight the column labeled "TUG (min)" and the performance goal "≤3 min".

    - LAB MEDIA: Table 2. Video editor: Highlight the column labeled "6MWT (m)" and the performance goal "≥40 m".

    - LAB MEDIA: Table 2. Video editor: Highlight the column headed "10MWT (m/s)" and the performance goal "≥0.20 m/s".

Authors: In Table 2 are you also considering the 0.197 and 0.198 values in the “goal achieved category” for 6-minute walk test performance goal of 40 meters or more?

    - LAB MEDIA: Table 2. Video editor: Highlight the column headed "Donning (min)" and the performance goal "≤10 min".

    - LAB MEDIA: Table 2. Video editor: Highlight the column headed "Doffing (min)" and the performance goal "≤10 min".

  - There were no significant differences between the tetraplegia and high paraplegia groups for the total session time [1], the upright time [2], the number of steps [3], or the distance walked [4] across the 4 training sessions and the 5th assessment session. Both groups demonstrated significant improvements across the sessions for all 4 outcomes [5].

    - LAB MEDIA: Figure 3A. Video editor: Highlight both lines in the graph labeled "Tetra" and "HPara" for panel A, Total Time.

    - LAB MEDIA: Figure 3B. Video editor: Highlight both lines in the graph labeled "Tetra" and "HPara" for panel B, Up Time.

    - LAB MEDIA: Figure 3C. Video editor: Highlight both lines in the graph labeled "Tetra" and "HPara" for panel C, Total Steps.

    - LAB MEDIA: Figure 3D. Video editor: Highlight both lines in the graph labeled "Tetra" and "HPara" for panel D, Distance Walked.

    - LAB MEDIA: Figure 3. Video editor: Highlight the "*P<0.0001" annotation in each of the four panels.

Authors: Can you label the point 5 as the assessment session on axis and the 1 to 4 as training sessions? This will help us in annotation during the video assembly.

  - Fourteen of the 16 participants successfully completed all 6 activities of daily living tasks [1], whereas 2 participants completed 5 of the 6 tasks by Session 4, having not been tested on the outdoor walk because of inclement weather [2].

    - LAB MEDIA: Table 3. Video editor: Highlight the "Completed All 6 ADLs" column and its "14/16 87.5%" total, on the bottom of the table.

    - LAB MEDIA: Table 3. Video editor: Highlight the "Completed 5 ADLs" column and its "2/16 12.5%" total, on the bottom of the table.

Authors: Table 3 marks those two participants “NT”, not tested, on the outdoor walk because of inclement weather. This line is included in the narration now, can you please confirm if this sounds good or should I remove this?