# Submission ID #: 71209

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/account/file-uploader?src=21718793

Title: Real-Time Label-Free Imaging and Quantitative Analysis of Macrophage Morphodynamics Using Optical Diffraction Tomography

# Authors and Affiliations:

Chen Sun1, Jie-jie Zhu2, Yan-qing Yang2, Si-yu Liu1, Ke-bin Shi2#, Cheng Peng1#

1Sichuan Provincial Key Laboratory of Innovation and Efficient Utilization of Chinese Medicine Germplasm Resources, Chengdu University of Traditional Chinese Medicine

2Yangtze River Delta Institute of Optoelectronics, Peking University

# Corresponding Authors:

Ke-bin Shi 	(kebinshi@pku.edu.cn)

Cheng Peng 	(pengchengchengdu@126.com)

# Email Addresses for All Authors:

Chen Sun 	(sunchencd@126.com)

Jie-jie Zhu	(zj291027@163.com)

Yan-qing Yang	(yangyanqing@pellucid-optics.cn)

Si-yu Liu	(1317295721@qq.com)

Ke-bin Shi 	(kebinshi@pku.edu.cn)

Cheng Peng 	(pengchengchengdu@126.com)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  YES

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible:https://review.jove.com/account/file-uploader?src=21718793

3. Filming location: Will the filming need to take place in multiple locations?   NO

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. YES

Current Protocol Length

Number of Steps:  11

Number of Shots:  35

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Chen Sun: We developed a label-free ODT-based workflow to monitor dynamic macrophage behavior at the single-cell level.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Chen Sun: Traditional endpoint assays cannot continuously track live-cell dynamics, while ODT enables long-term label-free observation of living cells.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

Describe the most important consideration or challenge when performing this protocol.

  - Ke-bin Shi: The most important consideration while performing this protocol is maintaining stable focus and appropriate cell density for reliable long-term single-cell tracking.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Are there any additional methods or analyses that can be performed following this procedure?

  - Ke-bin Shi: This workflow can be combined with multimodal imaging, 3D reconstruction, and additional quantitative morphological analyses.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Ke-bin Shi: Future studies may further improve automated analysis in complex cellular environments and expand this workflow to other live-cell systems.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Testimonial Questions (OPTIONAL):

# Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

- Testimonial statements will not appear in the video but may be featured in the journal’s promotional materials.

- Provide the full name and position (e.g., Director of [Institute Name], Senior Researcher [University Name], etc.) of the author delivering the testimonial.

- Please answer the testimonial question live during the shoot, speaking naturally and in your own words in complete sentences.

What problem in your field of research does your JoVE article address?

  - Ke-bin Shi, Professor, Peking University,: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

What value do you expect this video article to have for other researchers working in your field?

  - Ke-bin Shi, Professor, Peking University,: (authors will present their testimonial statements live)

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Authors: Could you please also deliver the above statements in Chinese?

Videographer: Please film the testimonials in both English and Chinese

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Optical Diffraction Tomography (ODT) Acquisition

Demonstrators: Chen Sun and Jie-jie Zhu

  - To begin, take cultured RAW264.7 (Raw-Two-Sixty-Four-Point-Seven) macrophage cells with 80 to 90 percent confluency and treat them with lipopolysaccharide and Baicalin [1]. Then, turn on the optical diffraction tomography imaging system [2] and start the environmental control module [3].

    - WIDE: Talent at the working area, adding LPS and BAI+LPS to the culture dish. Videographer: Please obtain CU shot as well for this shot.

    - Talent turning on the optical diffraction tomography imaging system.

    - SCREEN: Talent starting the environmental control module.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible:https://review.jove.com/account/file-uploader?src=21718793

  - Set the stage temperature to 37 degrees Celsius [1] and the carbon dioxide concentration to 5 percent [2]. Then, allow the system to equilibrate for at least 20 minutes [3].

    - Talent setting the stage temperature to 37 degrees Celsius.

    - Talent setting the carbon dioxide concentration to 5 percent.

    - Shot of the system equilibrating.

  - Next, open the acquisition software [1], place the culture dish on the imaging stage [2]. Use a 60X objective with a numerical aperture of 1.42 to locate cells [3] and select isolated cells with clear boundaries for imaging [4].

    - SCREEN: Show the acquisition software opening.

    - Talent placing the culture dish on the imaging stage.

    - SCREEN: Talent locating cells with the 60X objective.

    - SCREEN: Talent selecting isolated cells with clear boundaries.

  - Set the imaging interval to 8 minutes and the total number of frames to 180 [1].

    - SCREEN: Show the imaging interval set to 8 minutes and show the total number of frames set to 180.

  - Enable the autofocus function to maintain focus during time-lapse imaging and start time-lapse acquisition [1]. Monitor system stability during acquisition [2].

    - SCREEN: Show the autofocus function being enabled. Show time-lapse acquisition starting.

    - Talent monitoring system stability during acquisition.

- Quantitative Analysis of Macrophage Morphodynamics

  - After image acquisition, the software automatically performs image reconstruction using the built-in standard algorithm [1]. Wait until the progress bar is complete [2].

    - SCREEN: Show the image reconstruction progress bar.

    - SCREEN: Show the progress bar completed. Show the reconstructed image.

  - Next, open the analysis software [1]. Import the raw dataset by selecting Import Raw Data, browsing to the target folder, and choosing the file [2].

    - SCREEN: Show Intellyseg version 2.1.0 launching on the computer.

    - SCREEN: Show Import Raw Data selection. Show the file browser navigation, file selection, and dataset import in sequence.

  - Then, click “Cell“ to perform automatic segmentation using consistent settings across datasets [1].

    - SCREEN: Show clicking “Cell“ and the automatic segmentation in progress.

  - Select cells that do not overlap with neighboring cells [1], are not in contact with image boundaries [2], and are continuously trackable throughout the time series [3].

    - SCREEN: Show selection of a cell with no overlap with neighboring cells.

    - SCREEN: Show selection of a cell not touching the image boundaries.

    - SCREEN: Show selection of a cell that remains continuously trackable across frames.

  - Select a fixed number of cells per field of view for consistent analysis across datasets [1]. Open the Long-term Tracking module [1].

    - SCREEN: Show exactly 8 cells being selected within a single field of view.

    - SCREEN: Show the Long-term Tracking module opening.

  - Then, enter the start and end frame [2]. Click Track to initiate tracking based on cell position consistency between consecutive frames [1].

    - SCREEN: Show the start frame and end frame value being entered.

    - SCREEN: Show the cursor clicking Track to begin tracking.

# Results

- Results

  - Time-lapse optical diffraction tomography imaging enabled continuous recording of single-cell morphological changes over time [1].

    - LAB MEDIA: Figure 2.

  - At early time points, cells in both lipopolysaccharide and Baicalin plus lipopolysaccharide groups exhibited relatively round morphology with clear boundaries and minimal variation [1].

    - LAB MEDIA: Figure 2. Video editor: Highlight the panels labeled 0 h and 6 h in both rows showing round cells with smooth edges

  - After 8 hours, cells showed increased morphological diversity, including spreading, irregular edges, and protrusions [1]. At later stages, these changes intensified, with greater contour complexity and heterogeneity.[2].

    - LAB MEDIA: Figure 2. Video editor: Highlight the panels labeled 12 h and 18 h showing cells with uneven shapes and extensions

    - LAB MEDIA: Figure 2. Video editor: Highlight the 24 h panels and zoom into the inset showing irregular and complex cell shapes

  - Representative single-cell tracking trajectories were visualized over time for both treatment groups [1].

    - LAB MEDIA: Figure 3A.

  - Projected cell area remained relatively stable during the early phase in both groups, fluctuating around 140 to 170 square micrometers [1]. After approximately 12 hours, a subset of cells exhibited increased projected area, with more pronounced changes during 12–24 hours [1].

    - LAB MEDIA: Figure 3B. Video editor: Highlight the early time region (0–8 h) on the area plots where lines cluster around 140–170

    - LAB MEDIA: Figure 3B. Video editor: Highlight the region after 12 h where several lines trend upward

  - Under lipopolysaccharide treatment, some cells showed rapid expansion in projected area exceeding 400 to 500 square micrometers [1]. However, under Baicalin plus lipopolysaccharide treatment, most cells maintained a projected area of 250 to 300 square micrometers during the later stage [2].

    - LAB MEDIA: Figure 3B. Video editor: Highlight the lines in the LPS area plot that rise above 400–500

    - LAB MEDIA: Figure 3B. Video editor: Highlight the lines in the BAI+LPS area plot staying around 250–300 at later times

  - Cell perimeter remained around 40 to 50 micrometers during the early phase in both groups [1].

    - LAB MEDIA: Figure 3B. Video editor: Highlight the early time region (0–8 h) on the perimeter plots where values cluster near 40–50

  - In the mid-to-late phase, some lipopolysaccharide-treated cells exceeded 100 micrometers in perimeter [1], whereas most Baicalin plus lipopolysaccharide-treated cells remained around 70 micrometers at later stages [2].

    - LAB MEDIA: Figure 3B. Video editor: Highlight the lines in the LPS perimeter plot that rise above 100

    - LAB MEDIA: Figure 3B. Video editor: Highlight the lines in the BAI+LPS perimeter plot near 70 at later times