# Submission ID #: 71277

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=21739533

# Title: Remote Intraoperative Monitoring of Cochlear Implants

# Authors and Affiliations:

# Pascal Nachtigäller1, Joachim Müller2, Daniel Polterauer-Neuling1

# 1Division of Audiology, Department for Otorhinolaryngology, LMU University Hospital Munich

# 2Department for Otorhinolaryngology, LMU University Hospital Munich

# Corresponding Authors:

# Daniel Polterauer-Neuling		daniel.polterauer@med.uni-muenchen.de

# Email Addresses for All Authors:

# Pascal Nachtigäller			pascal.nachtigaeller@med.uni-muenchen.de

# Joachim Müller			joachim.mueller@med.uni-muenchen.de

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

☐ Correct

☐ Incorrect

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://review.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

As these files are necessary for finalizing your script, please upload all screen-captured video files to your project page as soon as possible.

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: MM/DD/YYYY

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  14

Number of Shots:  32 (23 SC)

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

Authors: Please provide the presenters’ names for the interviews after the shoot

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Enter author name.: Our main goal is to show the feasibility and efficiency of remote monitoring for cochlear implantation.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Enter author name: In situ monitoring is time-intensive for the audiologist. This time investment can be reduced by using remote monitoring.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Enter author name: Intraoperative monitoring indicates device functionality by checking integrity and connectivity to the auditory system, which is possible remotely.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Enter author name: The most important consideration during the procedure is giving the right instructions to the OR staff and surgeon to facilitate smooth operation.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Enter author name: In future, we will test the feasibility of remote eABR or ANTS measurements.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Title Card

Written informed consent was obtained from the patients to use and publish their data, where applicable

# Protocol

- Preparation and Setup for Remote Intraoperative Monitoring

Demonstrator: Click here to enter name of demonstrator(s)

  - To begin, obtain the cochlear implant model information [1] and notify the audiologist approximately 10 minutes before intraoperative measurements [2].

    - WIDE: OR staff asking surgeon, fetching the implant and showing it to surgeon for confirmation

    - OR staff calling the audiologist and giving out details. Audiologist speaking over the call. Authors: Please film the OR staff and the audiologist, both, in separate rooms. We can use split screen to show both to convey the scene

  - Turn on the measurement computer [1] and allow the audiologist to log in remotely [2].

    - OR staff turning on the measurement computer.

    - SCREEN: audiologist logging in remotely on the measurement computer.

Authors: Please create screen capture videos of the shots labeled as SCREEN, write a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21739533

  - Open the measurement software [1] and enter the patient details and the implant information [2]. Confirm the connection to the implant when the surgeon and the audiologist are ready to begin the measurement [2].

    - SCREEN: measurement software being opened.

    - SCREEN: patient details and the implant information being entered into the measurement software.

    - SCREEN: Show the confirmation of the connection to the implant.

- Intraoperative Cochlear Implant Monitoring

Demonstrator: Click here to enter name of demonstrator(s)

  - Measure the impedances [1]. Note the measurements and inform the surgeon of the values [2].

    - SCREEN: Show impedance measurement

    - Audiologist noting the impedance measurement results and calling the surgeon and talking.

  - Review the stimulation current-induced non-stimulating electrode voltages for anomalies related to the electrode placement, including possible tip fold-overs [1].

    - SCREEN: Show and highlight (hover cursor over) the stimulation current-induced non-stimulating electrode voltages being reviewed for placement-related anomalies.

  - After the impedance measurements, confirm whether the electrically evoked stapedial reflex is indicated based on anatomical factors and prior operations [1].

    - SCREEN: Show the data and hover the cursor around the area to look for the evoked stapedial reflex.

  - If the electrically evoked stapedial reflex measurement is indicated, measure the reflex [1] and, if time allows, measure its threshold [2].

    - SCREEN: the electrically evoked stapedial reflex being measured.

    - SCREEN: the electrically evoked stapedial reflex threshold being measured.

  - Measure at least one apical electrode [1], one medial electrode [2], and one basal electrode, including electrodes near the beginning and the end of the electrode array [3].

    - SCREEN: Show the measurement of at least one apical electrode.

    - SCREEN: Show the measurement of one medial electrode.

    - SCREEN: Show the measurement of one basal electrode, including electrodes near the beginning and the end of the electrode array.

  - Next, visually inspect whether the stapedial reflex is triggered [1] and provide oral feedback to the audiologist [2].

    - Surgeon visually inspecting whether the stapedial reflex is triggered.

    - Surgeon providing oral feedback to the audiologist.

  - If the reflexes are absent for selected electrodes, especially the most basal electrodes, confirm whether the electrode array is fully inserted and adjust it if required [1].

    - Surgeon checking the electrode array position and adjusting it.

- ECAP Recording and Data Saving

Demonstrator: Click here to enter name of demonstrator(s)

  - Record the electrically evoked compound action potentials or ECAP after the electrically evoked stapedial reflex measurement [1], using the recommended parameters [2].

    - SCREEN: Show the electrically evoked compound action potentials being recorded after the electrically evoked stapedial reflex measurement.

    - SCREEN: Show the selected parameters.

  - Use automatic measurements for routine cases [1].

    - SCREEN: Show the automatic measurements being performed.

  - If the responses are difficult to evaluate or further assessment is needed, repeat the measurement manually [1].

    - SCREEN: Show the measurement being repeated manually.

  - Finally, save the data to the connected database if no additional measurements are needed [1] and shut down the computer [2].

    - SCREEN: Show the measurements being saved to the connected database.

    - Audiologist shutting down the computer.

# Results

- Results

  - Impedances were the most common measurement and were performed every time [1]. Electrocochleography was the least frequent measurement [2].

    - LAB MEDIA: Figure 6. Video editor: Highlight the bar labelled Imp.

    - LAB MEDIA: Figure 6. Video editor: Highlight the bar labelled ECochG.

  - A normal stimulation current-induced non-stimulating electrode voltage result was observed [1] and a characteristic off-diagonal pattern of tip fold-over could also be detected [2].

    - LAB MEDIA: Figure 5A. Video editor: Show the panel labelled A.

    - LAB MEDIA: Figure 5B. Video editor: Show the panel labelled B.

  - Remote measurements reduced the median time spent by the audiologist from 29.03 minutes in situ [1] to 8 minutes remotely [2].

    - LAB MEDIA: Figure 7. Video editor: Highlight the Standard in-situ plot.

    - LAB MEDIA: Figure 7. Video editor: Highlight the Standard remote plot.

  - When electrocochleography was measured, remote measurements reduced the median time spent by the audiologist from 48.63 minutes in situ [1] to 27.75 minutes remotely [2].

    - LAB MEDIA: Figure 7. Video editor: Highlight the ECochG in-situ plot.

    - LAB MEDIA: Figure 7. Video editor: Highlight the Standard remote plot.

Watch this video and apply the settings while capturing the screen to obtain high-quality videos: https://www.youtube.com/watch?v=Y67dM1kKcTE

NOTE to the Authors:

You have two options for filming and submitting the SCREEN footage:

Option 1: Uploading Separate Clips (Highly Recommended)

Please upload a separate video file for each SCREEN shot (tagged as SCREEN), naming the file according to the shot number (e.g., 2.4.3, 1.5.1, 3.2.1, 3.1.3, etc.).

- Each clip must be a maximum of 20 to 25 seconds to match the voice narration.

- If an action takes longer, provide only the best, most representative 20-25 second segment.

- Please do not provide longer clips as we won’t be able to use them.

Option 2: Uploading a Single Continuous Video

If you prefer to record all steps together and upload a single video file, that is acceptable. In this case, you must write the precise timestamp for each shot within the script.

For example, if the action described in shot 2.13.1 is present in the file xxx.mp4 and occurs between timestamps 00:30 and 00:45 , then write the following in the script after each shot description or as a comment bubble.

(write the file name and timestamp after each shot): [File name] [Start Time]-[End Time]

- Example 	2.13.1  SCREEN: performing ...........action. xxx.mp4 00:30-00:45

2.13.2, SCREEN: cutting the…..........action. xxx.mp4 01:10-01:20

- The duration between the start and end times for each shot must be limited to 20 to 25 seconds.

- If a shot is longer than 25 seconds, choose two short segments (one at the beginning and one at the end) that collectively do not exceed 25 seconds.

- Example: 3.1.1 SCREEN: inserting a needle... xxx.mp4 00:00-00:10 and 02:30-02:40