71307_screenshot_1.mp4

  - Open the ImageJ (Image-J) or Fiji software [1]. 00:02-00:11

    - SCREEN: Show the ImageJ/Fiji application being launched and the main interface opening.

  - Click and drag the .vsi (V-S-I) file onto the main ImageJ or Fiji application window [1]. 00:24-00:28

    - SCREEN: Show the .vsi file being dragged from a folder and dropped onto the main ImageJ/Fiji window.

  - In the Bio-Formats Import Options prompt, click OK in the bottom right [1]. 00:28-00:32

    - SCREEN: Show the Bio-Formats Import Options dialog box and click the OK button in the bottom right.

  - Then, in the Bio-Formats Series Options prompt, ensure Series 1 is selected [1]. Click OK in the bottom left [2]. 00:32-00:39

    - SCREEN: Show the Bio-Formats Series Options dialog and cursor clicking on Series 1.

    - SCREEN: Show the cursor clicking the OK button in the bottom left of the dialog box.

  - Once the image has loaded, split the signals by selecting Image [1]. Select Color and choose Split Channels [2]. View the separate windows generated for GFP-labeled nuclei and RFP-labeled chromatin [3]. 00:59-01:12

    - SCREEN: Show the loaded image in ImageJ/Fiji and click the Image menu with the cursor.

    - SCREEN: Show the cursor clicking Color submenu and then clicking Split Channels.

    - SCREEN: Show the resulting separate image windows displaying the GFP-labeled nuclei channel and the RFP-labeled chromatin channel.

  - Determine which of the 2 image windows corresponds to chromatin [1]. 01:12-01:20

    - SCREEN: Show the 2 image windows and point to the window containing the chromatin signal using cursor.

  - After clicking the chromatin image window, apply the Z-project operation by selecting Image, choosing Stacks, and then clicking Z Project from the main ImageJ window [1]. 01:20-01:30

    - SCREEN: From the main ImageJ menu, navigate to Image > Stacks > Z Project and open the Z Project dialog box.

  - Change the projection type to Max Intensity [1]. Ensure that All Time Frames is active and click OK to generate the projection [2]. 01:30-01:44

    - SCREEN: Show the projection type being changed to Max Intensity.

    - SCREEN: Show the All Time Frames option enabled and cursor clicking OK.

  - Open the contrast menu by pressing Ctrl+Shift+C [1] or by selecting Image and choosing Adjust, and then clicking Brightness or Contrast from the main ImageJ window [2]. 01:44-01:58

    - Close-up shot of pressing Ctrl+Shift+C

    - SCREEN: Show the main ImageJ window and cursor selecting Image > Adjust > Brightness/Contrast.

  - Slide the Maximum bar to the left until the chromatin is clearly visible [1]. 01:58-02:06

    - SCREEN: Show the Maximum slider being moved to the left and the chromatin signal becoming more visible.

  - Navigate to the Plugins, click the down arrow in the Plugins menu to scroll to the tracking option [1]. 02:06-02:12

    - SCREEN: Show the cursor navigating to the Plugins, clicking the down arrow in the Plugins menu, and scrolling to the tracking section.

  - Select Tracking and click TrackMate [1]. 02:12-02:18

    - SCREEN: Show the Tracking submenu being selected and TrackMate being launched.

  - Confirm that the calibration settings are accurate and click Next [1]. 02:18-02:25

    - SCREEN: Show the TrackMate initial settings page with the calibration settings displayed and cursor clicking Next to continue.

  - Select Thresholding Detector and click Next [1]. 02:25-02:30

    - SCREEN: Show Thresholding Detector being selected and the cursor clicking Next.

  - Move to the last frame of the time-lapse on the image window [1]. 02:30-02:36

    - SCREEN: Shot of navigating to the final frame of the time-lapse.

  - Click Auto next to the intensity threshold and click Preview [1]. 02:36-02:40

    - SCREEN: Show the Auto threshold button being selected and the cursor clicking Preview.

  - Adjust the intensity threshold and click Preview [1]. Repeat the adjustment and preview process until all chromatin is included with minimal noise, and click Next [2]. 02:40-02:49

    - SCREEN: Show the threshold value being adjusted and cursor clicking Preview.

    - SCREEN: Show repeated threshold optimization until chromatin is accurately detected with minimal background noise and cursor clicking Next.

  - Allow the initial thresholding detection to complete and click Next [1]. 02:49-02:52

    - SCREEN: Show the thresholding detection results being generated and cursor clicking Next.

  - Manually adjust the threshold until only chromatin is visible and click Next [1]. 02:52-03:10

    - SCREEN: Show the threshold being adjusted manually and cursor clicking Next.

  - Ignore the spot filtering page and click Next [1]. 03:16-03:21

    - SCREEN: Show the spot filtering page and cursor clicking Next.

  - Select Simple LAP Tracker and click Next [1]. 03:21-03:26

    - SCREEN: Show Simple LAP Tracker being selected and cursor clicking Next.

  - Set the linking max distance to 5 microns [1], gap-closing max distance to 15 microns [2], gap-closing maximum frame gap to 1, and click Next [3]. 03:26-03:31

    - SCREEN: Show the linking max distance field being set to 5 microns.

    - SCREEN: Show the gap-closing max distance field being set to 15 microns

    - SCREEN: Show the gap-closing max frame gap field being set to 1, and cursor clicking Next.

  - Allow tracking to complete [1]. Observe the tracks displayed in the original image window and click Next [2]. 03:31-03:35

    - SCREEN: Show TrackMate processing the tracking analysis.

    - SCREEN: Show the resulting tracks overlaid on the original image sequence and cursor clicking Next.

  - Press the green + (green-plus) button in the bottom left corner to create a filter [1]. Select Number of Spots in Track mode and click Auto [2]. 03:38-03:50

    - SCREEN: Show the green + button being selected.

    - SCREEN: Show Number of Spots in Track being chosen from the filter options and the cursor clicking the Auto button.

  - Ensure the filter mode is set to Above, then click Next [1]. 03:50-04:05

    - SCREEN: Show the filter mode set to Above and cursor clicking Next.

  - Click the Tracks button at the bottom of the display options page [1]. Select Export to CSV [2]. 04:05-04:21

    - SCREEN: Show the display options page and click the Tracks button.

    - SCREEN: Show the export menu and select Export to CSV.

  - Ignore the built-in plotting page and click Next [1]. 04:26-04:31

    - SCREEN: Show the plotting page and cursor clicking Next.

  - Click Capture Overlay and then choose Execute [1]. Save the video by selecting File and clicking Save As, and then choosing GIF [2]. 04:31-04:53

    - SCREEN: Show the final TrackMate page and cursor selecting Capture Overlay and then clicking Execute.

    - SCREEN: Navigate to File > Save As > GIF and save the video file.