# Submission ID #: 71357

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/account/file-uploader?src=21763933

Title: A Standardized In Vitro Procedure for Evaluating Surface Microhardness Changes in Nanohybrid Resin Composites After Whey Protein Beverage Exposure

# Authors and Affiliations:

Ayse Tugce Tunac, Tugba Ayaz

University of Health Sciences, Hamidiye Dental Faculty, Department of Restorative Dentistry

# Corresponding Authors:

Ayse Tugce Tunac  		(aysetugce.tunac@sbu.edu.tr)

# Email Addresses for All Authors:

Ayse Tugce Tunac  		(aysetugce.tunac@sbu.edu.tr)

Tugba Ayaz 			(241002090@sbu.edu.tr)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21763933

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps:  32

Number of Shots:  44

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Tugba Ayaz: We examine the impact of whey protein beverages on the surface microhardness of nanohybrid resin composites.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera Suggested B roll: 4.1.1.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Tugba Ayaz: Existing methods lack standardized beverage exposure conditions, limiting reproducibility. This protocol establishes controlled exposure conditions, enabling reliable, comparable microhardness assessments of resin composites.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Tugba Ayaz: This method allows researchers to quantify microhardness changes caused by dietary protein beverages in restorative materials.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera Suggested B roll: 6.5.1

Describe the most important consideration or challenge when performing this protocol.

  - Tugba Ayaz: The most important consideration while performing this protocol is maintaining consistent specimen preparation and controlled immersion conditions to ensure reliable results.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera Suggested B roll: 2.2.2

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Tugba Ayaz: Future studies can investigate additional beverages, longer exposures, and other material properties.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Preparation of Nanohybrid Resin Composite Disc Specimens

Demonstrator: Tugba Ayaz

  - To begin, prepare a total of 90 disc-shaped specimens from three nanohybrid resin composites [1].

    - WIDE: Talent placing the nanohybrid resin composites on the bench. Videographer: Please obtain CU shot as well for this shot.

  - Fabricate the specimens using cylindrical clear silicone molds [1]. Place the composite material into the molds in a single 2-millimeter-thick increment [2]. Cover the surface with a clear microscope slide to obtain a flat surface [3].

    - Talent placing cylindrical clear silicone molds on the work surface.

    - Talent placing composite material into the silicone molds.

    - Talent covering the mold surface with a clear microscope slide.

  - Polymerize the specimen for 20 seconds using a blue LED light-curing unit at 1000 milliwatts per square centimeter, positioned approximately 1 millimeter from the surface [1].

    - Talent positioning the blue light-emitting diode light-curing unit above the specimen surface and polymerizing the specimen.

  - Verify the curing light intensity using a radiometer before each polymerization cycle to ensure consistent light exposure [1].

    - Talent checking the curing light intensity with a radiometer.

  - Store the prepared specimens in distilled water [1] at 37 degrees Celsius for 24 hours to allow post-polymerization and simulate initial oral conditions [2].

    - Talent placing the prepared specimens in distilled water for storage.

    - Images/recordings of placing the stored sample at 37 oC.

- Finishing and Polishing of Resin Composite Specimen

  - Using medium, fine, and superfine aluminum oxide-impregnated discs, polish the upper surfaces of all specimens sequentially [1] for 10 seconds each under water cooling at 10,000 rpm using light pressure and a unidirectional rotational motion [2].

    - Talent showing medium, fine, and superfine aluminum oxide-impregnated discs

    - Talent polishing the specimen surface with a medium polishing disc under water cooling.

  - Replace the polishing discs after every five specimens to maintain consistent abrasive efficiency and ensure surface standardization [1].

    - Talent replacing the polishing disc after polishing five specimens.

  - After each polishing step, rinse the specimens with distilled water for 10 seconds to remove debris [1]. Gently air-dry the specimens for 5 seconds before proceeding to the next disc [2].

    - Talent rinsing the polished specimens with distilled water.

    - Talent places the specimens aside for air drying.

  - Mark the unpolished surface of each specimen with a unique identification code to ensure consistent orientation during subsequent testing [1]. Then, store the specimens in distilled water [2] at 37 degrees Celsius for 24 hours prior to baseline microhardness testing [3].

    - Talent marking the unpolished surface of the specimen with an identification code.

    - Talent placing the specimens in distilled water for storage.

    - Images or recordings of placing the stored sample at 37 oC.

- Baseline Microhardness Measurement

  - Perform baseline Vickers microhardness or VHN (V-H-N) testing using a VHN tester [1]. Apply a 200 g load for 15 seconds during the microhardness measurement [2].

    - LAB MEDIA/SCREEN: Images or recordings of positioning a specimen on the Vickers microhardness tester for baseline testing.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21763933

    - LAB MEDIA/SCREEN: Images or recordings of performing the Vickers microhardness measurement using a 200 g load.

  - Make three indentations per specimen with at least 1 millimeter spacing between each indentation [1]. Perform the indentations on the central surface area of each specimen to avoid edge effects [2].

    - LAB MEDIA/SCREEN:  Images or recordings of creating multiple indentations on the specimen surface with appropriate spacing.

    - LAB MEDIA/SCREEN: Images or recordings of the indentations positioned on the central surface area of the specimen.

  - Calculate the mean baseline VHN for each specimen, with all measurements performed by a single calibrated operator to minimize variability [1].

    - SCREEN: Show the calculation of the mean baseline Vickers hardness number from the recorded measurements.

- Immersion Solution Preparation and Immersion Procedure

  - Dissolve 25 grams of whey protein powder in 200 milliliters of distilled water to prepare immersion solutions [1].

    - Talent adding whey protein powder to distilled water in a container.

  - Mix the solutions using a laboratory shaker for 60 seconds until complete dissolution is achieved [1]. Allow the solutions to equilibrate at room temperature for 5 minutes to minimize foam formation and ensure homogeneity [2].

    - Talent placing the solution containers on a laboratory shaker.

    - Shot of the prepared whey protein solutions resting at room temperature.

  - Measure and record the pH of each solution using a calibrated digital pH meter before immersion [1].

    - Talent measuring the pH of the whey protein solution with a digital pH meter.

  - Prepare fresh whey solutions prior to each immersion cycle to ensure consistent chemical composition and avoid protein degradation [1]. Use 10 milliliters of immersion solution per specimen to ensure complete submersion during each cycle [1].

    - Shot of the prepared solutions.

    - Talent dispensing immersion solution into a container.

  - Divide each composite group into three subgroups containing 10 specimens each [1].

    - Talent organizing the composite specimens into three separate subgroups.

  - Immerse the specimens in the assigned solutions for 10 minutes twice daily [1]. Select the immersion duration and frequency based on established in vitro beverage exposure models simulating daily consumption patterns [2].

    - Talent placing the specimens into the assigned immersion solutions.

    - Shot of the immersed specimens remaining in the solutions during the exposure period.

  - For whey-exposed groups, after each immersion cycle, rinse the specimens with distilled water [1]. Place the specimens in artificial saliva for 10 minutes to simulate short-term salivary exposure [2].

    - Talent rinsing the specimens with distilled water.

    - Talent placing the specimens into artificial saliva.

  - Store the specimens in distilled water at 37 degrees Celsius between immersion cycles until the next exposure cycle. Continue the immersion protocol for 30 days [1].

    - Talent placing the specimens at 37 degrees Celsius.

  - Maintain the control group continuously in distilled water at 37 degrees Celsius throughout the experimental period [1]

    - Shot the control group specimens placed at 37 degrees Celsius .

- Post-Immersion Microhardness Measurement and Analysis

  - Rinse the specimens with distilled water and gently dry them [1].

    - Talent rinses the specimens with distilled water and gently dries them.

  - Perform VHN as demonstrated earlier and calculate the mean post-immersion VHN [1].

    - SCREEN: The mean post-immersion Vickers hardness number is being calculated from the recorded measurements.

  - Determine the change in VHN or ΔVHN (Delta-V-H-N) for each specimen [1].

    - SCREEN: The change in Vickers hardness number for each specimen is being calculated.

  - Analyze VHN values using a repeated measures ANOVA model, with time point as the within-subject factor and composite type and immersion solution as between-subject factors [1].

    - SCREEN: The VHN values are being analysed using repeated measures ANOVA with time point, composite type, and immersion solution factors.

  - When appropriate, calculate the ΔVHN values as descriptive outcome measures to express the magnitude of microhardness change for each specimen [1].

    - SCREEN: The change in Vickers hardness number values is being calculated for each specimen.

  - Perform post-hoc pairwise comparisons using Tukey’s honestly significant difference test [1].

    - SCREEN: The Tukey honestly significant difference post-hoc pairwise comparison output is being performed.

  - Calculate 95 percent confidence intervals for all group means [1]. Report effect sizes using partial eta-squared for ANOVA results [2].

    - SCREEN: 95 percent confidence intervals for all group means is being calculated

    - SCREEN: The effect sizes are being reported using partial eta-squared values in the analysis of variance results.

  - Compare hydrogen potential values between whey protein solutions using Welch’s t-test [1]. Set the level of statistical significance for the analysis [2].

    - SCREEN: The Welch’s t-test comparison of hydrogen potential values between whey protein solutions is being performed.

    - SCREEN: Show the statistical significance setting for the analysis.

  - In addition to absolute change in VHN values, calculate percentage microhardness change using the presented formula [1].

    - TEXT ON A PLAIN BACKGROUND

[(baseline VHN − post-immersion VHN) / baseline VHN] × 100.

# Results

- Results

  - The lowest microhardness change was observed in the highly filled composite stored in distilled water [1], whereas the greatest change was recorded in the injectable composite exposed to whey concentrate [2].

    - LAB MEDIA: Table 1. Video editor: Highlight the row “EA” with the column “Distilled Water” and the value “0.9 ± 0.4”

    - LAB MEDIA: Table 1. Video editor: Highlight the row “GUI” with the column “Whey Concentrate” and the value “10.4 ± 2.1”

  - Exposure to whey protein solutions resulted in greater hardness loss compared to distilled water [1]. However, no statistically significant difference was observed for whey isolate in the highly filled composite [2].

    - LAB MEDIA: Figure 1. Video editor: Highlight the lines for “WI” and “WC” positioned above “DW” across the graph

    - LAB MEDIA: Figure 1. Video editor: Highlight the blue “EA” points at “DW” and “WI” showing only a small separation

  - The injectable composite exposed to whey concentrate demonstrated the highest hardness loss [1], whereas the highly filled composite stored in distilled water exhibited the lowest change [2].

    - LAB MEDIA: Figure 2. Video editor: Highlight the tallest bar labelled “GUI-WC”

    - LAB MEDIA: Figure 2. Video editor: Highlight the shortest bar labelled “EA-DW”

  - Immersion in whey concentrate caused significantly greater microhardness loss compared to distilled water for all tested composites [1].

    - LAB MEDIA: Figure 2. Video editor: Highlight the “WC” bars as visibly taller than the corresponding “DW” bars for each composite group

  - Specimens exposed to whey isolate also showed higher ΔVHN values than those stored in distilled water [1].

    - LAB MEDIA: Figure 2. Video editor: Highlight the “WI” bars as visibly taller than the corresponding “DW” bars for each composite group