# Submission ID #: 71542

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/account/file-uploader?src=21820358

Title: X-Band Electron Paramagnetic Resonance for Monitoring of Melanin-Associated Radical Changes in Zebrafish Melanoma Xenografts In Vivo

# Authors and Affiliations:

Katerina Makarova1, Katarzyna Zawada1, Mikołaj Baranowski2,3, Malgorzata Korzeniowska née Wiweger1,4

1Department of Organic and Physical Chemistry, Faculty of Pharmacy with the Laboratory Medicine Division, Medical University of Warsaw

2Novilet

3Department of Functional Materials Physics, Institute of Physics, Faculty of Physics and Astronomy, Adam Mickiewicz University

4Laboratory of Protein Engineering, Mossakowski Medical Research Institute, Polish Academy of Sciences

# Corresponding Authors:

Katerina Makarova 				katerina.makarova@wum.edu.pl

# Email Addresses for All Authors:

Katarzyna Zawada 				katarzyna.zawada@wum.edu.pl

Mikołaj Baranowski 				mikbar@amu.edu.pl

Malgorzata Korzeniowska née Wiweger 	mkorzeniowska@imdik.pan.pl

Katerina Makarova 				katerina.makarova@wum.edu.pl

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. Yes

Current Protocol Length

Number of Steps:  22

Number of Shots:  39

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Katerina Makarova: We developed a method to monitor melanin radicals in living zebrafish embryos using X-band electron paramagnetic resonance spectroscopy.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 2.7.1.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Katerina Makarova: Current approaches cannot directly detect melanin radicals in living embryos. This protocol enables label-free monitoring using electron paramagnetic resonance.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Katerina Makarova: Using this protocol, researchers can quantify melanin-associated radicals and assess their changes during melanogenesis, tumor progression, and drug treatment.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Katerina Makarova: The key challenge while performing this protocol is consistent embryo loading and normalization, because sample position and embryo number strongly affect signal intensity.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 2.4.1.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Katerina Makarova: Future studies can use this method to evaluate novel anti-melanoma therapies and investigate mechanisms regulating melanin-associated redox biology in melanoma..

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# `Testimonial Questions (OPTIONAL):

# Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

NOTE: Shots 1.6.1 and 1.7.1 recorded twice - once according to the instructions, the second time the talent speaks directly to the camera,

What problem in your field of research does your JoVE article address?

  - Katerina Makarova, Senior Researcher, Medical University of Warsaw: (authors will present their testimonial statements live)

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What value do you expect this video article to have for other researchers working in your field?

  - Katerina Makarova, Senior Researcher, Medical University of Warsaw: (authors will present their testimonial statements live)

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Melanin Radical Detection by Electron Paramagnetic Resonance Spectroscopy

Demonstrator: Katerina Makarova

  - To begin, turn on the X-band electron paramagnetic resonance spectrometer and allow the system to stabilize according to the manufacturer's instructions [1].

    - WIDE: Talent turning on the X-band electron paramagnetic resonance spectrometer. Videographer: Please obtain CU shot as well for this shot.   NOTE: 2.1.1.A extra shot added as a close up to be placed right after 2.1.1

  - Obtain zebra fish larvae transplanted with B16F10 (B-Sixteen-F-Ten) melanoma cells [1]. Transfer 20 to 30 zebrafish larvae at 3 to 5 days post-fertilization to a 1.5-milliliter microcentrifuge tube [2]. Fill the flat capillary for zebrafish larvae with 100 microliters of E3 medium [3].

    - Shot of the container containing zebra fish larvae.  NOTE: Numbering is switched, use 2.2.2 for 2.2.1 and 2.2.1 for 2.2.2

    - Talent transferring zebrafish larvae to a 1.5-milliliter microcentrifuge tube.

    - Talent filling the flat capillary with E3 medium. NOTE: extra shot 2.2.3.1 to be put after 2.2.3

  - Seal one end of the capillary with a plastic cap and ensure that the liquid column remains confined to the flat chamber [1].

    - Talent sealing one end of the capillary with a plastic cap. NOTE: shot 2.3.1 is recorded at the end of 2.2.3

  - Transfer several larvae together with a small volume of E3 medium into a 100-microliter hematocrit capillary [1]. Remove excess liquid from the outer tubular section of the capillary while maintaining continuous liquid contact within the flat chamber [2].

    - Talent transferring several larvae with E3 medium into a 100-microliter hematocrit capillary. NOTE: extra shot 2.4.1.A to be put before 2.4.2

    - Talent removing excess liquid from the outer tubular section of the capillary.

  - Repeat the previous loading and excess liquid removal process until 20 to 30 larvae are loaded into the flat capillary [1].

    - Shot of the capillary with 20 to 30 larvae.

  - Then, remove residual liquid from the tubular sections of the capillary [1].

    - Talent removing residual liquid from the tubular sections of the capillary.

  - Insert the capillary into the electron paramagnetic resonance spectrometer [1] and position the sample at the center of the resonator [2].

    - Talent inserting the capillary into the electron paramagnetic resonance spectrometer.

    - Talent positioning the sample at the center of the resonator.

  - Set the acquisition parameters in the spectrometer software by entering a microwave frequency of 9.6 gigahertz, a center field of 335 millitesla, and a sweep width of 10 millitesla [1]. Apply a modulation amplitude of 500 microtesla and a microwave power of 1.6 milliwatts [2]. Next, set the number of scans to 32, the sweep time to 11 seconds, and the time constant to 0 [3]. Click Set parameters to transfer parameters to spectrometer, then click Tune [4].

    - SCREEN: : 71542_screenshot_1.avi: 00:07-00:20

    - SCREEN:  71542_screenshot_2.avi: 00:02-00:15

    - SCREEN: 71542_screenshot_3.avi: 00:02-00:12

    - SCREEN: 71542_screenshot_4.avi: 00:02-00:12

  - Acquire the electron paramagnetic resonance spectrum [1].

    - SCREEN: 71542_screenshot_5.avi: 00:02-00:04, 00:18-00:20

  - Then, remove the capillary from the resonator [1] and release the larvae into fresh E3 medium in a Petri dish [2].

    - Talent removing the capillary from the resonator.

    - Talent releasing the larvae into fresh E3 medium in a Petri dish.

- Melanin Radical Detection in Frozen Zebrafish Larvae

Demonstrator: Katerina Makarova

  - Thaw a frozen sample containing 10 euthanized zebrafish larvae at room temperature for 5 to 10 minutes [1]. Add 100 microliters of E3 medium to the thawed sample [2].

    - Talent handling the frozen sample and placing it at room temperature for thawing.

    - Talent adding E3 medium to the thawed sample.

  - Transfer the larvae together with the medium into a 100-microliter hematocrit capillary [1]. Remove excess liquid from the capillary until the sample occupies only the active resonator volume of about 1 centimeter in length [1]. Seal the capillary with a sealing compound [1].

    - Talent transferring the larvae and medium into a 100-microliter hematocrit capillary.

    - Talent removing excess liquid from the capillary.

    - Talent sealing the capillary with a sealing compound.

  - Insert the capillary into the electron paramagnetic resonance spectrometer [1] and position the sample at the center of the resonator [2].

    - Talent inserting the capillary into the electron paramagnetic resonance spectrometer.

    - Talent positioning the sample at the center of the resonator.

  - Acquire the spectrum using the same settings used previously and save the electron paramagnetic resonance spectrum [1].

    - SCREEN:  71542_screenshot_5.avi: 00:32-00:40

- Extracting Electron Paramagnetic Resonance Data and Post-processing of Spectra

Demonstrator: Katerina Makarova

  - Open the recorded electron paramagnetic resonance spectrum in the multiharmonic analysis software [1]. Enable dual-channel signal processing by selecting Use two channels [2].

    - SCREEN: 71542_screenshot_6.avi

    - SCREEN: 71542_screenshot_7.avi

  - Set the number of processed harmonics to 30 [1]. Adjust the Microwave phase/deg (Microwave-Phase or Deg) parameter until symmetrical absorption and dispersion components are obtained [2].

    - SCREEN: 71542_screenshot_8.avi

    - SCREEN: : 71542_screenshot_9.avi

  - Then, adjust the Modulation phase/deg parameter until Channel 1 shows a pure absorption signal [1]. Then, continue adjusting the parameter until Channel 2 displays a pure dispersion signal. [2].  NOTE: The VO is edited for the deleted shot

    - SCREEN:  71542_screenshot_10.avi

    - SCREEN: Display a pure dispersion signal in Channel 2 after adjustment of the Modulation phase/deg parameter.  NOTE: Not filmed

  - Modify the Modulation amplitude/G (Modulation-Amplitude-or-G) parameter until the narrowest undistorted spectral line is obtained [1].

    - SCREEN: 71542_screenshot_12.avi

  - Then, set the Low pass/Hz (Low pass-Or-Hertz) parameter to reduce high-frequency noise while preserving the spectral line shape [1].

    - SCREEN: : 71542_screenshot_13.avi

  - Select the Noise suppression tab and click Enable noise suppression [1]. Adjust the Smoothing factor and Narrow line refinement parameters until the signal-to-noise ratio improves without visible distortion of the spectral line [2].

    - SCREEN: 71542_screenshot_14.avi

    - SCREEN: 71542_screenshot_15.avi:

  - Next, go to the Artifacts tab. While observing the EPR spectrum, enable Outlier correction and Bandwidth correction one at a time, continuously monitoring both the noise level and the reconstructed EPR line shape [1].

    - SCREEN: 71542_screenshot_16.avi

  - Save the spectra in a format used by the data analysis software for further analysis [1]. Import the processed data into the analysis software and perform the double integration procedure [2].

    - SCREEN: 71542_screenshot_17.avi: 00:00-00:14

    - SCREEN: 71542_screenshot_18.avi: 00:19-00:23, 00:30-00:34, 00:51-00:53

# Results

- Results

  - Wild-type embryos exhibited a characteristic single-line electron paramagnetic resonance signal [1], whereas albino embryos showed no detectable electron paramagnetic resonance signal in the same spectral region at 5 days post-fertilization [2].

    - LAB MEDIA: Figure 2. Video editor: Highlight the black curve labeled “WT 5dpf” LAB MEDIA: Figure 2. Video editor: Highlight the gray curve labeled “Albino 5dpf” .

  - Analysis of wild-type embryos from 2 to 5 days post-fertilization revealed a significant increase in the double-integrated electron paramagnetic resonance signal, normalized per embryo [1].

    - LAB MEDIA: Figure 3A. Video editor: sequentially highlight the bars from left to right.

  - Raw electron paramagnetic resonance spectra from samples with 7 and 10 embryos, taken at 5 days post-fertilization, exhibited differences in signal amplitude that were proportional to the sample size [1].

    - LAB MEDIA: Figure 3B. Video editor: Highlight the 2 curves labeled “7 fish” and “10 fish” and focus on the difference in peak height.

  - After normalization to the number of embryos per sample, the spectra from samples containing 7 and 10 embryos overlapped almost completely [1].

    - LAB MEDIA: Figure 3C. Video editor: Highlight the 2 overlapping curves labeled “normalized 10 fish” and “normalized 7 fish”.

  - Electron paramagnetic resonance spectra obtained with a custom flat capillary and a standard commercially available capillary displayed highly comparable line shapes and resonance positions. [1].

    - LAB MEDIA: Figure 4A.

  - No statistically significant differences were observed between fresh and frozen samples across 6 independent biological replicates [1].

    - LAB MEDIA: Figure 4B.

  - Non-injected albino embryos showed no detectable electron paramagnetic resonance signals in the melanin-specific region [1], nor were any signals found in albino embryos injected with B16F10 cells [2].

    - LAB MEDIA: Figure 5A. Video editor: Highlight the curve labeled “Uninjected albino 4dpf”.

    - LAB MEDIA: Figure 5A. Video editor: Highlight the curve labeled “Injected B16F10 albino 4dpf”.

  - Successful cell engraftment was confirmed by bright microscopy in albino embryos injected with B16F10 cells [1].

    - LAB MEDIA: Figure 5B. Video editor: Zoom in on the lower image labeled “Implanted B16F1 cells” and highlight the area indicated by the arrow.

  - Injection into circulation provided consistent electron paramagnetic resonance signals linked to melanoma cells [1], while injection into the yolk sac yielded weak or inconsistent signals [2].

    - LAB MEDIA: Figure 6. Video editor: Highlight the solid gray curve labeled “B16F10 blood injected”.

    - LAB MEDIA: Figure 6. Video editor: Highlight the dotted curve labeled “B16F10 yolk injected”.

  - Embryos bearing B16F10 xenografts exhibited a modified electron paramagnetic resonance signal compared with wild-type embryos at corresponding developmental time points [1].

    - LAB MEDIA: Figure 7. Video editor: Compare the light bars labeled “B16F10” with the dark bars labeled “Control” at each developmental time point.

- Electron paramagnetic resonance
Pronunciation link: https://www.merriam-webster.com/dictionary/electron%20paramagnetic%20resonance
IPA: /ɪˈlek.trɑːn ˌper.ə.mæɡˈnet̬.ɪk ˈrez.ə.nəns/
Phonetic Spelling: ih·lek·tron pair·uh·mag·net·ik rez·uh·nuhns

- Spectrometer
Pronunciation link: https://www.merriam-webster.com/dictionary/spectrometer
IPA: /spekˈtrɑː.mə.t̬ɚ/
Phonetic Spelling: spek·trah·muh·ter

- Zebrafish
Pronunciation link: https://www.merriam-webster.com/dictionary/zebrafish
IPA: /ˈziː.brəˌfɪʃ/ ; /ˈzeb.rəˌfɪʃ/
Phonetic Spelling: zee·bruh·fish; zeb·ruh·fish

- B16F10
Pronunciation link: No confirmed link found
IPA: No verified IPA available
Phonetic Spelling: bee·six·teen·eff·ten

- Melanoma
Pronunciation link: https://www.merriam-webster.com/dictionary/melanoma
IPA: /ˌmel.əˈnoʊ.mə/
Phonetic Spelling: mel·uh·noh·muh

- Microcentrifuge
Pronunciation link: https://www.merriam-webster.com/dictionary/microcentrifuge
IPA: /ˌmaɪ.kroʊˈsen.trəˌfjuːdʒ/
Phonetic Spelling: my·kroh·sen·truh·fyooj

- Hematocrit
Pronunciation link: https://www.merriam-webster.com/dictionary/hematocrit
IPA: /hɪˈmæt̬.ə.krɪt/
Phonetic Spelling: hee·mat·uh·krit

- Resonator
Pronunciation link: https://www.merriam-webster.com/dictionary/resonator
IPA: /ˈrez.əˌneɪ.t̬ɚ/
Phonetic Spelling: rez·uh·nay·ter

- Gigahertz
Pronunciation link: https://www.merriam-webster.com/dictionary/gigahertz
IPA: /ˈɡɪɡ.əˌhɝːts/
Phonetic Spelling: gig·uh·hurts

- Millitesla
Pronunciation link: https://www.howtopronounce.com/millitesla
IPA: /ˌmɪl.iˈtes.lə/
Phonetic Spelling: mil·ih·tes·luh

- Microtesla
Pronunciation link: https://www.howtopronounce.com/microtesla
IPA: /ˌmaɪ.kroʊˈtes.lə/
Phonetic Spelling: my·kroh·tes·luh

- Multiharmonic
Pronunciation link: https://www.howtopronounce.com/multiharmonic
IPA: /ˌmʌl.ti.hɑːrˈmɑː.nɪk/
Phonetic Spelling: mul·tee·har·mon·ik

- Dispersion
Pronunciation link: https://www.merriam-webster.com/dictionary/dispersion
IPA: /dɪˈspɝː.ʒən/
Phonetic Spelling: dih·spur·zhuhn

- Harmonics
Pronunciation link: https://www.merriam-webster.com/dictionary/harmonic
IPA: /hɑːrˈmɑː.nɪks/
Phonetic Spelling: har·mon·iks

- Absorption
Pronunciation link: https://www.merriam-webster.com/dictionary/absorption
IPA: /əbˈzɔːrp.ʃən/
Phonetic Spelling: ub·zorp·shuhn

- Outlier
Pronunciation link: https://www.merriam-webster.com/dictionary/outlier
IPA: /ˈaʊtˌlaɪ.ɚ/
Phonetic Spelling: out·ly·er

- Biological replicates
Pronunciation link: https://www.merriam-webster.com/dictionary/replicate
IPA: /ˌbaɪ.əˈlɑː.dʒɪ.kəl ˈrep.lə.kəts/
Phonetic Spelling: bye·uh·loj·ih·kuhl rep·luh·kuhts

- Engraftment
Pronunciation link: https://www.merriam-webster.com/dictionary/engraftment
IPA: /ɪnˈɡræft.mənt/
Phonetic Spelling: en·graft·muhnt

- Xenografts
Pronunciation link: https://www.merriam-webster.com/dictionary/xenograft
IPA: /ˈziː.nəˌɡræft/
Phonetic Spelling: zee·nuh·graft

- Albino
Pronunciation link: https://www.merriam-webster.com/dictionary/albino
IPA: /ælˈbiː.noʊ/
Phonetic Spelling: al·bee·noh