# Submission ID #: 71542

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/account/file-uploader?src=21820358

Title: X-Band Electron Paramagnetic Resonance for Monitoring of Melanin-Associated Radical Changes in Zebrafish Melanoma Xenografts In Vivo

# Authors and Affiliations:

Katerina Makarova1, Katarzyna Zawada1, Mikołaj Baranowski2,3, Malgorzata Korzeniowska née Wiweger1,4

1Department of Organic and Physical Chemistry, Faculty of Pharmacy with the Laboratory Medicine Division, Medical University of Warsaw

2Novilet

3Department of Functional Materials Physics, Institute of Physics, Faculty of Physics and Astronomy, Adam Mickiewicz University

4Laboratory of Protein Engineering, Mossakowski Medical Research Institute, Polish Academy of Sciences

# Corresponding Authors:

Katerina Makarova 				katerina.makarova@wum.edu.pl

# Email Addresses for All Authors:

Katarzyna Zawada 				katarzyna.zawada@wum.edu.pl

Mikołaj Baranowski 				mikbar@amu.edu.pl

Malgorzata Korzeniowska née Wiweger 	mkorzeniowska@imdik.pan.pl

Katerina Makarova 				katerina.makarova@wum.edu.pl

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21820358

3. Filming location: Will the filming need to take place in multiple locations?   No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. Yes

Current Protocol Length

Number of Steps:  22

Number of Shots:  39

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Katerina Makarova: We developed a method to monitor melanin radicals in living zebrafish embryos using X-band electron paramagnetic resonance spectroscopy.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 2.7.1.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Katerina Makarova: Current approaches cannot directly detect melanin radicals in living embryos. This protocol enables label-free monitoring using electron paramagnetic resonance.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Katerina Makarova: Using this protocol, researchers can quantify melanin-associated radicals and assess their changes during melanogenesis, tumor progression, and drug treatment.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Katerina Makarova: The key challenge while performing this protocol is consistent embryo loading and normalization, because sample position and embryo number strongly affect signal intensity.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B roll: 2.4.1.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Katerina Makarova: Future studies can use this method to evaluate novel anti-melanoma therapies and investigate mechanisms regulating melanin-associated redox biology in melanoma..

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# `Testimonial Questions (OPTIONAL):

# Videographer: Please capture all testimonial shots in a wide-angle format with sufficient headspace, as the final videos will be rendered in a 1:1 aspect ratio. Testimonial statements will be presented live by the authors, sharing their spontaneous perspectives.

- Testimonial statements will not appear in the video but may be featured in the journal’s promotional materials.

- Provide the full name and position (e.g., Director of [Institute Name], Senior Researcher [University Name], etc.) of the author delivering the testimonial.

- Please answer the testimonial question live during the shoot, speaking naturally and in your own words in complete sentences.

What problem in your field of research does your JoVE article address?

  - Katerina Makarova, Senior Researcher, Medical University of Warsaw: (authors will present their testimonial statements live)

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What value do you expect this video article to have for other researchers working in your field?

  - Katerina Makarova, Senior Researcher, Medical University of Warsaw: (authors will present their testimonial statements live)

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Melanin Radical Detection by Electron Paramagnetic Resonance Spectroscopy

Demonstrator: Katerina Makarova

  - To begin, turn on the X-band electron paramagnetic resonance spectrometer and allow the system to stabilize according to the manufacturer's instructions [1].

    - WIDE: Talent turning on the X-band electron paramagnetic resonance spectrometer. Videographer: Please obtain CU shot as well for this shot.

  - Obtain zebra fish larvae transplanted with B16F10 (B-Sixteen-F-Ten) melanoma cells [1]. Transfer 20 to 30 zebrafish larvae at 3 to 5 days post-fertilization to a 1.5-milliliter microcentrifuge tube [2]. Fill the flat capillary for zebrafish larvae with 100 microliters of E3 medium [3].

    - Shot of the container containing zebra fish larvae.

    - Talent transferring zebrafish larvae to a 1.5-milliliter microcentrifuge tube.

    - Talent filling the flat capillary with E3 medium.

  - Seal one end of the capillary with a plastic cap and ensure that the liquid column remains confined to the flat chamber [1].

    - Talent sealing one end of the capillary with a plastic cap.

  - Transfer several larvae together with a small volume of E3 medium into a 100-microliter hematocrit capillary [1]. Remove excess liquid from the outer tubular section of the capillary while maintaining continuous liquid contact within the flat chamber [2].

    - Talent transferring several larvae with E3 medium into a 100-microliter hematocrit capillary.

    - Talent removing excess liquid from the outer tubular section of the capillary.

  - Repeat the previous loading and excess liquid removal process until 20 to 30 larvae are loaded into the flat capillary [1].

    - Shot of the capillary with 20 to 30 larvae.

  - Then, remove residual liquid from the tubular sections of the capillary [1].

    - Talent removing residual liquid from the tubular sections of the capillary.

  - Insert the capillary into the electron paramagnetic resonance spectrometer [1] and position the sample at the center of the resonator [2].

    - Talent inserting the capillary into the electron paramagnetic resonance spectrometer.

    - Talent positioning the sample at the center of the resonator.

  - Set the acquisition parameters in the spectrometer software by entering a microwave frequency of 9.6 gigahertz, a center field of 335 millitesla, and a sweep width of 10 millitesla [1]. Apply a modulation amplitude of 500 microtesla and a microwave power of 1.6 milliwatts [2]. Next, set the number of scans to 32, the sweep time to 11 seconds, and the time constant to 0 [3].

    - SCREEN: Amicrowave frequency of 9.6 gigahertz, center field of 335 millitesla, sweep width of 10 millitesla, and a modulation amplitude of 500 microtesla is being set in the spectrometer software.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=21820358

    - SCREEN: A modulation amplitude of 500 microtesla and a microwave power of 1.6 milliwatts is being entered in the spectrometer software.

    - SCREEN: The number of scans is being set to 32, sweep time is being set to 11 seconds and time constant is being set to 0 in the spectrometer software.

  - Acquire the electron paramagnetic resonance spectrum [1].

    - SCREEN: Start the acquisition and display the electron paramagnetic resonance spectrum being recorded.

  - Then, remove the capillary from the resonator [1] and release the larvae into fresh E3 medium in a Petri dish [2].

    - Talent removing the capillary from the resonator.

    - Talent releasing the larvae into fresh E3 medium in a Petri dish.

- Melanin Radical Detection in Frozen Zebrafish Larvae

Demonstrator: Katerina Makarova

  - Thaw a frozen sample containing 10 euthanized zebrafish larvae at room temperature for 5 to 10 minutes [1]. Add 100 microliters of E3 medium to the thawed sample [2].

    - Talent handling the frozen sample and placing it at room temperature for thawing.

    - Talent adding E3 medium to the thawed sample.

  - Transfer the larvae together with the medium into a 100-microliter hematocrit capillary [1]. Remove excess liquid from the capillary until the sample occupies only the active resonator volume of about 1 centimeter in length [1]. Seal the capillary with a sealing compound [1].

    - Talent transferring the larvae and medium into a 100-microliter hematocrit capillary.

    - Talent removing excess liquid from the capillary.

    - Talent sealing the capillary with a sealing compound.

  - Insert the capillary into the electron paramagnetic resonance spectrometer [1] and position the sample at the center of the resonator [2].

    - Talent inserting the capillary into the electron paramagnetic resonance spectrometer.

    - Talent positioning the sample at the center of the resonator.

  - Acquire the spectrum using the same settings used previously and save the electron paramagnetic resonance spectrum [1].

    - SCREEN: Start the acquisition and save the electron paramagnetic resonance spectrum in the software.

- Extracting Electron Paramagnetic Resonance Data and Post-processing of Spectra

Demonstrator: Katerina Makarova

  - Open the recorded electron paramagnetic resonance spectrum in the multiharmonic analysis software [1]. Enable dual-channel signal processing by selecting Use two channels [2].

    - SCREEN: The recorded electron paramagnetic resonance spectrum is ebing opened in the multiharmonic analysis software.

    - SCREEN: Select Use two channels in the multiharmonic analysis software.

  - Set the number of processed harmonics to 30 [1]. Adjust the Microwave phase/deg (Microwave-Phase or Deg) parameter until symmetrical absorption and dispersion components are obtained [2].

    - SCREEN: The number of processed harmonics is being set to 30 in the multiharmonic analysis software.

    - SCREEN: The Microwave phase/deg parameter is being adjusted while displaying symmetrical absorption and dispersion components.

  - Adjust the Modulation phase/deg parameter until Channel 1 shows a pure absorption signal [1]. Then, continue adjusting the parameter until Channel 2 displays a pure dispersion signal. [2].

    - SCREEN: The Modulation phase/deg parameter is being adjusted while displaying a pure absorption signal in Channel 1.

    - SCREEN: Display a pure dispersion signal in Channel 2 after adjustment of the Modulation phase/deg parameter.

  - Modify the Modulation amplitude/G parameter until the narrowest undistorted spectral line is obtained [1].

    - SCREEN: The Modulation amplitude/G parameter is being adjusted while displaying the spectral line becoming narrower without distortion.

  - Set the Low pass/Hz (Low pass-Or-Hertz) parameter to reduce high-frequency noise while preserving the spectral line shape [1].

    - SCREEN: The low-pass/Hz parameter is being adjusted while displaying reduced high-frequency noise and a preserved spectral line shape.

  - Select the Noise suppression tab and click Enable noise suppression [1]. Adjust the Smoothing factor and Narrow line refinement parameters until the signal-to-noise ratio improves without visible distortion of the spectral line [2].

    - SCREEN: The Noise suppression tab is being selected and enable noise suppression is being clicked.

    - SCREEN: The Smoothing factor and Narrow line refinement is being adjusted while displaying the spectrum.

  - Select the Artifacts tab and enable Outlier correction and Bandwidth correction [1].

    - SCREEN: The Artifacts tab is being selected in the software. Outlier correction and Bandwidth correction is being enabled.

  - Save the spectra in a format used by the data analysis software for further analysis [1]. Calculate the double-integrated signal intensity using numerical integration in suitable data analysis software [2].

    - SCREEN: The spectra is being saved in a compatible format for the data analysis software.

    - SCREEN: The double-integrated signal intensity is being calculated in the data analysis software.

# Results

- Results

  - Wild-type embryos exhibited a characteristic single-line electron paramagnetic resonance signal [1], whereas albino embryos showed no detectable electron paramagnetic resonance signal in the same spectral region at 5 days post-fertilization [2].

    - LAB MEDIA: Figure 2. Video editor: Highlight the black curve labeled “WT 5dpf” and the large signal peak near the center of the graph.

    - LAB MEDIA: Figure 2. Video editor: Highlight the gray curve labeled “Albino 5dpf” showing a nearly flat trace across the graph.

  - Analysis of wild-type embryos from 2 to 5 days post-fertilization revealed a significant increase in the double-integrated electron paramagnetic resonance signal, normalized per embryo [1].

    - LAB MEDIA: Figure 3A. Video editor: Emphasize the progressive increase in bar height from left to right.

  - Raw electron paramagnetic resonance spectra from samples with 7 and 10 embryos, taken at 5 days post-fertilization, exhibited differences in signal amplitude that were proportional to the sample size [1].

    - LAB MEDIA: Figure 3B. Video editor: Highlight the 2 curves labeled “7 fish” and “10 fish” and focus on the difference in peak height.

  - After normalization to the number of embryos per sample, the spectra from samples containing 7 and 10 embryos overlapped almost completely [1].

    - LAB MEDIA: Figure 3C. Video editor: Highlight the 2 overlapping curves labeled “normalized 10 fish” and “normalized 7 fish”.

  - Electron paramagnetic resonance spectra obtained with a custom flat capillary and a standard commercially available capillary displayed highly comparable line shapes and resonance positions. [1].

    - LAB MEDIA: Figure 4A.

  - No statistically significant differences were observed between fresh and frozen samples across 6 independent biological replicates [1].

    - LAB MEDIA: Figure 4B.

  - Non-injected albino embryos showed no detectable electron paramagnetic resonance signals in the melanin-specific region [1], nor were any signals found in albino embryos injected with B16F10 cells [2].

    - LAB MEDIA: Figure 5A. Video editor: Highlight the curve labeled “Uninjected albino 4dpf”.

    - LAB MEDIA: Figure 5A. Video editor: Highlight the curve labeled “Injected B16F10 albino 4dpf”.

  - Successful cell engraftment was confirmed by bright microscopy in albino embryos injected with B16F10 cells [1].

    - LAB MEDIA: Figure 5B. Video editor: Zoom in on the lower image labeled “Implanted B16F1 cells” and highlight the area indicated by the arrow.

  - Injection into circulation provided consistent electron paramagnetic resonance signals linked to melanoma cells [1], while injection into the yolk sac yielded weak or inconsistent signals [2].

    - LAB MEDIA: Figure 6. Video editor: Highlight the solid gray curve labeled “B16F10 blood injected”.

    - LAB MEDIA: Figure 6. Video editor: Highlight the dotted curve labeled “B16F10 yolk injected”.

  - Embryos bearing B16F10 xenografts exhibited a modified electron paramagnetic resonance signal compared with wild-type embryos at corresponding developmental time points [1].

    - LAB MEDIA: Figure 7. Video editor: Compare the light bars labeled “B16F10” with the dark bars labeled “Control” at each developmental time point.