# Submission ID #: 71788

# Scriptwriter Name: Pallavi Sharma

# Project Page Link: https://review.jove.com/files_upload.php?src=21895388

Title: Light- and X-ray-Activated Liposomes for Controlled Gene Editing and Drug Delivery

# Authors and Affiliations:

Yagiz Alp Aksoy1,2,3, Biyao Yang2, Gerald Liew4,5, Paul Mitchell4,5, Ewa M. Goldys2, Wei Deng2,6

1EosGene Therapeutics Pty Ltd

2School of Biomedical Engineering, University of New South Wales

3Biomedical AI Centre, Centenary Institute and Faculty of Medicine and Health, The University of Sydney

4Centre for Vision Research, Department of Ophthalmology, and Westmead Institute for Medical Research, The University of Sydney

5Department of Ophthalmology, Westmead Hospital

6School of Biomedical Engineering, Faculty of Engineering and Information Technology (FEIT), University of Technology Sydney

# Corresponding Authors:

Yagiz Alp Aksoy 		(y.aksoy@unsw.edu.au)

# Email Addresses for All Authors:

Biyao Yang 			(biyao.yang@unsw.edu.au)

Gerald Liew 			(gerald.liew@sydney.edu.au)

Paul Mitchell 			(paul.mitchell@sydney.edu.au)

Ewa M. Goldys 		(e.goldys@unsw.edu.au)

Wei Deng 			(wei.Deng@uts.edu.au)

Yagiz Alp Aksoy 		(y.aksoy@unsw.edu.au)

## Author Questionnaire

- Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?   YES

5.2.1 — zebrafish microinjection, performed under a stereomicroscope.

Videographer: Please film the scope shot using the scope kit

- Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/files_upload.php?src=21895388

- Filming location: Will the filming need to take place in multiple locations?

- Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials.

Current Protocol Length

Number of Steps:  23

Number of Shots:  39

# Interviews

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Yagiz Alp Aksoy: Our research focuses on developing light- and X-ray-activated capsules for controlled delivery of gene-editing tools and drugs.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Yagiz Alp Aksoy: Conventional gene delivery affects the whole body, whereas this protocol enables localized gene editing by activating delivery only in illuminated cells.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Yagiz Alp Aksoy: This protocol enables researchers to precisely activate and monitor gene editing in cells and living embryos.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Wei Deng: The key challenge while performing this protocol is keeping the capsules protected from light until activation, as unintended light exposure can compromise their function.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Are there any additional methods or analyses that can be performed following this procedure?

  - Wei Deng: Following the procedure, sequencing, flow cytometry, or fluorescence analysis can be used to confirm successful on-target gene editing.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Ethics Title Card

This research has been approved by the Animal Research Ethics Committee at Macquarie University

# Protocol

- Preparation of Light-Triggered VP-Liposomes for Encapsulation of CRISPR-Cas9 RNP

Demonstrator: Yagiz Aksoy

  - To begin, prepare the hydration solution by combining the single guide RNA and the Cas9 (Cas-Nine) protein in 500 microliters of nuclease-free water [1]. Pre-incubate the hydration solution at room temperature for 5 minutes to form the ribonucleoprotein or RNP (R-N-P) complex [2].

    - WIDE: Talent combining the sgRNA and the Cas9 protein in 500 microliters of nuclease-free water.

    - Talent places the tube with hydration solution in the rack and keeping it aside.

  - Hydrate the dried lipid film by adding the 500-microliter RNP solution directly onto the film [1-TXT]. Vortex vigorously for 30 seconds, then allow the suspension to stand at room temperature for 30 minutes, with intermittent vortexing every 10 minutes [2].

    - Talent adding the 500-microliter ribonucleoprotein solution directly onto the dried lipid film. TXT: Lipid Composition: DOTAP, DOPE, Cholesterol, and Verteporfin

    - Talent vortexing the hydrated lipid suspension and keeping it aside.

  - Then, extrude the multilamellar suspension 11 times through a 200-nanometer polycarbonate membrane using a mini-extruder to produce unilamellar liposomes of uniform size [1].

    - Talent extruding the multilamellar suspension through a 200-nanometer polycarbonate membrane using a mini-extruder.

  - Store the verteporfin or VP (V-P) liposomes at 4 degrees Celsius, protected from light, and use them within 48 hours of preparation for optimal activity. [1].

    - Talent placing the VP-liposomes into storage protected from light at 4 degrees Celsius.

- Physicochemical Characterization of VP-Liposomes

  - Dilute the liposome suspension at a 1 to 100 ratio in nuclease-free water [1]. Measure the hydrodynamic diameter, the polydispersity index, and the zeta potential by dynamic light scattering [2]. Perform all dynamic light scattering measurements in triplicate and report the values as the mean plus or minus the standard deviation [3].

    - Talent diluting the liposome suspension at a 1:100 ratio in nuclease-free water.

    - Talent placing the diluted liposome suspension into the dynamic light scattering instrument for measurement.

    - SCREEN: The results are being reviewed and recorded.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/files_upload.php?src=21895388

  - Deposit 10 microliters of the diluted liposome suspension onto a carbon-coated copper grid for transmission electron microscopy [1]. Stain the sample with 2 percent uranyl acetate for 1 minute [2]. Image the sample at 100 kilovolts [3] and acquire at least 10 representative images per batch [3].

    - Talent depositing 10 microliters of the diluted liposome suspension onto a carbon-coated copper grid.

    - Talent staining the grid with 2 percent uranyl acetate.

    - Talent placing the grid into the transmission electron microscope for imaging.

    - SCREEN: Acquired images of the sample.

- Light-Triggered Cargo Release and Singlet Oxygen Measurement

  - Prepare the VP-liposomes using the same lipid formulation, hydration, and extrusion protocol [1]. Dispense 100 microliters of the liposome suspension in PBS into a dialysis device suspended in 12 milliliters of PBS, stirred at 80 rpm [2].

    - Shot of the prepared VP-liposomes.

    - Talent dispensing 100 microliters of the liposome suspension in PBS into a dialysis device suspended in 12 milliliters of phosphate-buffered saline.

  - Irradiate the samples with a 690-nanometer light-emitting diode light source at 0.15 milliwatts per square centimeter for 2, 4, and 6 minutes [1].

    - Talent irradiating the samples with a 690-nanometer light-emitting diode light source.

  - Sample the external dialysate at 0, 1, 3, 6, and 24 hours [1] and quantify the released VP by fluorescence [2]. Obtain the total VP by lysing an equivalent aliquot with 0.1 percent Triton X-100 [3].

    - Talent collecting samples from the external dialysate.

    - Talent places the sample in the fluorescence spectrophotometer and note down the reading.

    - Talent lysing an equivalent aliquot with 0.1 percent Triton X-100.

  - Calculate the VP release using the presented equation [1].

    - TEXT ON A PLAIN BACKGROUND

Rvp (%) = (Ft − F0)/(Fmax − F0) × 100

Ft = Fluorescence values at time t

F0 = Baseline fluorescence

Fmax = Fluorescence after Triton lysis

  - Measure the change in the hydrodynamic diameter by dynamic light scattering before and after irradiation to confirm membrane destabilization [1].

    - Talent placing the samples into the dynamic light scattering instrument.

- In Vitro Transfection and Gene Knockout Assay

  - Seed the HEK293-GFP (H-E-K-Two-Ninety-Three) cells in 96-well plates at a density appropriate to achieve approximately 70 percent confluency at the time of treatment [1]. Culture the cells in a complete medium at 37 degrees Celsius and 5 percent carbon dioxide [2].

    - Talent seeding the HEK293-green fluorescent protein cells into a 96-well plate.

    - Talent placing the 96-well plate into the incubator.

  - Add the VP-liposomes encapsulating the anti-GFP single guide RNA and Cas9 RNP complex to the wells at the optimized concentration [1]. Incubate the cells for 4 hours to allow cellular uptake [2].

    - Talent adding the VP-liposomes to the wells of the 96-well plate.

    - Talent placing the 96-well plate into the incubator.

  - Irradiate the treated wells with a 690-nanometer LED source at 0.15 milliwatts per square centimeter for 20 minutes, including dark and light-only controls [1].

    - Talent irradiating the treated 96-well plate with a 690-nanometer light-emitting diode light source.

  - Assess the GFP knockout by fluorescence microscopy at 48 to 72 hours after treatment [1].

    - SCREEN: Samples are being imaged by fluorescence microscopy.

  - To measure encapsulation efficiency, centrifuge a sample of the liposomes at 14,000 g for 1 hour at 4 degrees Celsius [1]. Filter the supernatant through a 20-nanometer membrane [2].

    - Talent placing the liposome sample into the centrifuge.

    - Talent filtering the collected supernatant through a 20-nanometer membrane.

  - Quantify the free unencapsulated Cas9 protein in the filtrate by a BCA assay [1].

    - Talent adding the reagent to the filtrate.

  - Calculate the encapsulation efficiency as presented [1].

    - TEXT ON A PLAIN BACKGROUND

EE (%) = [(Ctot − Csup) / Ctot] × 100

Ctot = Total Cas9 added

Csup = Unencapsulated Cas9 recovered in the supernatant

- In Vivo Validation: Zebrafish Visual Reporter Assay

  - Prepare the injection Master Mix by combining VP-liposomes with sgRNA at 0.01 micromolar and Cas9 protein at 0.1 milligrams per milliliter [2].

    - Talent combining the VP-liposomes encapsulating the single guide ribonucleic acid and the Cas9 protein.

  - Microinject 2 nanoliters of the MasterMix into the cell of one-cell-stage zebrafish embryos using a calibrated microinjection system [1].

    - SCOPE: Talent microinjecting 2 nanoliters of the MasterMix into the cell of a one-cell-stage zebrafish embryo.

  - At 2 hours post-fertilization, irradiate the injected embryos with a 690-nanometer LED light source at 0.15 milliwatts per square centimeter for a defined duration to activate cargo release [1].

    - Talent irradiating the injected zebrafish embryos with a 690 nanometer light-emitting diode light source.

  - At 72 hours post-fertilization, anesthetize the embryos in Tricaine [1] and mount them in 1 percent low-melting-point agarose [2]. Quantify the slow-muscle fiber knockout as the loss of enhanced GFP fluorescence using an inverted fluorescence microscope and a confocal microscope with smyhc1:eGFP (S-M-Y-H-C-One-E-G-F-P) reporter line [3].

    - Talent anaesthetizing the embryos in Tricaine.

    - Talent mounting the embryos in 1 percent low-melting-point agarose.

    - SCREEN: The mounted embryos using an inverted fluorescence microscope and a confocal microscope.

  - Acquire focus-stacked images [1]. Count the EGFP-negative slow-muscle fibers per embryo by analyzing typically 16 embryos per group [2].

    - SCREEN: The focus-stacked images of the embryos are being acquired

    - SCREEN: Enhanced green fluorescent protein-negative slow-muscle fibers being counted.

Video editor: Please use title card to show the following information

X-ray-triggered VP-liposomes for chemotherapy drug delivery

- Prepare VP/AuNP-liposomes using DOTAP and DOPC, then incorporate VP and 3–5 nm gold nanoparticles.

- Extrude 11 times through a 200 nm membrane, then dialyze against PBS to establish an ammonium sulfate gradient.

- Load doxorubicin at a 1:10 drug-to-lipid weight ratio at 60 °C for 1 h, then remove unencapsulated drug by dialysis.

- Characterize the liposomes by DLS and determine doxorubicin encapsulation efficiency by fluorescence.

- Irradiate VP/AuNP-liposomes with 1, 2, or 4 Gy X-rays and quantify cargo release by calcein fluorescence.

- Treat HCT116 cells with doxorubicin-loaded VP/AuNP-liposomes, irradiate with X-rays, and include appropriate controls.

- Assess cell viability by MTS assay at 0, 2, 4, and 24 h after 4 Gy irradiation.

# Results

- Results

  - Light-activated embryos showed knockout of 308.37 eGFP-positive slow-muscle fibres per embryo [1], compared with 53.15 fibres in dark controls [2], increasing to 326.12 fibres after 5 minutes of irradiation [3].

    - LAB MEDIA: Figure 1B,C. Video editor: Highlight the last three panels in 1B and the bars labeled as “light +” in Figure 1C

    - LAB MEDIA: Figure 1B,C. Video editor: Highlight the first left-hand side panel in 1B and Light (-) Bar in the right-hand side plot

    - LAB MEDIA: Figure 1B. Video editor: Highlight the last panel on the right side.

  - In vitro, light-activated VP-liposomes achieved GFP knockdown comparable to Lipofectamine 2000, while uniquely providing light-gated spatiotemporal control [1].

    - LAB MEDIA: Figure 1C. Video editor: Highlight the bar labeled “light +” and +ve control

  - VP with gold nanoparticle or VP-AuNP (V-P-A-U-N-P)-liposomes showed X-ray dose-dependent doxorubicin release [1], with significant release at clinically relevant doses of 4 to 6 gray [2].

    - LAB MEDIA: Figure 2B.

    - LAB MEDIA: Figure 2B. Video editor: Highlight the data points at 4 gray.

  - Mice receiving doxorubicin-loaded VP-AuNP-liposomes followed by 4 gray X-ray irradiation showed significant tumor-growth suppression [1] compared with free doxorubicin [2], X-ray alone [3], and non-irradiated liposome controls [4].

    - LAB MEDIA: Figure 2C. Video editor: Highlight the curve labeled “X-ray triggered LipoDox”.

    - LAB MEDIA: Figure 2C. Video editor: Highlight the curve labeled “LipoDox”.

    - LAB MEDIA: Figure 2C. Video editor: Highlight the curve labeled “X-ray”.

    - LAB MEDIA: Figure 2C. Video editor: Highlight the curve labeled “PBS”.

  - Histological analysis revealed larger necrotic areas in the combined-treatment group [1].

    - LAB MEDIA: Figure 2D. Video editor: Highlight the group labeled “X-ray triggered LipoDox”.