# Submission ID #: 72146

# Scriptwriter Name: Sri Lakshmi Priya

# Project Page Link: https://review.jove.com/account/file-uploader?src=22004578

# Title: Stabilizing Protein and Protein-RNA Condensates in Low-Melting Agarose Hydrogels for Quantitative Biophysical Measurements

# Authors and Affiliations:

Nina C. Kathe, Leonidas Emmanouilidis, Mihajlo Novakovic, Frédéric H.-T. Allain

Institute of Biochemistry, ETH Zürich

# Corresponding Authors:

Nina C. Kathe 			nina.kathe@bc.biol.ethz.ch

Frédéric H.-T. Allain		allain@bc.biol.ethz.ch

# Email Addresses for All Authors:

Nina C. Kathe 			nina.kathe@bc.biol.ethz.ch

Frédéric H.-T. Allain		allain@bc.biol.ethz.ch

Leonidas Emmanouilidis	leonidas_emmanouilidis@protonmail.com

Mihajlo Novakovic		mihajlo.novakovic@bc.biol.ethz.ch

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  NO

3. Filming location: Will the filming need to take place in multiple locations?   NO

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. NO

Current Protocol Length

Number of Steps:  10

Number of Shots:  23

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Answers are slightly edited in accordance with journal guidelines. Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - N. Kathe: Our research focuses on developing a standardized agarose embedding protocol that immobilizes biomolecular condensates in three dimensions while preserving their dynamic properties.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - N. Kathe: Condensates tend to move, fuse, or stick to surfaces, which makes certain measurements difficult, whereas this agarose embedding protocol addresses these limitations.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - N. Kathe: This protocol allows researchers to image and quantify condensates over large time scales while preserving dynamic behavior.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the most important consideration or challenge when performing this protocol.

  - N. Kathe: The biggest challenge when performing this protocol is getting both the agarose temperature and the timing of the embedding right.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Are there any additional methods or analyses that can be performed following this procedure?

  - N. Kathe: Once embedded, condensates can be studied by a wide range of microscopy techniques and other biophysical methods, such as NMR.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Preparation of Agarose Stock Solution

Demonstrator: N. Kathe

  - To begin, weigh the appropriate amount of low-melting agarose or LMA (L-M-A) or regular agarose [1] and add it to a 1.5-milliliter tube containing droplet buffer [2].

    - WIDE: Talent weighing low-melting agarose or regular agarose. Videographer: Please obtain CU shot as well for this shot

    - Talent adding the agarose into the 1.5mL tube containing droplet buffer.

  - Heat the agarose stock to 80 to 90 degrees Celsius in a heating block or water bath [1] and fully dissolve the agarose by vortexing for approximately 10 minutes [2].

    - Talent placing the agarose stock into the heating block or water bath

    - Shot of vortexing the tube containing agarose.

  - Equilibrate the agarose stock to 37 degrees Celsius for low-melting agarose or 55 degrees Celsius for regular agarose in the heating block or water bath [1]. Verify that the solution remains clear and liquid at this temperature [2]. Check the temperature of the agarose stock with a thermometer [3].

    - Shot of agarose stock placed in the heating block or water bath for equilibration.

    - Shot of the agarose solution remaining clear and liquid.

    - Talent checking the temperature of the agarose stock with a thermometer.

- Preparation and Gelation of Agarose-Stabilized Condensates

  - Prewarm all materials that will come into direct contact with the agarose to approximately 37 degrees Celsius by placing them on a heating block or in an incubator for 2 to 3 minutes [1].

    - Talent placing pipette tips, tubes, and microscopy slides on a heating block or in an incubator for prewarming.

  - Prepare the agarose stock as previously demonstrated [1].

    - Talent preparing the agarose stock.

  - Gather the required reagents to [1] prepare the droplet buffer, phase-separating components, and additional additives for the samples [2]. Equilibrate all prepared solutions to room temperature [3]. Calculate the required volumes of droplet buffer, agarose stock, phase-separating components, and supplementary additives for the sample [4]. As starting values, use a final agarose concentration of 0.3 to 0.5 percent weight per volume for standard biophysical assays and 0.8 to 1.0 percent weight per volume for long-term stabilization experiments [5].

    - Talent arranging the reagents on the workbench.

    - TEXT ON A PLAIN BACKGROUND:

Buffer A (pH 7.2):

25 mM HEPES

50 mM NaCl

Nucleocapsid protein stock:

60 µM in Buffer A

Polyadenylic acid stock:

30 µM in Buffer A

    - Talent placing the prepared solutions at room temperature for equilibration.

    - TEXT ON A PLAIN BACKGROUND:

Buffer A: 6 µL

2% Agarose Stock: 5 µL (Final 0.5%)

Nucleocapsid Protein: 5 µL (Final 15 µM)

Poly(A): 4 µL (Final 6 µM)
Final Volume: 20 µL

    - Talent noting the starting final agarose concentration values for the sample.

  - Add the calculated volume of buffer into a prewarmed 1.5-milliliter tube [1]. Add the agarose stock to the same tube [2], followed by any supplementary additives [3] and then the phase-separating components [4].

    - Talent adding the calculated volume of buffer into a prewarmed 1.5 milliliter tube.

    - Talent adding the agarose stock to the tube.

    - Talent adding supplementary additives to the tube.

    - Talent adding the phase-separating components to the tube.

  - Keep the tube in the heating block or water bath [1], and pipette the mixture up and down a few times, avoiding air bubbles [2].

    - Talent placing the tube in the heating block

    - Talent pipetting the mixture up and down while the tube remains in the heating block or water bath.

  - Using a pipette, quickly transfer the sample onto the microscopy slide [1].

    - Shot of transferring the sample onto a microscopy slide using a pipette.

  - Allow the sample to gel at room temperature for 5 to 10 minutes [1]. Measure the sample [2].

    - Talent placing the microscopy slide at room temperature for gelation.

    - Talent measuring the sample according to the standard protocol.

# Results

- Results

  - Droplets formed with N protein and poly A (Poly-A) in the presence of 0.5% weight per volume low-melting agarose were roughly the same size [1] as droplets formed without low-melting agarose after 5 minutes at room temperature [2].

    - LAB MEDIA: Figure 1A,B. Video editor: In 1A, in the Cy3-poly(A) row, highlight the "no agarose" at "5 min" image. In 1B, highlight the blue bar at 5 min under the no agarose

    - LAB MEDIA: Figure 1A,B. Video editor: In 1A, in the Cy3-poly(A) row, highlight the "+ agarose" at "5 min" image. In 1B, highlight the blue bar at 5 min under the “+ agarose “

  - Sixty minutes after sample preparation, agarose-embedded droplets did not change in size [1], whereas condensates without low-melting agarose showed a drastic increase in size due to fusion and surface wetting [2].

    - LAB MEDIA: Figure 1A,B. Video editor: In 1A, in the Cy3-poly(A) row, highlight the "+ agarose" at "60 min" image. In 1B, highlight the dark grey bar at 60 min under the “+ agarose “

    - LAB MEDIA: Figure 1A,B. Video editor: In 1A, in the Cy3-poly(A) row, highlight the "no agarose" at "60 min" image. In 1B, highlight the dark grey bar at 60 min under the “no agarose “

  - Fluorescence recovery after photobleaching recovery and apparent droplet diffusion were highly similar with low-melting agarose [1] and without low-melting agarose [2].

    - LAB MEDIA: Figure 1C. Video editor: Highlight the blue recovery curve labelled "+ agarose".

    - LAB MEDIA: Figure 1C. Video editor: Highlight the grey recovery curve labelled "no agarose".

  - Diffusion rates of FUS N-terminal (fus-N-terminal) domain or FUS NTD (fus-N-T-D) condensates were not detectably influenced by agarose concentrations of 0.25 [1], 0.5 [2], and 1% weight per volume [3].

    - LAB MEDIA: Figure 1D. Video editor: Highlight the red data point above between 0.2 and 0.4 on the Agarose % axis.

    - LAB MEDIA: Figure 1D. Video editor: Highlight the red data point between 0.4 and 0.6 on the Agarose percentage axis.

    - LAB MEDIA: Figure 1D. Video editor: Highlight the red data point above "1.0" on the Agarose percentage axis.

  - N protein plus s2m RNA (N-protein-plus-S-two-M-RNA) droplets were stabilized in low-melting agarose for long enough to acquire stochastic optical reconstruction microscopy images [1].

    - LAB MEDIA: Figure 1E. Video editor: Highlight the image labelled "STORM".

  - Nuclear magnetic resonance spectra of FUS NTD droplets with agarose [1] and without agarose had comparable line widths [2].

    - LAB MEDIA: Figure 1F. Video editor: Highlight the black spectrum labelled "no agarose".

    - LAB MEDIA: Figure 1F. Video editor: Highlight the red spectrum labelled "0.5% agarose".

  - Agarose-embedded condensates showed higher signal intensity [1] than non-stabilized droplets [2].

    - LAB MEDIA: Figure 1F. Video editor: Highlight the red spectrum labelled "0.5% agarose".

    - LAB MEDIA: Figure 1F. Video editor: Highlight the black spectrum labelled "no agarose".