# Submission ID #: 72496

# Scriptwriter Name: Ruchira Mukherji

# Project Page Link: https://review.jove.com/files_upload.php?src=22111328

# Title: A Standardized Head‑Up Tilt Test Protocol to Assess Cardiovascular Autonomic Dysfunction in Neurodegenerative Diseases

# Authors and Affiliations:

# Masashi Suzuki1,2, Kenji Otsuka1, Tomoyuki Kato1, Takashi Tsuboi1, Tomohiko Nakamura3, Masahisa Katsuno1

# 1 Department of Neurology, Nagoya University Graduate School of Medicine

# 2 Department of Clinical Laboratory, Nagoya University Hospital

# 3 Department of Neurology, Hamamatsu University Hospital

# Corresponding Authors:

# Masashi Suzuki		suzuki.masashi.c9@f.mail.nagoya-u.ac.jp

# Email Addresses for All Authors:

# otsuka.kenji.r9@f.mail.nagoya-u.ac.jp

# kato.tomoyuki.t9@s.mail.nagoya-u.ac.jp

# tsuboi.takashi.z9@f.mail.nagoya-u.ac.jp

# tomohiko@hama-med.ac.jp

# katsuno.masahisa.i1@f.mail.nagoya-u.ac.jp

# suzuki.masashi.c9@f.mail.nagoya-u.ac.jp

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar? No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage? No

3. Filming location: Will the filming need to take place in multiple locations? No

4. Testimonials (optional): Would you be open to filming two short testimonial statements live during your JoVE shoot? These will not appear in your JoVE video but may be used in JoVE’s promotional materials. No

Current Protocol Length

Number of Steps: 8

Number of Shots: 23

# Interviews

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Masashi Suzuki: We aim to establish a reproducible head-up tilt test for reliable assessment of autonomic dysfunction in neurodegenerative diseases.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Masashi Suzuki: Existing head-up tilt test procedures vary across centers; this protocol standardizes assessment of autonomic dysfunction.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Masashi Suzuki: It enables reliable assessment of cardiovascular autonomic dysfunction and orthostatic hypotension in neurodegenerative diseases.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Describe the most important consideration or challenge when performing this protocol.

  - Masashi Suzuki: Controlling testing conditions, ensuring continuous blood pressure monitoring, and maintaining patient safety are critical.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Masashi Suzuki: Future studies can explore the diagnostic, prognostic, and therapeutic implications of autonomic dysfunction in neurodegenerative diseases.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Ethics Title Card

# The procedure represents a standard clinical autonomic function test. No human subjects research was conducted for the purposes of this protocol; therefore, institutional review board approval was not required under institutional guidelines. Written informed consent was obtained from participants for the recording and publication of the procedure.

# Protocol

Videographer: Please always record an audio slate while filming each shot mentioned in the shotlist.

- Equipment Set-Up and Test Environment Preparation

Demonstrator: Masashi Suzuki, Otsuka Kenji, Kato Tomoyuki

  - To begin, set up a tilt table capable of passive head-up tilt [1]. Monitor the electrocardiogram continuously, throughout the examination [2]. Also, continuously monitor beat-to-beat noninvasive blood pressure, using a volume-clamp finger-cuff device, to detect transient changes during head-up tilt [3]. Secure the cuffed hand at heart level, with a sling, and perform regular upper-arm cuff calibration to improve measurement accuracy [4].

    - WIDE: Talent setting up the tilt table in the examination room and checking its controls.

    - Shot of electrocardiogram monitoring system being set up.

    - CU: Talent placing volume-clamp finger-cuff device on the table which will detect transient changes in BP.

    - Talent placing a sling and upper-arm cuff on the table to support accurate blood pressure measurement.

  - Ensure that, appropriately trained personnel remain available throughout the examination, to manage and respond to sudden hypotension, syncope, or arrhythmias [1]. To minimize circadian variation, standardize test start time and perform serial examinations at the same time of day [2]. Maintain the room at approximately 25 degrees Celsius, while avoiding direct cold airflow towards the subject [3]. Control lighting with a windowless room or blackout curtains, and do not direct bright lights toward the subject's face [4].

    - Shot of trained examiners taking their assigned positions in the room and waiting for testing to begin.

    - Shot of talent at the computer reviewing the schedule and documenting a standardized start time for serial examinations

    - Talent adjusting the room temperature and airflow vents to avoid direct cold airflow on subjects.

    - Talent closing the blackout curtains and adjusting the lighting to avoid bright light on subject's face.

  - Ensure an adequate fasting period before testing, such as overnight fasting, fasting on the morning of the examination, or testing at least 2 hours after a meal [1]. Review all medications before testing and apply the selected medication-management approach consistently [2].

    - WIDE: The examiner confirming the subject's fasting status before testing.

    - Shot of talent at the computer reviewing the subject's medication list and documenting the medication plan for the test.

- Hemodynamic Stabilization Before Tilt-up

  - Position the subject supine on the tilt table at zero degrees [1] and apply safety belts [2]. Initiate continuous blood pressure monitoring by securing the finger-cuff arm at heart level [3]. Apply continuous ECG (e-c-g) monitoring [4]. Maintain the supine position for at least 10 minutes to confirm whether blood pressure and heart rate have stabilized before tilt-up [5].

    - WIDE: Talent assisting the subject into the supine position on the tilt table.

    - Talent securing the subject firmly on the table using safety belts to prevent falls.

    - CU: Talent placing the finger cuff, securing the recording arm at heart level, to initiate BP monitoring.

    - Talent applying the ECG electrodes on the subject and confirming a stable signal.

    - Shot of talent keeping the subject supine till heartrate and blood pressure monitor show stable values.

- Tilt Maneuver and Post-Tilt Hemodynamic Assessment

  - Raise the tilt table smoothly, to 60 to 70 degrees, within approximately 10 to 30 seconds [1] while continuously recording blood pressure and heart rate [2]. Maintain the upright position for at least 3 minutes to assess classic orthostatic hypotension [3 -TXT] and approximately 5 minutes for routine assessment [4].

    - WIDE: Talent raising the tilt table slowly to 60 to 70 degrees taking up to 15 seconds.

    - CU: Talent monitoring patient’s blood pressure and heartrate during tilting using the on-screen values.

    - Talent maintaining the patient in upright position for 3 minutes while monitoring blood pressure values to detect OH. TXT - Monitor up to 40 min for delayed OH and 10 min for POTS.

    - Talent maintaining the subject in upright position for 5 minutes while monitoring blood pressure values for routine assessment.

- Test Termination and Post-Test Precautions

  - After the predetermined upright period, return the subject safely to the supine position [1]. Confirm hemodynamic recovery by checking stabilization of blood pressure, and heart rate, before examination is complete [2]. Monitor the subject carefully during transfer from the tilt table [3]. After the test, observe for hypotension, dizziness, or falls, exercising caution in patients with neurodegenerative diseases and subjects that experienced a blood pressure reduction during head-up tilt [4].

    - Talent slowly turning the tilt-table to return the subject to supine position.

    - Talent checking subject’s blood pressure and heartrate to monitor hemodynamic recovery.

    - Talent assisting and monitoring the subject while being transferred from tilt-table.

    - Shot of talent closely monitoring subjects and patients in the examination area for post-test hypotension, dizziness, or fall risk.

# Results

- Results

  - Representative patterns of blood pressure and heart rate responses observed in a healthy subject during the head-up tilt test are shown [1]. BP (bee-pee) and HR (aych-arr) fluctuation was seen during the early resting period [2]. Tilt-ups started only after BP and HR stabilized [3]. Immediately after tilt-up, SBP (ess-bee-pee) remained unchanged or transiently decreased, followed by compensatory increase in HR and SBP which ultimately stabilized slightly above baseline [4]. DBP (dee-bee-pee) remained unchanged or increased mildly [5].

    - LAB MEDIA: Figure 3

    - LAB MEDIA: Figure 3 Video Editor: Please highlight section A across both panels and all three traces labelled -SBP, DBP, and HR

    - LAB MEDIA: Figure 3 Video Editor: Please highlight to section B in both panels and all three traces labelled -SBP, DBP, and HR.

    - LAB MEDIA: Figure 3 Video Editor: Please highlight section C and D in only two traces labelled SBP and HR.

    - LAB MEDIA: Figure 3 Video Editor: Please highlight section C and D only in the trace labelled DBP.

  - Representative SBP and HR responses were measured and plotted for initial OH (oh-aych) [1], classic OH [2], delayed OH [3], and POTS (potts) which is characterized by an excessive HR increase without a significant BP decrease [4]. The dashed lines indicate diagnostic thresholds of minus 20 millimeters of mercury for SBP and plus 30 beats per minute for HR [5].

    - LAB MEDIA: Figure 4 Video Editor: Please highlight the section of the graph labelled initial OH in both traces labelled SBP and HR.

    - LAB MEDIA: Figure 4 Video Editor: Please highlight the section of the graph labelled classic OH in both traces labelled SBP and HR.

    - LAB MEDIA: Figure 4 Video Editor: Please highlight the section of the graph labelled delayed OH only in the SBP trace.

    - LAB MEDIA: Figure 4 Video Editor: Please highlight the section of the graph labelled POTS only in the HR trace.

    - LAB MEDIA: Figure 4 Video Editor: Please highlight the section of the graph showing dashed red lines with red down arrow for ‘−20 mmHg’ for SBP and dashed red line with red up arrow for ‘+30 bpm’ for HR, at the same time.