# Submission ID #: 73347

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=22370883

# Title: An In Vivo Murine Simulated Seaside System for Assessing Sunscreen Efficacy and Durability

# Authors and Affiliations:

Jingheng Huang1*, Qiujuan Ma2*, Kaihong Chen3*, Rong Yan4, Chulin Zhang4, Yuwen Zhao4, Chunyan Chen4, Zhangwen Peng5

1 Guangdong Country Garden School

2 Department of Infectious Diseases, The First People's Hospital of Foshan, School of Medicine, Southern University of Science and Technology

3 Majestic International College

4 Foshan Key Laboratory of Precision Therapy in Oncology and Neurology, The First People's Hospital of Foshan, School of Medicine, Southern University of Science and Technology

5 Ultrasound Diagnosis and Treatment Center, The First People's Hospital of Foshan, School of Medicine, Southern University of Science and Technology

*These authors contributed equally

# Corresponding Authors:

Chunyan Chen  18038866327@163.com

Zhangwen Peng  pzwsysu@126.com

# Email Addresses for All Authors:

Jingheng Huang  hjheng0058@126.com

Qiujuan Ma  2460711416@qq.com

Kaihong Chen  irenejj27@126.com

Rong Yan  1035307216@qq.com

Chulin Zhang  chuling_cheung@163.com

Yuwen Zhao  542612440@qq.com

Chunyan Chen  18038866327@163.com

Zhangwen Peng  pzwsysu@126.com

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://review.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

As these files are necessary for finalizing your script, please upload all screen captured video files to your project page as soon as possible.

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 10/20/2026

When you are ready to submit your video files, please contact our China Location Producer, Junling Cheng.

Current Protocol Length

Number of Steps: 35

Number of Shots: 48 (4 SC)

# Interviews

Authors: Please note that the questions will not appear on screen. Please answer in stand-alone full sentence with sufficient context.

INTRODUCTION:

REQUIRED: Summarize your research focus and the main questions it aims to answer.

  - Jingheng Huang: We developed a simulated seaside murine system to evaluate sunscreen retention and efficacy under complex environmental stressors.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the research gap or limitation in existing methods and how this protocol addresses them.

  - Jingheng Huang: Existing methods neglect real-world marine dynamics. We address this by testing sunscreens under combined wave, wind, and UV exposure.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

CONCLUSION:

What does this protocol allow researchers to study or measure?

  - Kaihong Chen: This protocol allows researchers to dynamically evaluate sunscreen durability and UV protection in a realistic marine environment.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Describe the most important consideration or challenge when performing this protocol.

  - Kaihong Chen: A key challenge when performing this protocol is to constantly observe the mice to ensure they stay within the designated testing area.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

How can future studies build on this work? Describe the questions or research directions that can be investigated next.

  - Kaihong Chen: Future studies can adapt this system to evaluate barrier creams, wound dressings, and UV-protective textiles.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

Ethics Title Card

This research has been approved by the Laboratory Animal Care and Ethics Committee at Guangdong Laidi Biomedical Research Institute

# Protocol

- Setup of the Simulated Seaside Chamber

Demonstrator: Click here to enter name of demonstrator(s)

  - To begin, assemble the test chamber, a transparent, airtight metal box measuring 40 by 30 by 30 centimeters, with access ports for the animals [1-TXT].

    - WIDE: Talent assembling the 40 x 30 x 30 cm transparent airtight test chamber. TXT: Refer to the text protocol for the chamber parameters

  - Install the 6-watt ultraviolet lamps at the top of the chamber, providing an intensity of about 2 milliwatts per square centimeter to simulate the average strong midday seaside sunlight [1].

    - Talent installing the 6 W UV lamps at the top of the chamber, at ~2 mW/cm².

Authors: Please ensure that safety goggles and protective clothing will be worn and visible while the ultraviolet unit is operated.

  - Prepare the artificial seawater by dissolving commercial sea salt in purified water, at 1 kilogram of salt to 30 liters of water [1-TXT]. Then, fill the built-in water tank with the seawater [2]. Set the water temperature controller to maintain a constant 25 degrees Celsius [3]. Activate the wave-maker at a flow rate of about 10 liters per minute, to simulate the mechanical friction of moderate shore waves [1].

    - Close-up of Talent dissolving 1 kg of sea salt in 30 L of purified water.

    - Talent filling the built-in water tank with the artificial seawater.

    - Shot of the water temperature controller being set to a constant 25 °C.

    - Talent activating the wave-maker at ~10 L/min in the water tank.

  - Position the blower fan to direct the airflow across the animal holding area, and calibrate the airflow to 35 cubic feet per minute, to simulate a seaside breeze [1].

    - Talent positioning the blower fan and calibrating the airflow to 35 CFM across the holding area.

  - Connect all the components to the central control interface, and input the target environmental parameters [1]. Then, verify them with a handheld ultraviolet radiometer, a waterproof thermometer, and an anemometer [2].

    - SCREEN: The target UV intensity, water temperature, and wind speed being entered on the central control interface.

    - Close-up of Talent verifying the UV intensity, the water temperature, and the wind speed with a handheld UV radiometer, a waterproof thermometer, and an anemometer.

Authors: Please create screen capture videos of the shots labeled as SCREEN or upload representative images to be shown:  https://review.jove.com/account/file-uploader?src=22370883

  - During the experiment, toggle the ultraviolet lamps, the wave-maker, and the water drainage on or off from the interface to execute the alternating immersion and emersion phases precisely [1].

    - Shot of the UV lamp, wave-maker, and drainage toggles being switched on and off at the control interface.

- Preparation of the Animal

Demonstrator: Click here to enter name of demonstrator(s)

NOTE to Authors: Please consider the following key points for shots involving live

animals/survival surgery (as applicable) to avoid raising concerns by JoVE’s veterinary

reviewers.

- If lifting the animal by the tail, hold it close to the tail base, not far from it.

- Shave 150% of the area surrounding a surgical site (preferably using a depilatory

- cream). Ensure that there are no skin injuries and no fur in the surgical field.

- Prep the surgical site with both iodine-based scrubs and alcohol.

- Use sterile gloves only for surgery (not examination or non-sterile gloves).

- Make skin incisions with a surgical blade, not scissors.

- Avoid using silk thread for closing skin incisions in survival surgeries.

- Please point out the organs before dissecting

  - House an adult female BALB/c (balb-C) mouse weighing about 20 to 25 grams in standard conditions, and allow it to acclimatize for one week before the experiment [1]. After anesthetizing the animal, shave the fur on the back [2-TXT].

    - Talent placing the BALB/c mouse into a standard housing cage.

    - Close-up of Talent pre-shaving the dorsal fur of the mouse with the hair removal device. TXT: Anesthesia: 1.5% Isoflurane

  - Apply a commercial chemical depilatory cream evenly to the dorsal skin [1]. Then, after 3 to 5 minutes, gently wipe the hair away with a warm, damp gauze [2].

    - Close-up of Talent applying the chemical depilatory cream evenly to the dorsal skin.

    - Close-up of Talent wiping the hair away with warm, damp gauze after 3 - 5 min.

  - House the depilated mouse for a further 2 days to allow recovery [1]. Then, inspect the dorsal skin to confirm the complete absence of erythema or irritation from the depilation [2]. Draw a cross on the exposed dorsal skin using a black marker to divide the area into 4 distinct quadrants [1].

    - Talent returning the depilated mouse to its cage for the 2 day recovery.

    - Close-up of the dorsal skin being inspected for erythema or irritation before the experiment.

    - Close-up of Talent drawing a cross on the dorsal skin with a black marker to mark the 4 quadrants.

- Application of the Sunscreens

Demonstrator: Click here to enter name of demonstrator(s)

  - Randomly select 3 commercial sunscreens, A, B, and C, with a sun protection factor of 50 or more [1].

    - Close-up of Talent laying out the 3 selected commercial sunscreens, A, B, and C, on the bench.

Authors: Please confirm the sun protection factor and the filter type of each of the 3 sunscreens

Reply: All three sunscreens (A, B, and C) are standard commercially available sunscreens with an SPF of 50+.

Authors: Is the animal anesthetized for sunscreen application and imaging?

Reply: Yes, with 1.5% Isoflurane to avoid movement

  - After anesthetizing the mouse, place it on a clean surface [1-TXT]. Using a 20 microliter micropipette, apply 20 milligrams per square centimeter of Sunscreen A to Quadrant 1 of the prepared skin site [2]. Then, spread the formulation evenly with a sterile gloved fingertip, in a gentle circular motion for 10 seconds [3].

    - Talent placing the anesthetized mouse on a clean surface. TXT: Anesthesia: 1.5% Isoflurane

    - Close-up of Talent applying 20 mg/cm² of Sunscreen A to Quadrant 1 with a 20 µL micropipette.

    - Close-up of Talent spreading Sunscreen A with a sterile gloved fingertip for 10 s.

  - Apply Sunscreen B to Quadrant 2, at the top right, and Sunscreen C to Quadrant 3, at the bottom left, in the same way [1]. Leave Quadrant 4, at the bottom right, untreated as the unprotected control [2].

    - Close-up of Talent applying Sunscreen B to the top-right quadrant and Sunscreen C to the bottom-left quadrant.

    - Close-up of the untreated bottom-right quadrant left bare as the unprotected control.

  - Finally, place the mouse in a resting position for 5 minutes, so that the sunscreen distributes evenly and stably on the dorsal skin [1].

    - Talent resting the mouse for 5 min to let the sunscreen settle on the dorsal skin.

- Assessment of the Sunscreen Retention

Demonstrator: Click here to enter name of demonstrator(s)

  - Mix the fluorescent marker Sulfo Cy7 (sul-fo-sigh-7), at 0.01 percent by weight, into Sunscreen A, Sunscreen B, and Sunscreen C [1]. Then, apply the labeled sunscreens to the freshly prepared dorsal skin of a previously untreated mouse [2-TXT].

    - Close-up of Talent mixing 0.01% w/w Sulfo Cy7 into each of the 3 sunscreens.

    - Close-up of Talent applying the fluorescently labeled sunscreens to the 3 marked quadrants. TXT: Anesthesia: 1.5% Isoflurane

Authors: The Protocol never states the total number of animals or whether Sections 4 and 5 use the same mice. Step 5.1 reapplies plain sunscreen to backs that already carry the fluorescent sunscreen from Section 4, with no washing step in between. Please confirm the cohorts, and whether the skin is cleaned between the 2 experiments.

Response: Sections 4 and 5 use two separate cohorts of mice. Section 4 uses the first cohort for fluorescence evaluation, and Section 5 uses a new, previously untreated cohort of mice for the photoprotective efficacy and histological evaluation

  - Capture the baseline fluorescence image of the dorsal skin using an in vivo fluorescence imaging system [1].

    - SCREEN/IMAGE: The baseline F0 fluorescence image of the dorsal skin being captured in the in vivo imaging software.

  - Place the mouse into the testing chamber, and turn off the ultraviolet lamps [1]. Then, set the water temperature to 25 degrees Celsius [2].

    - WIDE: Talent placing the mouse into the testing chamber with the UV lamps switched off.

    - Shot of the water temperature being set to 25 °C at the control interface.

  - Raise the water level and activate the wave-maker, so that the mouse undergoes a 30-minute immersion phase, keeping the head safely above the water surface [1-TXT].

    - Talent raising the water level and starting the wave-maker for the 30 min immersion phase. TXT: Cycle: 30 min immersion + 30 min emersion, x 2 (120 min total)  Authors: Please check if this text overlay is corrected placed here

  - Then, turn off the wave-maker and lower the water level to expose the stepped metal platform, allowing the mouse to rest on it for 30 minutes [1]. Repeat the immersion and the emersion phases for a second cycle, giving 120 minutes in total [2].

    - Talent lowering the water level to expose the stepped metal platform for the 30 min emersion phase.

    - Talent starting the second immersion and emersion cycle in the chamber.

  - Capture the post-exposure fluorescence image using the in vivo imaging system [1].

    - SCREEN/IMAGE: The post-exposure F1 fluorescence image of the dorsal skin being captured in the in vivo imaging software.

  - Finally, calculate the retention level using the equations [1].

    - TEXT ON PLAIN BACKGROUND:

Retention Level=F1F0×100%

Relative Fluorescence Intensity=F1XF1A×100%

F0= Baseline fluorescence intensity
F1= Post-experiment fluorescence intensity
F1A= Post-experiment fluorescence intensity of the area with Sunscreen A
F1X= Post-experiment fluorescence intensity of the area with Sunscreen B or C

- Detection of the Photoprotective Efficacy

Demonstrator: Click here to enter name of demonstrator(s)

  - Apply the standard, non-fluorescent sunscreens to the back of the anesthetized mouse as demonstrated earlier [1-TXT].

    - Close-up of Talent applying the 3 standard sunscreens to the marked quadrants. TXT: Anesthesia: 1.5% Isoflurane

  - Apply a piece of black, ultraviolet-blocking tape to the upper neck area to serve as an ultraviolet-shielded internal control [1]. Then, record the baseline skin condition by macroscopic photography [2].

    - Close-up of Talent pressing the black UV-blocking tape firmly onto the upper neck of the mouse.

    - Close-up of the baseline dorsal skin being photographed before the exposure.

  - Expose the mouse to the same water cycling program, this time with the ultraviolet simulation unit activated, for 2 cycles of 30-minute immersion and 30-minute emersion [1].

    - Talent starting the 120 min cycling program with the UV simulation unit switched on.

  - Remove the mouse from the chamber, and record the erythema, peeling, or hardening of the skin by macroscopic photography [1].

    - Close-up of the dorsal skin being photographed after the exposure, showing the erythema in the untreated quadrant.

  - Finally, at the experimental endpoint, euthanize the mouse by cervical dislocation under deep isoflurane anesthesia [1]. Then, excise the dorsal skin from the 4 quadrants and from the tape-protected area [2].

    - Talent placing the euthanized animal on the table.

    - Close-up of Talent excising the dorsal skin from the 4 quadrants and the tape-protected area.

- Histology and Analysis of the Skin Sections

Demonstrator: Click here to enter name of demonstrator(s)

  - Fix the tissues in 4 percent paraformaldehyde for 24 hours [1]. After paraffin embedding, cut them into 4-micrometer thick sections following standard protocols [2].

    - Close-up of Talent transferring the excised skin into 4% paraformaldehyde for 24 h.

    - Close-up of Talent dehydrating and embedding the tissue, then cutting the paraffin sections on the microtome.

Authors: Please give the section thickness.

Reply: The paraffin sections were cut at a thickness of 4 μm.

  - Then, deparaffinize the sections in xylene, and rehydrate them through graded ethanol solutions [1].

    - Talent immersing the slide rack in xylene and then in the graded ethanol series.

  - Stain the sections with hematoxylin for 5 minutes, rinse them in running tap water, and counterstain with eosin for 1 minute [1]. Then, dehydrate and mount them with a resinous mounting medium [2].

    - Close-up of Talent staining the sections with hematoxylin.

    - Close-up of Talent mounting the stained sections with a resinous mounting medium.

  - Finally, observe the stained sections under a light microscope, measure the thickness of the epidermal thickness, and count the infiltrating inflammatory cells [1].

    - SCREEN/LAB MEDIA: Show the stained skin section images and show measuring the thickness of the epidermis.

Authors: Step 5.8 measures the thickness of the stratum corneum, but Figure 4B plots the relative epidermal thickness, and the Figure 4 legend itself says the indicator lines mark the stratum corneum in the sentence before it calls panel B epidermal thickness. Please confirm which layer was measured, so that the narration and the legend match the figure.

Reply: We measured the entire epidermal thickness. Please use "epidermal thickness" in the narration to match Figure 4B. The text in step 5.8 of the manuscript ("stratum corneum") was a slight terminology oversight.

# Results

- Results

  - After the 120 minute cycle, the fluorescence of every sunscreen group fell [1], showing that the seawater waves washed a large amount of sunscreen off the skin [2].

    - LAB MEDIA: Figure 2A. Video editor: Please emphasize the 0 h row and then the 2 h row to show the loss of signal without deleting other parts

    - LAB MEDIA: Figure 2B. Video editor: Please emphasize the retention level values of all 3 groups without deleting other parts

  - Relative to Sunscreen A, Sunscreen C retained significantly more fluorescence [1].

    - LAB MEDIA: Figure 2C. Video editor: Please emphasize the Sunscreen C points and the significance bars above them without deleting other parts

  - The unprotected quadrant showed pronounced erythema and sunburn, while the sunscreen-treated regions were markedly less affected [1].

    - LAB MEDIA: Figure 3A. Video editor: Please show the Pre-exposure row and then the Post-exposure row, emphasizing the reddening that appears after the exposure, without deleting other parts

  - Histologically, the unprotected skin showed marked thickening and inflammatory cell infiltration [1], which the sunscreens reduced [2].

    - LAB MEDIA: Figure 4A. Video editor: Please emphasize the No protection panel against the Normal skin panel, showing the indicator lines, without deleting other parts

    - LAB MEDIA: Figure 4B and Figure 4C. Video editor: Please emphasize the None column and its significance bars in both panels without deleting other parts