| Video filename | Script shot(s) | Screen-capture description | Timecode |
| --- | --- | --- | --- |
| 73521_screenshot_1.mp4 | 2.1.2–2.1.4 | Open Global Settings → Audio & Video; enable Data Collection and Forward Samples; leave Multiface Detection, Face ID, and Recognize Faces unselected; select Baseline. | 00:01–00:23 |
| 73521_screenshot_2.mp4 | 2.2.1–2.2.5 | Set facial sampling to 30 Hz; select Emotions, Prosody, and Speaker Attributes; set voice sampling to 11 Hz; set speech segment minimum to 1 s, maximum to 10 s, and end delay to 0.25 s; save settings. | 00:00–00:37 |
| 73521_screenshot_3.mp4 | 2.3.1–2.3.4 | Open sensor settings; select GSR; set sampling to 128 Hz; select heart rate and enable Shimmer data collection with accelerometer unselected; confirm sensor connections and connection quality. | 00:00–00:30 |
| 73521_screenshot_4.mp4 | 3.2.2 | Present the welcome page. | 00:02–00:18 |
| 73521_screenshot_5.mp4 | 3.3.2 | Present the no-sound instruction screen followed by the mock-test film-clip instruction screen. | 00:00–00:08 |
| 73521_screenshot_6.mp4 | 3.4.1 | Present the picture containing the film-clip icon. | 00:00–00:04 |
| 73521_screenshot_7.mp4 | 3.4.3 | Review the test recording for facial visibility, audio clarity, and physiological signal quality. | 00:00–00:40 |
| 73521_screenshot_8.mp4 | 3.5.1 | Document corrections in the session log and enter room light level, temperature, and relative humidity in the comment box. | 00:00–00:30 |
| 73521_screenshot_9.mp4 | 4.3.1–4.3.2 | Advance to the next stimulus after the participant has read all three sentences aloud. | 00:00–00:34 |
| 73521_screenshot_10.mp4 | 4.3.3 | Monitor facial-expression, GSR, heart-rate, and microphone recordings during the clip. | 00:00–00:20 |
| 73521_screenshot_11.mp4 | 4.4.3 | Document technical issues, pauses, participant discomfort, incomplete tasks, or possible artifact sources after Study Flow 1. | 00:00–00:26 |
| 73521_screenshot_12.mp4 | 5.1.2 | Check GSR, PPG, facial-expression, and microphone recordings before Study Flow 2. | 00:03–00:21 |
| 73521_screenshot_13.mp4 | 5.3.1 | Start Study Flow 2 recording and show continuous GSR and PPG recording. | 00:03–00:21 |
| 73521_screenshot_14.mp4 | 5.3.3 | Show synchronized physiological and audio data streams during the spoken description. | 00:00–00:16 |
| 73521_screenshot_15.mp4 | 5.4.1 | Document technical issues, pauses, participant discomfort, incomplete tasks, or possible artifact sources during Study Flow 2. | 00:00–00:22 |
| 73521_screenshot_16.mp4 | 5.5.1 | Generate representative facial-expression, GSR, and vocal-valence outputs for individual film clips. | 00:00–00:45 |