# Lab Manual: Phy101

# Lab: Fluid Flow

# Scriptwriter Name: Linda Wangoh

- A fluid is any material such as liquid or gas [1] that continuously changes when a shearing force is applied [2]. They can travel in a steady flow [3], where the fluid passing a given point [4] maintains a constant velocity [5]. They can also travel in a turbulent flow [6], where the speed and the direction of the flow vary at a given point [7].

  - See Storyboard

- Fluids can be compressible or incompressible [1]. Compressible fluids, like gases [2], will change volume in response to a change in pressure [3]. Incompressible fluids, like liquids [4], do not change volume in response to a pressure change [5]. A fluid can also be viscous, meaning pours slowly [6], or non-viscous, meaning pours easily at the same temperature [7].

  - See Storyboard

- Fluid dynamics is the study of how fluids behave [1] when in motion [2] or when forces are exerted upon them [3]. One way to make fluid flow through a pipe is to tilt the pipe so the flow travels downhill [4]. The second way is to make the pressure at one end of the pipe greater than the pressure at the other end [5].

  - See Storyboard

- One of the fundamental equations in fluid flow states that for an incompressible fluid flowing in a tube of varying cross-section [1], the mass flow rate is the same everywhere in the tube [2]. In other words, it relates the speed of the fluid through one cross-sectional area [3] to the speed of the fluid through a different cross-sectional area [4].

  - See Storyboard

- The relationship between pressure, velocity and height of a fluid at one point [1] to the same parameters at a second point [2] is quantitively described by Bernoulli's equation [3].

  - See Storyboard

- The Bernoulli's equation can be simplified if we assume the flow is horizontal thus we do not need to consider the gravity term [1]. We can then combine this equation with the continuity equation [2] to determine the velocity of the fluid through a cross-section due to a change in pressure between two given points [3].

  - See Storyboard

- The Venturi effect applies Bernoulli's principle to a fluid that flows through a tube with a constriction in it [1]. In a Venturi tube, the pressure in the smaller cross-sectional area [2] should be lower than it is outside that region [3].

  - See Storyboard

- Based on Bernoulli's principle, high pressure before the narrowing accelerates the fluid into the low-pressure region of the smaller cross-sectional area. [1] Then, high pressure after the smaller cross-sectional area slows the fluid down again as it reaches a greater cross-sectional area to flow through [2].

  - See Storyboard

- The Bernoulli's principle is useful in explaining how airplanes fly. The wing of an airplane curved on top and completely flat underneath [1]. In flight, the air travels across the top and the bottom [2]. Due to the design, the air at the bottom has a lower velocity, which creates more pressure than at the top [3]. This effect creates lift and allows the aircraft to fly [4].

  - See Storyboard

- In this lab, we will use a Venturi tube to determine the velocity of the fluid through a cross-section due to a change in pressure between two given points [1]. The experimental value of pressure will be measured using a precision manometer [2].

  - Re-use shot 2.2.7

  - Re-use shot 3.1.2

Learning Objectives:

- What is a fluid?

- What is fluid dynamics?

- What is Bernoulli's principle?

- What is the equation of continuity?

- What is the Venturi effect?

Take-Homes:

- What is a fluid?

A fluid is any liquid, gas, or material that cannot sustain a shearing force when at rest and that continuously changes when such a stress is applied.

- What is fluid dynamic?

Fluid dynamics is the study of how fluids behave when in motion or when forces are exerted upon them.

- What is Bernoulli's principle?

It is the principle of energy conservation for ideal fluids in steady flow. It relates the pressure, velocity, and height of a fluid at one point to the same parameters at a second point.

- What is the equation of continuity?

This equation relates the speed of the fluid through different cross-sectional areas. It states that for an incompressible fluid flowing in a tube of varying cross-section, the mass flow rate is the same everywhere in the tube.

- What is the Venturi effect?

The Venturi effect applies Bernoulli's principle to a fluid that flows through a tube with a constriction in it.