JoVE Article Template:

TITLE  (maximum 90 characters)

Electrophysiological methods for recording synaptic potentials from the NMJ of Drosophila larvae.

Authors: Wendy L. Imlach, Brian D. McCabe.

Authors: institution(s)/affiliation(s) for each author:

Wendy L. Imlach

Department of Physiology and Cellular Biophysics

Columbia University Medical Center

College of Physicians and Surgeons, 11‐420

630W 168th Street,

New York, NY 10032, USA.

wi2103@columbia.edu

Brian D. McCabe

Department of Physiology and Cellular Biophysics

Columbia University Medical Center

College of Physicians and Surgeons, 11‐420

630W 168th Street,

New York, NY 10032, USA.

bm2157@columbia.edu

Corresponding author:

Wendy L. Imlach

wi2103@columbia.edu

Keywords: Neuromuscular junction, synaptic transmission, Drosophila larvae, electrophysiology

Short Abstract: should include a general description of the article (10 word minimum, 50 word

maximum)

Here we describe the electrophysiological methods for measuring synaptic transmission through the

neuromuscular junction in Drosophila larva. Signals are initiated by stimulating the motor neuron and

transmission through the NMJ is measured by the postsynaptic response evoked in the muscle.

Long Abstract: (150 word minimum, 400 word maximum)

In this video, we describe the electrophysiological methods for recording synaptic transmission through

the neuromuscular junction (NMJ) of Drosophila larvae. The larval neuromuscular system is a model

synapse for the study of synaptic physiology and neurotransmission and is a valuable research tool that

has defined genetics and is accessible to experimental manipulation. Larvae can be dissected to expose

the body wall musculature, central nervous system, and peripheral nerves. The Drosophila muscles and

innervation are well characterized and the muscles are large and easy to access for intracellular

recording. Individual muscles can be identified by their location and orientation within the 8 abdominal

segments, each with 30 muscles arranged in a pattern that is repeated in segments A2 ‐A7. Dissected

drosophila larvae are thin and individual muscles and bundles of motor neuron axons can be visualized

using a light microscope (Atwood et al., 1993). Transgenic constructs can be used to label target cells for

visual identification or for manipulating gene products in specific tissues. In the larvae, excitatory

junction potentials (EJP’s) are generated in response to vesicular release of glutamate from the

motoneurons at the synapse. Action potentials can be artificially evoked in the peripheral neurons that

have been cut posterior to the ventral ganglion, drawn into a glass pipette by gentle suction and

stimulated with an electrode. These motor neurons have distinct firing thresholds when stimulated, and

when they fire simultaneously, they generate a response in the muscle. Signals transmitted across the

NMJ synapse can be recorded in the muscles that the motor neurons innervate. The EJP’s and minature

excitatory junction potentials (mEJP’s) are seen as changes in membrane potential. Electrophysiological

responses are recorded at room temperature in standard hemolymph‐like solution (HL3) (Stewart et al.,

1994) with 0.4 – 1.5 mM Ca

2+

.  Changes in the amplitude of evoked EJP’s can indicate differences in

synaptic function and structure. Digitized recordings are analyzed for EJP amplitude and mEJP frequency

and amplitude.

Before starting prepare:

Third instar wandering Drosophila larvae

HL3 (Standard hemolymph‐like) solution

Sylgard (transparent silicone rubber) dissection plates

Cut dissection pins short

Stimulating electrode pipettes

Sharp recording pipettes

HL3 Solution:

1.During dissections and electrophysiological experiments, larva are immersed in HL3 saline

(Stewart et al., 1994) that contains (in mM): 70 NaCl, 5 KCl, 20 MgCl2, 10 NaHCO3, 5 trehalose,

115 sucrose, and 5 HEPES, pH 7.2.

2.The solution is filter sterilized before using and stored at 4˚C.

3.Concentrations of Ca2 can be added to the HL3 solution as required.

oLarval dissections are performed at low Ca

2+

concentrations (to avoid muscle

contraction) using HL3 + 0.25 mM CaCl2, kept on ice.

oElectrophysiological experiments are typically recorded in HL3 + 1.0 mM CaCl2, but this

can be adjusted from 0.4 – 1.5 mM Ca2+ if desired.

Preperation of sylgard plates:

Dissections were performed on plates containing a layer of transparent silicone elastomer (Sylgard).

These were prepared by mixing Sylgard base with curing agent (10:1) which was poured into small

plastic petri dishes (35 x 10 mm) and left to harden overnight at 37C.

Preparation of stimulating and recording pipettes:

1.Pipettes were pulled with a Sutter P‐2000 Laser based micropipette puller.

2.Stimulating electrodes are prepared from thin‐walled borosilicate glass capillaries with wide

ends that were slightly bigger than the width of the severed motor axons. The shaped ends were

firepolished to give a smooth finish, so minimize damage to the nerve. Stimulating electrodes

are filled with bath solution (HL3 + 1.0 mM Ca2+).

3.The recording electrodes are prepared from 1.2mm borosilicate glass, which is pulled to form a

sharp pipette (30–60 mΩ), and filled with 3M KCl.

Part 1: Dissection of Drosophila Larvae

1.Third instar wandering larvae are dissected to expose the muscles in the body wall as described

previously (Estes et al., 1996). In preparation for the dissections, larvae are collected from stocks

of the experimental and control genotypes.

2.Dissections are performed in ice‐cold HL3 + 0.25 mM Ca2 on small silicone plates (35 x 10 mm).

HL3 solution must be ice‐cold for dissections in order to stun the larva, so is kept on ice before

and during dissections.

3.Position the larvae tracheal side up with the anterior end pointing away from your hand.

4.For electrophysiology experiments, larva are pinned down using short dissection pins (~2 mm in

length), which are less likely to hit the microscope objective and electrodes during an

experiment.

5.Pin the centre of the tail region followed by the head of the larva positioning pins as close to the

ends as possible and angle the pins outwards. Slightly stretch the larvae lengthwise when

inserting the second pin.

6.Make a horizontal incision with scissors as far caudal as possible on the dorsal side of the larva.

From the incision, cut up the midline toward the rostral end. At the rostrum of the animal make

horizontal incisions to the left and right of the pin.

7.Remove the trachea and digestive organs with forceps by gently pulling from the tail end

upwards.

8.Pin the four corners of the larval body wall to the sylgard plate so it is stretched out horizontally

and vertically.

9.Using forceps, gently hold the CNS and raise it slightly. Cut the peripheral nerves that innervate

the muscles posterior to the ventral ganglion and remove the brain.

10.Wash the dissected prep twice with HL3 + 1.0 mM Ca2+.

11.The dissection should be completed within 3‐10 minutes, and the electrophysiological

recordings started immediately while the cells are still healthy and responsive. If the dissection

takes too long, the resting membrane potential of the muscle cells will be less than ‐60 mV,

which is too low to record.

Part 2: Intracellular recordings from larval muscle cells.

1.Place the dissection plate on the electrophysiology microscope and submerge the prep with HL3

+ 1 mM Ca2+ and fix the bath electrode so it is in contact with the solution.

2.All experiments are performed on muscle 6 within the third abdominal segment (A3). Peripheral

nerves that innervate the muscles are stimulated using a suction electrode (Jan and Jan, 1976).

3.Position the stimulating electrode in the dish first to avoid vibrations when the intracellular

electrode is in place. Place the stimulating electrode in close vicinity to the motor neuron

innervating muscle 6 and apply gentle suction until the cut nerve is inside the glass pipette. Be

careful not to stretch the nerves when suction is applied. Raise the pipette slightly so it is not

touching the muscle and position it so it is not pulling on the cut nerve fibers.

Figure 1.   A schematic of the musculature and innervating motor neurons of one abdominal segment

from the Drosophila larva. The position of the stimulating pipette and severed nerves and the recording

electrode in position at muscle 6 are illustrated.

4.Intracellular recordings are made with sharp microelectrodes filled with 3 M KCl. Position the

recording electrode above the centre of muscle 6 on the third abdominal segment (A3) and

adjust the input offset so it reads zero for the bath solution. Slowly lower the electrode and

approach the muscle under high optical magnification until it touches the muscle surface. Watch

the oscilloscope to confirm that the muscle has been penetrated. The resting membrane

potential should be at least ‐60 mV. Sporadic miniature endplate potentials should now be

visible. Leave the cell to stabilize for one minute before starting to record.

7 6 13 12 5 4

8

Recording

electrode

Stimulating

pipette

5.Changes in membrane potential are detected with an Axon HS‐2A head stage and an Axoclamp

2B amplifier and recorded with Clampex v 8.2.0.235 (Axon Instruments). The Axoclamp 2B is

interfaced to a computer that runs pCLAMP software and works in conjunction with the Digidata

1322A interface.

6.Cells were recorded for 3 minutes without any stimuli to measure mEJP responses. Traces were

analysed using Mini analysis software (Synaptosoft, v 6.0.3) and the mEJP amplitude and

frequency determined.

Nerve Stimulation:

1.The severed end of the motor neuron is stimulated with a series of square voltage pulses (0.3

ms duration) at an intensity that is sufficient for both motor axons, to evoke consistent

responses in the whole muscle.

2.The stimulus is generated with the Master‐8 pulse generator, which is programmed to deliver

pulses continuously according to the duration and interval times.

3.Allow five seconds between stimuli for synaptic recovery at the NMJ.

4.When determining the strength of the stimulus, begin with the lowest level and increase the

intensity slowly over several trials. The minimum stimulus intensity that results in an evoked

response should be used in experiments.

5.When the appropriate stimulus intensity is reached, there should be one compound EJP and one

muscle contraction evoked by the stimulus.

6.Record at least 10 evoked potentials from each muscle and average the amplitudes.

Part 3: Representative Results

Figure 2.

Representative intracellular recording from muscle 6 showing the evoked EJP’s is response to electrical

stimulation of the segmental nerve, and the sporadic minature endplate potentials, or mEJP’s. EJP

amplitudes in muscle 6 of healthy wild‐type larvae, such as Canton S, are typically around 35 mV and the

mEJP amplitudes between 1‐2 mV.

Discussion

The methods described here provide a relatively quick and broad way to detect changes in synaptic

function at the NMJ. The ability to perform electrophysiological recordings using intact animals in vivo,

and perform genetic or pharmacological manipulations, make Drosophila an ideal animal model for

investigating the physiological and genetic aspects of neurotransmission.

Since muscle cells are very large, some might prefer to add an additional step to this protocol for two

electrode voltage clamp (TEVC) recording. This can be performed on the same larval preparation with

the intracellular electrode in place, by positioning a current‐passing electrode on the cell. Once the cell

is sufficiently voltage clamped, the current response can be recorded.

Although muscle 6 is the most commonly used for electrophysiology recordings, other muscles can also

be used. For example muscle 12, which is not required for larval locomotion as much as muscle 6, is a

good target for driving the expression of potentially lethal genes that may of interest.

The methods described here show the essentials of Drosophophila NMJ electrophysiology – techniques

that were first described by Jan and Jan in 1976, and have since become the model system for

researching synaptic physiology.

Acknowledgments:

Disclosures: I have nothing to disclose

Table of specific reagents and equipment:

Name of the reagent Company Catalogue

number

Comments

(optional)

Small Petri dishes (35 x 10 mm) Becton Dickinson 1008

SYLGARD 182 Silicone Elastomer Kit Dow Corning Corporation 3097366-1004

Dissecting microscope Carl Zeiss 475002-9902

Light for microscope Schott KLI500

Dissection pins Fine Science Tools 26002-10

Dissection scissors: 3mm Vannas Spring

Scissors Fine Science Tools 15000-0

Dumont SS Forceps Fine Science Tools 11200-33

Dumont #5 Forceps Fine Science Tools 11252-20

Thin-walled borosilicate glass capillaries, with

filament (1.0 mm, 4 in) World Precision

Instruments, Inc. TW100F-4

Borosilicate glass capillaries, with filament

(1.2 mm, 4 in) World Precision

Instruments, Inc. 1B120F-4

Sutter P-2000 Laser Based

Micropipette Puller Sutter Instruments Model P-2000

Pipette polisher Narishiga MF-83

Master-8 Pulse Generator A.M.P.I

Axon HS-2A head stage Axon CNS, Molecular

Devices Model HS-2A

Micromanipulators Sutter Instruments MP-85

Axoclamp 2B amplifier Axon CNS, Molecular

Devices AXOCLAMP 2B

Clampex Software Axon CNS, Molecular

Devices v 8.2.0.235

Mini analysis software. v 6.0.3 Synaptosoft,

Digidata 1322A D-A converter Molecular Devices,

Foster City, CA Model 1322A

Brownlee Precision Amplifier Brownlee Model 410

pClamp 9 software Axon CNS, Molecular

Devices PCLAMP 9

STANDARD

Vibration table for electrophysiology set up Technical manufacturing

corporation.

Faraday Cage

NaCl Baker 4058-01

KCl Sigma p-9333

NaHCO3 Sigma s6297-1kg

Trelahose Sigma TO167

Sucrose Fisher bp220-212

HEPES Sigma h-3375

MgCl-6H2O Sigma m2670-1kg

CaCl2 Fisher c79-500

References: please use the same format you would use for a Nature article.  For example:

Atwood, H.L. Govind, C.K. & Wu, C.F. Differential ultrastructure of synaptic terminals on ventral

longitudinal abdominal muscles in Drosophila larvae. J. Neurobiol. 24 8 (1993), pp. 1008–1024.

Estes PS, Roos J, van der Bliek A, Kelly RB, Krishnan KS, Ramaswami M (1996) Traffic of dynamin within

individual Drosophila synaptic boutons relative to compartment‐specific markers. J Neurosci 16:5443‐

5456

Jan, L. Y. & Jan, Y. N. Properties of the larval neuromuscular junction in Drosophila melanogaster. J

Physiol (Lond) 262:189‐214 (1976)

Katz, L. C. & Shatz, C. J.  Synaptic activity and the construction of cortical circuits. Science 274, 1133–

1138 (1996)

Stewart, B. A., Atwood, H. L., Renger, J. J., Wang J, & Wu, C‐F. Improved stability of Drosophila larval

neuromuscular preparations in haemolymph‐like physiological solutions. J Comp Physiol [A] 175:179‐191

(1994)

Figure legends

Figures: should be uploaded electronically as separate files (TIFF, JPEG and other formats are

acceptable)